| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0306-v4 |
| --- | --- |

| Title: | Non CE5: DMVR internal edge deblocking without using refined MVs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jeeva Raj Arumugam Sagar Kotecha Shailesh Ramamurthy | Tel: Email: | +91-8066601027 Jeeva.Raj@ittiam.com Sagar.Kotecha@ittiam.com Shailesh.Ramamurthy@ittiam.com |
| Source: | Ittiam Systems Private Limited |  |  |

_____________________________

# Abstract

DMVR has been proven to show significant coding gains across sequences also ensuring no degradation in subjective quality. Since the VTM implementation is such that deblocking does not use refined motion vectors, one of the contributions has alluded to potential subjective issues in some corner cases in just a few frames of certain sequences when DMVR is turned on. This input contribution provides a method to use a fixed boundary strength and fixed filter length for DMVR internal edges under the conditions of current block QP greater than a particular value without using refined MVs

# Introduction

While DMVR has been shown to have significant coding gains with no degradation of subjective quality in general, JVET-O0158, points out that since the VTM implementation is such that deblocking does not use refined motion vectors, a very few frames of a few sequences could show potential subjective issues. We now provide a method to use a fixed boundary strength and fixed filter length for DMVR internal edges under the conditions of current block QP greater than a particular value without using refined MVs.

# Proposed method

The following figure helps explain our methods towards deblocking of DMVR internal edges without using refined MVs. Q denotes the current sub-block. The sub-blocks marked as P denote the neighbours (left and top) of Q.

Figure 1: DMVR CU size of 64x64 with 16x16 sub-block edges marked

Method 1:

Method 1 includes the following

- Internal edges at sub-block size of 16x16 are marked for filtering (as shown in Fig-1 as dotted lines)

- Maximum filter length P and Maximum filter length Q will be set to 1 for all the DMVR internal edges

- Boundary strength for the DMVR internal edges are set to a fixed value of 1 if the quantization parameter of the current DMVR coding unit is greater than a chosen threshold (of 45)

This implementation does not need any extra memory requirement for line buffers to store the refined MVs.

Method 2

| if( deblocking_filter_override_flag ) { |  |
| --- | --- |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| slice_dmvr_qp_th_minus_45 | se(v) |
| } |  |
| } |  |

In method 2, all aspects of method 1 is retained, except that instead of using fixed QP threshold of 45, it is derived from the signaled parameter “slice_dmvr_qp_th_minus_45” as follows

QP threshold for internal edges =  45 + slice_dmvr_qp_th_minus_45

# Results

Method 1 (QP threshold of 45)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.04% | -0.02% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.01% | 100% | 102% |
| Class B | -0.01% | 0.02% | 0.00% | 100% | 101% |
| Class C | 0.01% | 0.03% | 0.05% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 101% |
| Class D | 0.00% | -0.04% | 0.06% | 100% | 101% |
| Class F | 0.01% | 0.01% | 0.01% | 100% | 101% |

Method 2 (with slice_dmvr_qp_th_minus_45 = 0)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.04% | -0.02% | 100% | 102% |
| Class A2 | 0.00% | 0.00% | 0.01% | 101% | 102% |
| Class B | -0.01% | 0.02% | 0.00% | 100% | 102% |
| Class C | 0.01% | 0.03% | 0.05% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 102% |
| Class D | 0.00% | -0.04% | 0.06% | 100% | 102% |
| Class F | 0.01% | 0.01% | 0.01% | 100% | 102% |

Following table provides the overall CTC average for different QP thresholds

| DMVR CU with BS1 for internal edges and Max Filter length 1 for CU QP | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| > 45 | 0.00% | 0.01% | 0.01% | 100% | 101% |
| > 44 | 0.00% | 0.01% | 0.01% | 100% | 102% |
| > 43 | 0.00% | 0.01% | 0.01% | 100% | 102% |
| > 42 | 0.00% | 0.01% | 0.01% | 100% | 102% |
| > 41 | 0.00% | 0.01% | 0.01% | 100% | 102% |
| > 40 | 0.06% | 0.01% | 0.01% | 100% | 102% |

# Subjective Quality Assessment

Careful visual quality analyses have been performed internally and the following observations are reported:

- In video playback, it is very difficult to notice the gain of de-blocking DMVR internal edges either using refined MVs or non-refined MVs w.r.t anchor (VTM6.0)

- According to frame by frame analysis it is observed that sub-Pu blocking artefacts are almost completely eliminated by the proposal when QP threshold if set to QP=41.

# Conclusion

This contribution provides solutions to filter the DMVR internal edges without the use of refined MVs, by having a fixed boundary strength and fixed filter length under the conditions of current block QP greater than 41. It is recommended to adopt the proposed changes and set the default QP threshold to 41 in VTM.

# References

- JVET-O0158, Non-CE5: Deblocking based on DMVR refined motion K. Andersson and J. Enhorn

- JVET-O2001-vE, Versatile Video Coding (Draft 6), Benjamin Bross, Jianle Chen and Shan Liu

# Patent rights declaration(s)

Ittiam Systems Pvt. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).