| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0310-v2 |
| --- | --- |

| Title: | Non-CE4: On Simplification of PROF and BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangdong Chen Li Wang | Tel: Email: | +86 15156063604 chenfangdong@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

This proposal reports two modifications about PROF and BODF. First, the padding process for PROF and BODF modes is simplified with removing up to 20% unnecessary sample fetching. The first simplification has no influence on the coding performance since all of the removed samples are not used in the coding process. Second, the derivation of motion vector difference array for PROF mode in the specification, which is different from that in VTM6.0, is tested. It is reported that the specification version brings 0.64% and 0.48% loss for RA and LB configurations respectively compared to VTM-6.0. Hence, it is proposed to align the specification with the software.

Introduction

For PROF or BDOF modes, one column/row samples are padding as Figure 1. But the 4 padding samples in the four corners are useless.

Figure 1  The padding samples for PROF and BDOF modes in VTM6.0

In the specification (Draft 6 vE), when cbProfFlagLX is 1, the motion vector difference array diffMv is derived as follows:

- The variables sbWidth and sbHeight, dmvLimit, posOffsetX and posOffsetY are derived as follows:

sbWidth  =  cbWidth / numSbX		(8-729)

sbHeight  =  cbHeight / numSbY		(8-730)

dmvLimit  =  1  <<  Max( 6, BitDepthY − 6 )		(8-731)

posOffsetX  =  6 * dHorX + 6 * dVerX		(8-732)

posOffsetY  =  6 * dHorY + 6 * dVerY		(8-733)

- For x = 0..sbWidth − 1 and y = 0..sbHeight − 1, the following applies:

diffMv[ x ][ y ][ 0 ] = x * ( dHorX << 2 ) + y * ( dVerX << 2 ) – posOffsetX	(8-734)

diffMv[ x ][ y ][ 1 ] = x * ( dHorY << 2 ) + y * ( dVerY << 2 ) – posOffsetY	(8-735)

Proposed Modification

Modification 1

The padding process is modified with removing the fetching of 4 corner samples. The proposed padding samples are shown in Figure 2.

Figure 2  The proposed padding samples for PROF and BDOF modes

The text is modified as follows.

  - If bdofFlag is equal to TRUE or ( sps_affine_prof_enabled_flag is equal to TRUE and inter_affine_flag[ xSb ][ ySb ] is equal to TRUE ), and only one or more of the following conditions are true, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma integer sample fetching process as specified in clause 8.5.6.3.3 with ( xIntL + ( xFracL >> 3) − 1), yIntL + ( yFracL >> 3 ) − 1 ) and refPicLX as inputs.

    - xL is equal to 0.

    - xL is equal to sbWidth + 1.

    - yL is equal to 0.

    - yL is equal to sbHeight + 1.

Modification 2

It is proposed to modify the text as follows, and keep the related part in the software unchanged.

When cbProfFlagLX is 1, the motion vector difference array diffMv is derived as follows:

- The variables sbWidth and sbHeight, dmvLimit, posOffsetX and posOffsetY are derived as follows:

sbWidth  =  cbWidth / numSbX		(8-729)

sbHeight  =  cbHeight / numSbY		(8-730)

dmvLimit  =  1  <<  Max( 6, BitDepthY − 6 )		(8-731)

posOffsetX  =  6 * dHorX + 6 * dHorY dVerX		(8-732)

posOffsetY  =  6 * dVerX  dHorY + 6 * dVerY		(8-733)

- For x = 0..sbWidth − 1 and y = 0..sbHeight − 1, the following applies:

diffMv[ x ][ y ][ 0 ] = x * ( dHorX << 2 ) + y * (dHorY dVerX << 2 ) – posOffsetX	(8-734)

diffMv[ x ][ y ][ 1 ] = x * (dVerX  dHorY << 2 ) + y * ( dVerY << 2 ) – posOffsetY	(8-735)

Experimental Results

To evaluate the coding performance of the proposed method, the VTM-6.0 is served as the experimental platform. The RA and LB configurations specified in [1] are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

We have two tests in the experiment.

Test1: The proposed Modification 1.

Test2: The derivation of motion vector difference array for PROF mode in the specification.

The results of Test 1 and Test 2 are given in Table 1 and Table 2. It is reported that the modification in Test 1 has no influence on the coding performance compared to VTM-6.0. And it is reported that the specification version of the derivation of motion vector difference array brings 0.64% and 0.48% loss for RA and LB configurations respectively compared to VTM-6.0. Hence, it is proposed to align the specification with the software.

Table 1 Experimental Results of Test 1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-6.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 98% |

Table 2 Experimental Results of Test 2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-6.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.23% | 0.17% | 0.09% | 100% | 100% |
| Class A2 | 0.89% | 0.19% | 0.18% | 100% | 100% |
| Class B | 0.97% | 0.69% | 0.49% | 100% | 103% |
| Class C | 0.34% | 0.23% | 0.29% | 101% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.64% | 0.37% | 0.30% | 100% | 102% |
| Class D | 0.26% | 0.06% | 0.14% | 101% | 102% |
| Class F | 0.36% | 0.19% | 0.22% | 101% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.74% | 0.66% | 0.58% | 100% | 97% |
| Class C | 0.26% | 0.24% | 0.20% | 100% | 102% |
| Class E | 0.33% | 0.58% | 0.73% | 100% | 102% |
| Overall | 0.48% | 0.50% | 0.49% | 100% | 100% |
| Class D | 0.24% | 0.23% | 0.42% | 100% | 101% |
| Class F | 0.52% | 0.58% | 0.50% | 101% | 101% |

Conclusion

In this contribution, two modifications about PROF and BODF are proposed. The first one simplify the padding process for PROF and BODF modes, and the second one proposes to align the specification with the software in the derivation of motion vector difference array for PROF mode. Based on the above results, it is suggested to adopt them into the VTM.

Reference

- F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).