| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0314-v2 |
| --- | --- |

| Title: | Non-CE4: Harmonization of PROF, BDOF and DMVR syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng, Xiaozhen Zheng, Shanshe Wang, Siwei Ma | Tel: Email: | xwmeng@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | Peking University, SZ DJI Technology Co., Ltd., |  |  |

_____________________________

- Abstract

This contribution presents a slice level on/off design for PROF. In the current VTM-6.1, PROF are always enabled for Affine mode CU that satisfies the condition of PROF, and DMVR / BDOF is controlled by the SPS as well as slice level on/off flag. In this contribution, it is proposed to add slice level on/off control for PROF. There are two methods proposed. The first one is to use the same syntax for DMVR/BDOF/PROF in slice and SPS level. The second one is to use the same syntax for PROF and BDOF, and use the separate syntax for DMVR.

Introduction

The three implicit inter-prediction refinement tools adopted in the current draft of VVC, namely DMVR, BDOF and PROF, are applied to inter coding units satisfies the condition of DMVR/BDOF/PROF. DMVR and BDOF are controlled by SPS and slice level on/off flag. However, there are no explicit flags that signaled to indicate whether PROF should be applied or not to the current slice.

In this proposal, a slice level flag to disable PROF are proposed to allow independent control for each slice.

Proposed Method

Two different methods are proposed in this propose,

Method1: Use the same slice level on/off control for DMVR/BDOF/PROF as illustrated in 2.2

Method2: Use the same slice level on/off control for BDOF/PROF and the separate control for DMVR as illustrated in 2.3, because the process of BDOF and PROF is similar with each other.

Mismatch between Software and Spec

There are some mismatches between software and spec in SPS level syntax. We propose to align the text with softeware.

| …… |  |
| --- | --- |
| sps_amvr_enabled_flag | u(1) |
| sps_bdof_enabled_flag | u(1) |
| sps_smvd_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| if( sps_bdof_enabled_flag ¦ ¦ sps_dmvr_enabled_flag) |  |
| sps_bdof_dmvr_slice_present_flag | u(1) |
| sps_mmvd_enabled_flag | u(1) |
| sps_isp_enabled_flag | u(1) |
| sps_mrl_enabled_flag | u(1) |
| sps_mip_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| sps_cclm_enabled_flag | u(1) |
| if( sps_cclm_enabled_flag && chroma_format_idc = = 1 ) |  |
| sps_cclm_colocated_chroma_flag | u(1) |
| sps_mts_enabled_flag | u(1) |
| if( sps_mts_enabled_flag ) { |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| } |  |
| sps_lfnst_enabled_flag | u(1) |
| sps_smvd_enabled_flag | u(1) |
| sps_sbt_enabled_flag | u(1) |
| if( sps_sbt_enabled_flag ) |  |
| sps_sbt_max_size_64_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| sps_affine_type_flag | u(1) |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| } |  |
| if( chroma_format_idc = = 3 ) |  |
| sps_palette_enabled_flag | u(1) |
| sps_bcw_enabled_flag | u(1) |
| if( chroma_format_idc = = 3 ) |  |
| sps_palette_enabled_flag | u(1) |
| sps_ibc_enabled_flag | u(1) |
| sps_ciip_enabled_flag | u(1) |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_enabled_flag ¦ ¦ sps_dmvr_enabled_flag) |  |
| sps_bdof_dmvr_slice_present_flag | u(1) |
| sps_triangle_enabled_flag | u(1) |
| …… | u(1) |

Method1: Use the same on/off control for DMVR/BDOF/PROF

| ……… | … |
| --- | --- |
| sps_amvr_enabled_flag | u(1) |
| sps_bdof_enabled_flag | u(1) |
| sps_smvd_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| if( sps_bdof_enabled_flag ¦ ¦ sps_dmvr_enabled_flag) |  |
| sps_bdof_dmvr_slice_present_flag | u(1) |
| sps_mmvd_enabled_flag | u(1) |
| sps_isp_enabled_flag | u(1) |
| sps_mrl_enabled_flag | u(1) |
| sps_mip_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| sps_cclm_enabled_flag | u(1) |
| if( sps_cclm_enabled_flag && chroma_format_idc = = 1 ) |  |
| sps_cclm_colocated_chroma_flag | u(1) |
| sps_mts_enabled_flag | u(1) |
| if( sps_mts_enabled_flag ) { |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| } |  |
| sps_sbt_enabled_flag | u(1) |
| if( sps_sbt_enabled_flag ) |  |
| sps_sbt_max_size_64_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| sps_affine_type_flag | u(1) |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| } |  |
| if( chroma_format_idc = = 3 ) |  |
| sps_palette_enabled_flag | u(1) |
| sps_bcw_enabled_flag | u(1) |
| sps_ibc_enabled_flag | u(1) |
| sps_ciip_enabled_flag | u(1) |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_enabled_flag ¦ ¦ sps_dmvr_enabled_flag ¦¦ sps_affine_prof_enabled_flag) |  |
| sps_bdof_dmvr_prof_slice_present_flag | u(1) |
| sps_triangle_enabled_flag | u(1) |
| ……………. | ….. |

| if( sps_affine_enabled_flag && !pps_five_minus_max_num_subblock_merge_cand_plus1 ) |  |
| --- | --- |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| slice_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_dmvr_prof_slice_present_flag ) |  |
| slice_disable_bdof_dmvr_prof_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 ) |  |
| max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |

Method2: Use the same on/off control for BDOF/PROF, use separate on/off control for DMVR

| ……… | … |
| --- | --- |
| sps_amvr_enabled_flag | u(1) |
| sps_bdof_enabled_flag | u(1) |
| sps_smvd_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| if( sps_bdof_enabled_flag ¦ ¦ sps_dmvr_enabled_flag) |  |
| sps_bdof_dmvr_slice_present_flag | u(1) |
| sps_mmvd_enabled_flag | u(1) |
| sps_isp_enabled_flag | u(1) |
| sps_mrl_enabled_flag | u(1) |
| sps_mip_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| sps_cclm_enabled_flag | u(1) |
| if( sps_cclm_enabled_flag && chroma_format_idc = = 1 ) |  |
| sps_cclm_colocated_chroma_flag | u(1) |
| sps_mts_enabled_flag | u(1) |
| if( sps_mts_enabled_flag ) { |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| } |  |
| sps_sbt_enabled_flag | u(1) |
| if( sps_sbt_enabled_flag ) |  |
| sps_sbt_max_size_64_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| sps_affine_type_flag | u(1) |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| } |  |
| if( chroma_format_idc = = 3 ) |  |
| sps_palette_enabled_flag | u(1) |
| sps_bcw_enabled_flag | u(1) |
| sps_ibc_enabled_flag | u(1) |
| sps_ciip_enabled_flag | u(1) |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| if( sps_dmvr_enabled_flag) |  |
| sps_dmvr_slice_present_flag | u(1) |
| if( sps_bdof_enabled_flag ¦ ¦ sps_affine_prof_enabled_flag) |  |
| sps_bdof_prof_slice_present_flag | u(1) |
| sps_triangle_enabled_flag | u(1) |
| ……………. | ….. |

| if( sps_affine_enabled_flag && !pps_five_minus_max_num_subblock_merge_cand_plus1 ) |  |
| --- | --- |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| slice_fpel_mmvd_enabled_flag | u(1) |
| if( sps_dmvr_slice_present_flag ) |  |
| slice_disable_dmvr_flag | u(1) |
| if( sps_bdof_prof_slice_present_flag ) |  |
| slice_disable_bdof_prof_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 ) |  |
| max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |

Simulation results

No experimental results are provided as the proposal only involves the change of slice header and PPS level disable flag syntax elements for DMVR, BDOF and PROF.

Conclusions

In this contribution, slice level DMVR/BDOF/PROF control is proposed to optimize the PROF design. Therefore, we propose to adopt this proposal to the next VVC.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-O2001, July. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.1

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).