| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0317-v2 |
| --- | --- |

| Title: | Non-CE4: BCW clean-up for weight signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dohyeon Park Yong-Uk Yoon Jihoon Do Jae-Gon Kim (KAU) Jinho Lee Jungwon Kang (ETRI) | Tel: Email: | +82-2-300-0414 dhpark@kau.kr yuyoon@kau.kr jhdo@kau.kr jgkim@kau.ac.kr jinosoul@etri.re.kr jungwon@etri.re.kr |
| Source: | Korea Aerospace University (KAU), ETRI |  |  |

_____________________________

- Abstract

The syntax element of BCW weight index is adaptively coded according to the value of NoBackwardPredFlag. It can violate the flexibility of codec and cause the dependency issue. This contribution proposes BCW clean-up design which 5 weights, w∈-2, 3, 4, 5, 10, can be parsed without any condition. The experimental results reportedly show 0.02% and 0.00% luma BD-rates performance for RA and LDB, respectively.

Introduction

In the VTM6, bi-prediction with CU-level weight (BCW) is integrated to allow weighted averaging of the two inter-predicted signals, as shown in the following formula.

Pbi-pred=8-w*P0+w*P1+4≫3

For each bi-predictive CU, the weight w is determined among five weight values, w∈-2, 3, 4, 5, 10. In the current BCW, some weights are restricted depending on the value of NoBackwardPredFlag, and binarizations for bcw_idx are also different. If NoBackwardPredFlag is equal to 1, which means there is no future picture in the display order among the reference pictures, all five weights are allowed. Otherwise, only three weights can be used. The weights and corresponding bin strings according to NoBackwardPredFlag are shown in Table 1.

Table 1. The weights and binarization for bcw_idx

| bcw_idx | Weights | Bin string (NoBackwardPredFlag == 1) | Bin string (NoBackwardPredFlag == 0) |
| --- | --- | --- | --- |
| 0 | 4 | 0 | 0 |
| 1 | 5 | 10 | 10 |
| 2 | 3 | 110 | 11 |
| 3 | 10 | 1110 | Not allowed |
| 4 | -2 | 1111 | Not allowed |

Proposed methods

The current syntax design of bcw_idx can cause some issues. Normative restriction on the possible weights can violate the flexibility of the codec. It is desirable that the adjustment on the number of available weights is handled at encoder non-normatively. Besides, NoBackwardPredFlag, which is determined by slice type and reference pictures, can cause a dependency  on the parsing of bcw_idx. To solve these issues, we proposed a method to remove the condition check of NoBackwardPredFlag at the parsing of bcw_idx. In the proposed method, the selected weight among 5 weights (w∈-2, 3, 4, 5, 10) can be parsed without any condition check. The adjustment on the number of available weights should be handled non-normatively.

The BCW weight index is binarized with truncated unary (TU) as shown in Table 2. The first bin of the bin string is context coded with its own context model, and rest bins are bypass coded.

Table 2. The proposed codeword for bcw_idx

| bcw_idx | Weights | Bin string |
| --- | --- | --- |
| 0 | 4 | 0 |
| 1 | 5 | 10 |
| 2 | 3 | 110 |
| 3 | 10 | 1110 |
| 4 | -2 | 1111 |

Experimental results

The proposed method is implemented on top of VTM6.0 and evaluated according to VTM CTC configurations [2]. The algorithm for weight searching in VTM6 is not changed (3 weights are searched in RA), and only the part of bcw_idx coding is modified. Table 2 shows the results of the proposed method. The experimental results reportedly show 0.02%/0.00% luma BD-rate changes with 100%/100% and 101%/99%, encoding and decoding run-time in RA/LDB configurations, respectively.

Table 3. Experimental results of the proposed method over VTM6.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.05% | 0.04% | 100% | 101% |
| Class A2 | 0.04% | 0.06% | 0.06% | 100% | 101% |
| Class B | 0.02% | 0.08% | -0.04% | 100% | 101% |
| Class C | 0.02% | 0.09% | 0.03% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.07% | 0.02% | 100% | 101% |
| Class D | -0.01% | -0.06% | 0.12% | 100% | 102% |
| Class F | 0.01% | 0.02% | 0.01% | 100% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 97% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 95% |

Draft text (based on JVET-O2001-vE)

| Table 9-77 – Syntax elements and associated binarizations |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax structure | Syntax element |  | Binarization |  |  |  |  |  |  |  |
|  |  |  | Process |  | Input parameters |  |  |  |  |  |
| coding_unit( ) | bcw_idx[ ][ ] |  | TR |  | cMax = NoBackwardPredFlag ? 4: 2cMax = 4 |  |  |  |  |  |
| Table 9-82 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |  |  |  |
| Syntax element |  | binIdx |  |  |  |  |  |  |  |
|  |  | 0 |  | 1 |  | 2 | 3 | 4 | >= 5 |
| bcw_idx[ ][ ] NoBackwardPredFlag = = 0 |  | 0 |  | bypass |  | na | na | na | na |
| bcw_idx[ ][ ] NoBackwardPredFlag = = 1 |  | 0 |  | bypass |  | bypass | bypass | na | na |

Conclusion

This contribution presents clean-up design for the syntax of BCW by removing the condition check of NoBackwardPredFlag in the weight usage. In the proposed method, 5 BCW weights are used without any condition. The method resolves the flexibility issue and dependency issues of BCW. It is recommended to consider the proposed method in the next VTM.

Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” JVET-O2001, July 2019.

- F. Bossen, J. Boyce, et al., “JVET common test conditions and software reference configurations for SDR video,” JVET document, JVET-N1010, May 2019.

Patent rights declaration(s)

Korea Aerospace University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).