| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0325-v1 |
| --- | --- |

| Title: | Non-CE4: Construction of spatial merge candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhao Wang, Yan Ye, Jiancong Luo | Tel: Email: | (+86)13716516722 baixiu.wz@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

In current VVC, the merge candidate list is constructed by including spatial merge candidates, temporal merge candidate, history-based motion vector predictor, pairwise average candidates and zero MVs in that order. The spatial merge candidates are constructed in VVC in the same way as in HEVC. In this contribution, two modifications are proposed to change the spatial merge candidate construction process. In the first proposed method,  the number of spatial merge candidates is proposed to be reduced from 5 to 3. The luma BD rate savings of the first method are reported to be 0.00% (RA) and 0.10% (LD). In the second proposed method, the spatial merge candidates are constructed differently for the MMVD mode. The luma BD rate savings of the second method are reported to be 0.08% (RA) and 0.21% (LD).

Introduction

In current VVC, the first candidates in the merge candidate list are the spatial neighbors. Figure 1 shows the positions of five spatial candidates. According to the order {A1, B1, B0, A0, B2}, the availability of each candidate position is checked. If a spatial neighboring block is intra predicted or the position is outside of the current slice or tile, it is considered as unavailable as a merge candidate. If the number of available candidates reaches 4, B2 will be skipped. In addition, some redundancy checks are performed to ensure that the motion data from the neighboring blocks are somewhat unique. To reduce complexity due to the redundancy checks, only limited redundancy checks are performed. For example, given the order of {A1, B1, B0, A0, B2}, B1 checks A1, B0 only checks B1, A0 only checks A1, and B2 only checks A1 and B1.

Figure 1. Positions of spatial merge candidates.

In VVC draft 6, the above merge list is used for the regular merge mode, the merge mode with motion vector difference (MMVD) and triangle partition mode (TPM). In the CTC, the maximum number of merge candidates is set to 6, 2, and 5 for regular merge, MMVD and TPM respectively.

Proposed method

Method 1: Simplified construction of spatial merge candidates

In this proposed method, the number of spatial merge candidates is reduced to 3, and the order {B1, A1, B0} is used. The remaining process stays the same as in current VVC.

Method 2: Separate construction orders for MMVD

In this method, the number of spatial merge candidates is kept to be 5. Besides the current construction order {A1, B1, B0, A0, B2}, a new construction order {B1, A1, B0, A0, B2} is proposed. For the regular merge mode and TPM, the new construction order {B1, A1, B0, A0, B2} is used. For the MMVD mode, the new construction order {B1, A1, B0, A0, B2} is used for the low-delay pictures and the existing order {A1, B1, B0, A0, B2} is used for the non-lowdelay pictures. The remaining process stays the same as in current VVC.

Experimental results

Test 1: Simplified construction of spatial merge candidates

|  | Random access Main10 Over VTM-6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.08% | -0.01% | 100% | 100% |
| Class A2 | 0.06% | 0.12% | 0.10% | 100% | 100% |
| Class B | -0.01% | 0.13% | 0.09% | 100% | 100% |
| Class C | -0.06% | -0.01% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.08% | 0.05% | 100% | 100% |
| Class D | -0.06% | 0.01% | 0.10% | 100% | 100% |
| Class F | 0.00% | -0.02% | 0.00% | 100% | 100% |

|  | Lowdelay Main10 Over VTM-6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.06% | 0.13% | 100% | 100% |
| Class C | -0.08% | -0.13% | -0.05% | 100% | 100% |
| Class E | -0.30% | 0.21% | -0.18% | 100% | 99% |
| Overall | -0.10% | 0.03% | -0.01% | 100% | 100% |
| Class D | -0.05% | -0.05% | 0.89% | 100% | 99% |
| Class F | 0.11% | -0.04% | 0.14% | 100% | 100% |

Test 2: Separate construction orders for MMVD

|  | Random access Main10 Over VTM-6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.02% | -0.04% | 100% | 100% |
| Class A2 | -0.02% | -0.05% | -0.06% | 100% | 100% |
| Class B | -0.08% | 0.09% | 0.01% | 100% | 100% |
| Class C | -0.12% | -0.12% | -0.08% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.02% | -0.04% | 100% | 100% |
| Class D | -0.08% | 0.01% | 0.09% | 100% | 100% |
| Class F | -0.02% | -0.03% | 0.01% | 100% | 100% |

|  | Lowdelay Main10 Over VTM-6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | -0.09% | -0.24% | 100% | 100% |
| Class C | -0.18% | -0.15% | -0.06% | 100% | 100% |
| Class E | -0.50% | -0.10% | -0.43% | 100% | 100% |
| Overall | -0.21% | -0.11% | -0.23% | 100% | 100% |
| Class D | -0.11% | -0.02% | 0.42% | 100% | 100% |
| Class F | 0.08% | 0.00% | 1.50% | 100% | 100% |

Conclusion

This contribution proposes two methods to modify the spatial merge candidate construction process. In the first method, construction of spatial merge candidates is simplied to use only 3 candidates. The first method reduces the complexity of the merge construction process with 0.00% (RA) and 0.10% (LD) luma coding gain. In the second method, the spatial merge candidate construction uses separate construction orders for different merge modes. The second method achieves 0.08% (RA) and 0.21% (LD) luma coding gain.

References

High Efficiency Video Coding, Rec. ITU-T H.265 and ISO/IEC 23008-2, Jan. 2013

B. Bross, J. Chen, S. Liu, “Versatile Video Coding draft 6,” JVET-O2001, July 2019.

J. Chen, Y. Ye, and S. Kim, “Algorithm description for Versatile Video Coding and Test Model 5 (VTM 5), JVET-N1002, April 2019.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).