| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0333 |
| --- | --- |

| Title: | On SAO performance |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Patrice Onno Guillaume Laroche Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution reports the performance of SAO when the Adaptive Loop Filter is disabled. The test is performed under the Common Testing Conditions of VTM 6.0, where the anchor is the VTM 6.0 without ALF and SAO. The average Y/Cb/Cr BDR gains of the VTM 6.0 with SAO over the VTM-6.0 without both SAO and ALF are -0.37%/-0.83%/-1.24% for the AI configuration, -0.89%/-1.33%/-1.25% for the RA configuration, -1.06%/-2.67%/-4.25% for the Low Delay B configuration, and -2.74%/-4.08%/-5.43% for the Low Delay P configuration.

# Introduction

For video products with limited CPU resources, some VVC encoders will probably not use the Adaptive Loop Filter due to its relatively high complexity. However, the SAO filter which is quite simple will be certainly used. It is important to note that SAO is already widely deployed in several encoders for HEVC products having limited resources. This contribution is evaluating the benefit of SAO when ALF is disabled.

# Experiments

## Description

In this contribution, the goal is to report the coding gain performance of SAO when ALF is turned off. The anchor for this experiment is the VTM 6.0 using the Common Testing Conditions of VTM6 [1] with the following command line options: --ALF==0 and --SAO=0. For the SAO evaluation test in VTM6.0, the following command line options have used:  --ALF==0 and --SAOGreedy=1.

## Results

# Conclusion

Considering the coding gains of SAO when ALF is off, it is recommended to consider both ALF and SAO filters in the final design of the VVC standard to address different types of application and product having various encoding capabilities.

# References

- JVET common test conditions and software reference configurations for SDR video, JVET-N1010.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).