| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0334-v2 |
| --- | --- |

| Title: | AHG17: On constant slice header parameter set in PPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Biao Wang, Semih Esenlik, Anand Meher Kotra, Han Gao, Elena Alshina | Tel: Email: | biao.wang@huawei.com |
| Source: | Huawei Technologies Dusseldorf GmbH |  |  |

_____________________________

# Abstract

In VVC draft 6, constant_slice_header_params_enabled_flag is introduced in the PPS which controls presence of syntax elements which specify default values of a set of sytax elements in the slice header (constant slice header parameter set). The current contribution proposes three alternative modifications on top of it. First, additional syntax elements are proposed to be included in the constant slice header parameter set. Second, the number of constant parameter sets is increased from 1 to 7, in order to be able to signal 7 different default coding tool configuration sets in the PPS. Thirdly an overriding mechanism in the slice header is proposed to override all of the default tool configurations. It is asserted that the proposed modifications allow selecting different default coding parameters for different picture partitions (sub-pictures, slices etc.) inside a same picture.

The changes in version 2 are summarized as follows:

- For modification 1, the deletion of cabac_init_present_flag is not necessary. It was kept as the way in VVC draft 6.

- For modification 3, the word overwriting is replaced with overriding, in order to be consistent with the naming of other flags bearing the same idea.

- Changes to specification reflecting modification 1 and 3 were improved accordingly.

- Presentation slides are uploaded.

# Background

According to draft 6 of VVC specification [1], the default values for the following syntax elements can be signaled in the PPS, with the option to signal in the slice header. The following table summarizes the syntax elements in the slice header and the corresponding default values in the PPS.

| Syntax Element in PPS (constant slice header parameter set) | Corresponding Syntax Element in Slice header |
| --- | --- |
| pps_dep_quant_enabled_idc | dep_quant_enabled_flag |
| pps_ref_pic_list_sps_idc[ i ] | ref_pic_list_sps_flag[ i ] |
| pps_temporal_mvp_enabled_idc | slice_temporal_mvp_enabled_flag |
| pps_mvd_l1_zero_idc | mvd_l1_zero_flag |
| pps_collocated_from_l0_idc | collocated_from_l0_flag |
| pps_six_minus_max_num_merge_cand_plus1 | six_minus_max_num_merge_cand |
| pps_five_minus_max_num_subblock_merge_cand_plus1 | five_minus_max_num_subblock_merge_cand |
| pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 | max_num_merge_cand_minus_max_num_triangle_cand |

The constant slice header parameter set provides flexibility to choose whether a syntax element is signalled in the slice header or in the PPS. As an example if the value of pps_dep_quant_enabled_idc is equal to zero, dep_quant_enabled_flag is present in all of the slices referring to the PPS. Otherwise (pps_dep_quant_enabled_idc is not equal to zero), the dep_quant_enabled_flag is not present and its value is inferred to be equal to pps_dep_quant_enabled_idc – 1, i.e. default value in the PPS is used.

In most of the applications the said parameters are expected to be constant, in which case the signalling of default values in the PPS help saving bits in the slice header.

# Proposed Modifications

## Modification 1

The following syntax elements do not have default values, it is suggested to include these syntax elements in the list of constant slice header parameter set.

- slice_six_minus_max_num_ibc_merge_cand

- slice_fpel_mmvd_enabled_flag

The syntax elements above are not expected to be changed by a typical encoder frequently. For example if fractional offset of MMVD is enabled in the SPS, the value of slice_fpel_mmvd_enabled_flag is most likely constant and equal to 1 if the encoded content is natural content or 0 in the case of screen content. In CTC the values of the syntax elements are constant for the whole video sequence.

Moreover a second group of syntax elements are suggested to be included in the constant slice header parameter set, since the value of those parameters cannot be changed by a real time encoder once the encoding of the slice begins. The syntax elements in this category are:

- slice_joint_cbcr_sign_flag

- slice_sao_luma_flag

- slice_sao_chroma_flag

- slice_alf_enabled_flag

- slice_alf_chroma_idc

- slice_lmcs_enabled_flag

- slice_chroma_residual_scale_flag

- cabac_init_flag

- sign_data_hiding_enabled_flag

- slice_disable_bdof_dmvr_flag

In the CTC, the values of the elements in the second group change for each slice, since the values of the syntax elements are determined by frame-based RDO process. Depending on the value of the slice level on/off flags (such as slice_sao_luma_flag or  slice_alf_enabled_flag), additional control information is embedded in the bitstream (CTU-level control information for SAO and ALF). Therefore in order to be able to change the value of the slice level on/off flag, one needs to reencode the CABAC bitstream, which is how the reference software operates.

For most real world encoders however the bitstream cannot be reencoded, hence the values of the above syntax elements are determined beforehand and cannot be changed once the encoding of the slice data starts. Therefore it is suitable to include these syntax elements in the constant slice header parameter set so that default values can be signalled in the PPS.

Finally the log2_transform_skip_max_size_minus2 syntax element is proposed to be included in the constant slice parameter set, which is beneficial when more than one constant parameter set is signalled in the PPS (to be explained in the second modification).

The corresponding modification to syntax in PPS is shown as follows:

7.3.2.4	Picture parameter set RBSP syntax

| … |  |
| --- | --- |
| if( !single_tile_in_pic_flag ¦ ¦ entropy_coding_sync_enabled_flag ) |  |
| entry_point_offsets_present_flag | u(1) |
| … |  |
| if( sps_transform_skip_enabled_flag ) |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| … |  |
| constant_slice_header_params_enabled_flag | u(1) |
| if( constant_slice_header_params_enabled_flag ) { |  |
| pps_dep_quant_enabled_idc | u(2) |
| for( i = 0; i < 2; i++ ) |  |
| pps_ref_pic_list_sps_idc[ i ] | u(2) |
| pps_temporal_mvp_enabled_idc | u(2) |
| pps_mvd_l1_zero_idc | u(2) |
| pps_collocated_from_l0_idc | u(2) |
| pps_six_minus_max_num_merge_cand_plus1 | ue(v) |
| pps_five_minus_max_num_subblock_merge_cand_plus1 | ue(v) |
| pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 | ue(v) |
| pps_six_minus_max_num_ibc_merge_cand_plus1 | ue(v) |
| pps_fpel_mmvd_enabled_idc | u(2) |
| pps_joint_cbcr_sign_idc | u(2) |
| pps_sao_luma_idc | u(2) |
| pps_sao_chroma_idc | u(2) |
| pps_alf_enabled_idc | u(2) |
| pps_alf_chroma_idc | u(3) |
| pps_lmcs_enabled_idc | u(2) |
| pps_chroma_residual_scale_idc | u(2) |
| pps_cabac_init_idc | u(2) |
| pps_sign_data_hiding_enabled_idc | u(2) |
| pps_disable_bdof_dmvr_idc | u(2) |
| log2_transform_skip_max_size_minus2 | ue(v) |
| } |  |

The changes in slice header and the semantics section can be found in the attached specification changes document for modification 1.

## Modification 2

In some cases, the parameters included in constant slice header parameter set have the possibility to change in different part of a picture. Applying a single default constant slice header parameter set for all slices cannot adapt the difference between different slices.

To address the above-mentioned concerns, it is proposed to include up to 7 constant slice header parameter sets in the PPS. An additional syntax element is included in the slice header to indicate which constant parameter set is used from the PPS. The proposed modification also allows to override default value at slice level.

The proposed syntax modifications are as follows:

7.3.2.4	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| num_constant_slice_header_params | u(3) |
| constant_slice_header_params_enabled_flag |  |
| if( constant_slice_header_params_enabled_flag ) |  |
| for( k=0;k< num_constant_slice_header_params;k++ ) { |  |
| log2_transform_skip_max_size_minus2[ k ] | ue(v) |
| pps_dep_quant_enabled_idc[ k ] | u(2) |
| for( i = 0; i < 2; i++ ) |  |
| pps_ref_pic_list_sps_idc[ i ] [ k ] | u(2) |
| pps_temporal_mvp_enabled_idc[ k ] | u(2) |
| pps_mvd_l1_zero_idc[ k ] | u(2) |
| pps_collocated_from_l0_idc[ k ] | u(2) |
| pps_six_minus_max_num_merge_cand_plus1[ k ] | ue(v) |
| pps_five_minus_max_num_subblock_merge_cand_plus1[ k ] | ue(v) |
| pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1[ k ] | ue(v) |
| } |  |

7.3.6.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| non_reference_picture_flag | u(1) |
| slice_type | ue(v) |
| if (num_constant_slice_header_params > 0) |  |
| constant_slice_header_params_id_plus1 | ue(v) |
| k = constant_slice_header_params_id_plus1 – 1 |  |
| … |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && (k==-1 ?1:!pps_ref_pic_list_sps_idc[ i ][k]) && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_sps_flag[ i ] | u(1) |
| if( ref_pic_list_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && |  |
| ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| slice_poc_lsb_lt[ i ][ j ] | u(v) |
| delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( delta_poc_msb_present_flag[ i ][ j ] ) |  |
| delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| … |  |
| if ( slice_type != I ) { |  |
| if( sps_temporal_mvp_enabled_flag && ( k == -1 ? 1: !pps_temporal_mvp_enabled_idc[ k ] ) ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| if( slice_type = = B && ( k == -1 ? 1: !pps_mvd_l1_zero_idc[ k ] ) ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B && ( k == -1 ? 1: !pps_collocated_from_l0_idc[ k ]) ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && NumRefIdxActive[ 0 ] > 1 ) ¦ ¦ ( !collocated_from_l0_flag && NumRefIdxActive[ 1 ] > 1 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( pps_weighted_pred_flag && slice_type = = P ) ¦ ¦ ( pps_weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| if( k == -1 ? 1: !pps_six_minus_max_num_merge_cand_plus1[ k ] ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag && ( k == -1 ? 1: !pps_five_minus_max_num_subblock_merge_cand_plus1[ k ] ) ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| slice_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_dmvr_slice_present_flag ) |  |
| slice_disable_bdof_dmvr_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && (k==-1?1:!pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1[ k ]) ) |  |
| max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| } |  |
| … |  |
| if ( k == -1 ? 1: !pps_dep_quant_enabled_flag [k]) |  |
| dep_quant_enabled_flag | u(1) |
| … |  |

The changes in the semantics section can be found in the attached specification changes document for modification 2.

## Modification 3

When num_constant_slice_header_params is larger than 0 and constant_slice_header_params_id_plus1 is signaled as 0, an overriding mechanism is enabled. For example, the condition (k==-1 ?1:!pps_ref_pic_list_sps_idc[ i ][k] ) would be always true regardless of the default value in the PPS. All of the default values signalled in the PPS can be overridden in the slice header by signalling them explicitly.

A slice-level overriding mechanism for the constant slice header parameter set might be desirable. It allows an encoder to change the encoding setting at slice level by overriding the default values in the constant slice header parameter set.

The overriding mechanism can be applied without multiple sets of constant parameters (i.e. modification 2), when only one constant slice header parameter set is allowed in the PPS. In this case the syntax constant_slice_header_params_id_plus1 can be considered as an overriding flag, indicating whether the overriding mechanism is enabled or not for the current slice. For the sake of readability, the syntax constant_slice_header_params_id_plus1 is renamed as slice_override_constant_params_flag in the attached specification change document for modification 3.

# Benefit of the modifications

The proposed method would be beneficial for the following scenarios.

## Scenario 1

It is asserted that the proposed modification allows usage of different sets of default coding parameters for different slices of a same picture. A first scenario is e-sport broadcasting. In e-sport broadcasting, sometime part of the picture contains screen content about the game whereas rest of the picture is natural content showing the player, it is advantageous to use different sets of coding parameters in each part. For example it would be advantageous to modify the following coding parameters for the part of the picture that contains screen content:

- slice_fpel_mmvd_enabled_flag

- slice_six_minus_max_num_ibc_merge_cand

- slice_alf_enabled_flag

- slice_sao_enabled_flag

- five_minus_max_num_subblock_merge_cand

- log2_transform_skip_max_size_minus2

- slice_disable_bdof_dmvr_flag

DMVR, BDOF, fractional MMVD offsets and in-loop filters are tools that do not perform well in the case of screen content whereas transform skip and IBC are tools that are specific for SCC. Therefore in the case of a picture containing mixed content, it is advantageous to use different coding parameters in different parts of the picture.

## Scenario 2

The second scenario requires multiple sets of parameters in PPS is video conferencing. For video conferencing, a picture might be divide into multiple parts and each part corresponds to one participant, as shown in Figure 1. It might be possible that higher fidelity is required for the area of the speaker, or different participants have different background. In both cases, different video coding configurations might be required for different areas.

Figure 1 – Example of a picture with areas showing video conferencing participants [2]

## Scenario 3

The proposed method allows to override default values at the slice level, i.e. the value of the syntax constant_slice_header_params_id_plus1 is signaled as 0. It is asserted that the proposed overriding mechanism is desirable as a similar overriding flag (partition_constraints_override_enabled_flag) enables overriding partition configuration parameters at slice level has been adopted.

For example, this might be helpful for the case that the encoder runs out of the hardware resource and needs to keep a fixed frame rate. In such cases, the encoder can override the encoding configuration parameters at slice level to adapt the low hardware resource to keep the frame rate. Vice versa, when an encoder has increased hardware resources and needs to improve encoding quality, it can override parameters at slice level to adapt the high coding quality demand.

# Conclusions

Three modifications are proposed. First, additional syntax elements are proposed to be included in the constant slice header sett. Second, it is proposed to signal up to 7 different constant slice header parameter sets in the PPS (up from only 1 in VVC draft 6). According to the third modification an overriding mechanism is introduced at slice level. It is asserted that the proposed modifications allow selecting different default coding parameters for different picture partitions (sub-pictures, slices etc.) inside a same frame, hence increasing the amount of bit savings in the slice header. The overriding mechanism can be employed in the encoder to handle unexpected scenarios, such as running out of hardware resources. It is suggested to include the proposed modifications in the next version of VVC draft specification.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “Versatile Video Coding (Draft 6)”, JVET-O2001-vE, B. Bross, J. Chen, and S. Liu

- “AHG12: Flexible tiling", JVET-N0111, Y.-K. Wang, Hendry, M. Sychev