| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0340 |
| --- | --- |

| Title: | Non-CE4: Unification of merge interpolation filter (triangle and pairwise-average) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Antoine Robert, Fabrice Le-Léannec, Franck Galpin 975 Avenue des champs blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33.2.56.85.62.76 philippe.bordes@interdigital.com, fabrice.leleannec@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

It is proposed to unify the derivation of the variable hpelIfIdx (use of adaptive interpolation filter, AIF) in case of triangle merge and pairwise-average merge modes.

It is reported the proposed unifications in triangle and pairwise-average provides BD-rate gains of 0.01% and 0.06% in Luma in RA and LDB configurations, with same complexity as VTM-6.0.

# Introduction

In VTM-6.0 [1], in case of the resolution of the adaptive motion vector difference (amvr_precision_idx) is HALF_PEL and the motion vector value is half-pel, the adaptive interpolation filter (AIF) allows to select one among two different motion compensation filters. In this case, the variable hpelIfIdx indicate which interpolation filter to be used. The value of the adaptive motion vector difference resolution is stored into the motion-info buffer after the CU has been reconstructed (imv).

In case of non-merge inter modes, the value of hpelIfIdx is inferred to:

hpelIfIdx = (amvr_precision_idx == HALF_PEL) ? 1 : 0

In case of merge modes, the value of hpelIfIdx is inferred from the motion-info:

hpelIfIdx = (imv == HALF_PEL) ? 1 : 0

except in case of IBC and triangle-merge and pairwise-average merge:

- In case of triangle-merge, the value of hpelIfIdx is inferred to 0.

- In case of merge pairwise-average candidate, the value of hpelIfIdx is inferred to (hpelIfIdx-0== hpelIfIdx-1) ? hpelIfIdx-0 : 0, where hpelIfIdx-0 and hpelIfIdx-1 are the values of hpelIfIdx of cand-0 and cand-1 used to build the pairwise-average candidate.

# Proposals

## Interpolation filter with Triangle-merge

To unify the derivation of the index of interpolation filter (hpelIfIdx), it is proposed to enable the adaptive interpolation filter (AIF) in triangle-merge and to derive one value of hpelIfIdx for each triangle partition, with the value of hpelIfIdx associated with the candidate of each partition.

Table 1: summary of proposed interpolation filter with triangle-merge (VTM-6.0).

|  | Proposal difference with VTM-6.0 (AIF enabled with triangle-merge) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | -0.07% | 99% | 100% |  |  |  |  |  |
| Class A2 | 0.01% | -0.04% | -0.08% | 101% | 101% |  |  |  |  |  |
| Class B | -0.02% | 0.07% | 0.03% | 99% | 99% | -0.01% | -0.04% | -0.28% | 100% | 101% |
| Class C | -0.01% | -0.11% | -0.07% | 101% | 96% | -0.02% | -0.09% | 0.07% | 99% | 101% |
| Class E |  |  |  |  |  | 0.02% | 0.25% | 0.20% | 98% | 98% |
| Overall | -0.01% | -0.02% | -0.04% | 100% | 99% | 0.00% | 0.02% | -0.05% | 99% | 100% |
| Class D | -0.03% | -0.10% | -0.05% | 101% | 100% | 0.03% | 0.55% | 0.40% | 99% | 100% |
| Class F | -0.01% | 0.01% | 0.01% | 101% | 99% | 0.05% | 0.15% | 0.53% | 101% | 101% |

## Interpolation filter with pairwise-average merge

In VTM-5.0, in case of merge pairwise-average candidate, the value of hpelIfIdx is inferred to:

hpelIfIdx  = (hpelIfIdx-0== hpelIfIdx-1) ? hpelIfIdx-0 : 0

where hpelIfIdx0 and hpelIfIdx1 are the values of hpelIfIdx of the two first candidates of the list (cand-0 and cand-1). However, it may happen that hpelIfIdx-0 and hpelIfIdx-1 are not equal and only one of (cand-0, cand-1) is used to build the pairwise-merge motion values. This may turn out in deriving hpelIfIdx equal to 0 even if the one candidate used (cand-i) to build the pairwise-merge motion values has hpelIfIdx-i equal to 1.

To cope with this issue, it is proposed to derive hpelIfIdx as follows:

hpelIfIdx  = max ( hpelIfIdx-0 , hpelIfIdx-1 )

Proposed syntax change:

- The half sample interpolation filter index hpelIfIdxavgCand is derived as follows:

- If hpelIfIdxp0Cand is equal to hpelIfIdxp1Cand, hpelIfIdxavgCand is set equal to hpelIfIdxp0Cand.

- Otherwise, hpelIfIdxavgCand is set equal to 0.

- hpelIfIdxavgCand = max( hpelIfIdxp0Cand, hpelIfIdxp1Cand ).

Table 2: summary of proposed interpolation filter with pairwise-average and triangle-merge (VTM-6.0).

|  | Proposal difference with VTM-6.0 (AIF enabled with triangle-merge + proposed pairwise-average) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.10% | -0.04% | 100% | 99% |  |  |  |  |  |
| Class A2 | 0.03% | 0.08% | 0.05% | 101% | 101% |  |  |  |  |  |
| Class B | -0.02% | 0.05% | -0.05% | 98% | 100% | -0.01% | -0.05% | -0.09% | 101% | 101% |
| Class C | -0.02% | -0.02% | 0.02% | 98% | 99% | -0.04% | 0.01% | 0.06% | 100% | 99% |
| Class E |  |  |  |  |  | -0.17% | 0.07% | 0.40% | 99% | 100% |
| Overall | -0.01% | 0.05% | -0.01% | 99% | 100% | -0.06% | 0.00% | 0.08% | 100% | 100% |
| Class D | -0.05% | -0.09% | -0.01% | 99% | 98% | 0.02% | 0.16% | 0.12% | 101% | 99% |
| Class F | -0.01% | 0.00% | -0.01% | 101% | 101% | -0.02% | -0.14% | 0.24% | 99% | 100% |

# References

- Text of Versatile Video Coding (Draft 6), document JVET-O2001, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- High efficiency video coding (HEVC), combined format range extensions (RExt), scalability (SHVC), and multi-view (MV-HEVC) extensions, ITU-T H.265, April 2015.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).