| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 Oct. 2019 | Document: JVET-P0379-v1 |
| --- | --- |

| Title: | Non-CE6: A unified zero-out range for 4x4 LFNST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kui Fan, Li Zhang, Kai Zhang, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-265-9942 kui.fan@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

The current LFNST design defines the zero-out range according to transform block sizes. In this contribution, a unification of the zero-out ranges for 4×4 LFNST is proposed. With the proposed method, it is asserted that the computational complexity of LFNST could be saved by 50% for 4×N and N×4 blocks. Meanwhile, the on-chip memory size required by 4×4 LFNST is reported to be reduced by 50%. Experimental results reportedly show that the BD-rate changes are negligible, i.e., 0.03%, 0.01% for AI and RA configurations, respectively. For the low QP settings according to CE6 test conditions, the coding loss is reported to be 0.03% for AI configuration.

# Introduction

In the current design, eight 16×16 matrices and eight 16×48 matrices are stored for LFNST. These 16×16 matrices are used for the left-top 4×4 coefficients in 4×N or N×4 (N>=4) blocks. These 16×48 matrices are applied to the left-top 8×8 coefficients in 8×N or N×8 (N>=8) blocks. The memory costs of matrices in LFNST are summarized in Table 1.

Table 1. Memory costs of matrices in LFNST

| Transform size | Number of matrices | Memory cost (Byte) |
| --- | --- | --- |
| 16×16 | 8 | 2048 |
| 16×48 | 8 | 6144 |
| Total: 8192 |  |  |

In the current LFNST design, zero-out ranges are specified according to the transform block size, as shown in Table 2. The zero-out range is set to 8 for 4×4 and 8×8 blocks. Otherwise, the zero-out range is 16.

Table 2. Zero-out ranges for different transform block sizes

| Size | LFNST Size | Zero-out range |
| --- | --- | --- |
| 4×4 | 4×4 | 8 |
| 4×8/8×4 | 4×4 | 16 |
| 4×M/M×4 (M>8) | 4×4 | 16 |
| 8×8 | 8×8 | 8 |
| M×N (M>8, N>8) | 8×8 | 16 |

# Proposed methods

In this contribution, the zero-out ranges for 4×4 LFNST are unified to be 8. Specifically, the zero-out range is set to be 8 for 4×N and N×4 transform blocks. With the unification of zero-out ranges, the bottom half of 16×16 matrices for 4×4 LFNST is useless and can be removed.

The proposed unification of zero-out range for 4×4 LFNST has three benefits:

- Unify the zero-out value for 4×4 LFNST in all cases

- Reduce the required multiplication operations required by LFNST for 4×N and N×4 (N>4) blocks by 50%

- The memory cost of 4×4 LFNST matrices is reduced by 50%

Table 3 summarizes the differences between the proposed method and the current VVC design in terms of computational complexity and memory requirement.

Table 3. Comparison between the proposed method and the current design for 4×4 LFNST

|  | Transform size | Number of matrices | Memory cost (Byte) | Number of multiplication operations |
| --- | --- | --- | --- | --- |
| Current LFNST | 16×16 | 8 | 2048 | 16×16 = 256 |
| Proposed | 8×16 | 8 | 1024 | 16×8 = 128 |

# Experimental results

The experiments were carried out under “All Intra”, “Random Access”, and “Low Delay B” configurations as defined in the JVET Common Test Condition (CTC) [1] on top of VTM-6.0. In addition, the experimental results of low QP for AI configuration defined in Description of CE6 [2] are also reported.

Table 4 shows the CTC results of the proposed method. Table II shows the results of Low QP.

Table 4. CTC test results of the proposed method on top of VTM-6.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.12% | 0.02% | 101% | 98% |
| Class A2 | -0.01% | 0.14% | 0.13% | 100% | 99% |
| Class B | 0.03% | 0.10% | 0.03% | 101% | 100% |
| Class C | 0.04% | 0.03% | 0.15% | 99% | 101% |
| Class E | 0.07% | 0.13% | 0.09% | 100% | 107% |
| Overall | 0.03% | 0.10% | 0.08% | 100% | 101% |
| Class D | 0.00% | 0.24% | 0.08% | 98% | 102% |
| Class F | 0.04% | 0.09% | 0.09% | 99% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.06% | 0.06% | 101% | 100% |
| Class A2 | 0.03% | 0.06% | 0.07% | 101% | 99% |
| Class B | -0.01% | 0.15% | 0.00% | 101% | 102% |
| Class C | 0.01% | 0.13% | 0.15% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.11% | 0.07% | 101% | 101% |
| Class D | -0.03% | 0.00% | 0.15% | 100% | 105% |
| Class F | -0.02% | -0.01% | -0.01% | 99% | 99% |

Table 5. Low QP test results of the proposed method on top of VTM-6.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.11% | 0.10% | 98% | 100% |
| Class A2 | 0.01% | -0.01% | 0.00% | 97% | 100% |
| Class B | 0.02% | 0.01% | 0.02% | 97% | 100% |
| Class C | 0.01% | -0.02% | 0.05% | 97% | 104% |
| Class E | 0.10% | 0.07% | 0.06% | 97% | 100% |
| Overall | 0.03% | 0.03% | 0.04% | 97% | 101% |
| Class D | 0.01% | -0.01% | 0.02% | 97% | 104% |
| Class F | 0.01% | 0.00% | 0.02% | 98% | 102% |

# Proposed changes to working draft

The newly added parts on top of JVET-O2001 are highlighted in gray and the deleted parts are highlighted in gray strikethrough.

      - Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| if( lastSubBlock = = 0 && log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ] && lastScanPos > 0 ) |  |
| LfnstDcOnly = 0 |  |
| if( ( lastSubBlock > 0 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ¦ ¦ ( lastScanPos > 7 && ( ( log2TbWidth = = 2 ¦ ¦ log2TbHeight = = 2 ) ¦ ¦ ( log2TbWidth = = 3 ) && log2TbWidth = = log2TbHeight ) ) ) ) |  |
| LfnstZeroOutSigCoeffFlag = 0 |  |
| QState = 0 |  |
| … |  |

      - General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

When lfnst_idx[ xTbY ][ yTbY ] is not equal to 0 and both nTbW and nTbH are greater than or equal to 4, the following applies:

- The variables predModeIntra, nLfnstOutSize, log2LfnstSize, nLfnstSize, and nonZeroSize are derived as follows:

predModeIntra = ( cIdx = = 0 )  ?  IntraPredModeY[ xTbY ][ yTbY ]  :  IntraPredModeC[ xTbY ][ yTbY ] (8-965)

nLfnstOutSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  48  :  16	(8-966)

log2LfnstSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  3  :  2	(8-967)

nLfnstSize = 1  <<  log2LfnstSize		(8-968)

nonZeroSize = ( ( nTbW = = 4  &&| |  nTbH  = = 4 )  | |  ( nTbW = = 8  &&  nTbH  = = 8 ) )  ?  8  :  16	(8-969)

- When intra_mip_flag[ xTbComp ][ yTbComp ] is equal to 1 and cIdx is equal to 0, predModeIntra is set equal to INTRA_PLANAR.

#### 8.7.4.3 Low frequency non-separable transformation matrix derivation process

Inputs to this process are:

- a variable nTrS specifying the transform output length,

- a variable predModeIntra specifying the intra prediction mode for LFNST set selection,

- a variable lfnstIdx specifying the LFNST index for transform selection in the selected LFNST set.

Output of this process is the transformation matrix lowFreqTransMatrix.

The variable lfnstTrSetIdx is specified in Table 8-16 depending on predModeIntra.

Table 8-16 – Specification of lfnstTrSetIdx

| predModeIntra | lfnstTrSetIdx |
| --- | --- |
| predModeIntra < 0 | 1 |
| 0 <= predModeIntra <= 1 | 0 |
| 2 <= predModeIntra <= 12 | 1 |
| 13 <= predModeIntra <= 23 | 2 |
| 24 <= predModeIntra <= 44 | 3 |
| 45 <= predModeIntra <= 55 | 2 |
| 56 <= predModeIntra <= 80 | 1 |

The transformation matrix lowFreqTransMatrix is derived based on nTrS, lfnstTrSetIdx, and lfnstIdx as follows:

- If nTrS is equal to 16, lfnstTrSetIdx is equal to 0, and lfnstIdx is equal to 1, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{  108  -44  -15    1  -44   19    7   -1  -11    6    2   -1    0   -1   -1    0  }

{  -40  -97   56   12  -11   29  -12   -3   18   18  -15   -3   -1   -3    2    1  }

{   25  -31   -1    7  100  -16  -29    1  -54   21   14   -4   -7    2    4    0  }

{  -32  -39  -92   51   -6  -16   36   -8    3   22   18  -15    4    1   -5    2  }

{    8   -9   33   -8  -16 -102   36   23   -4   38  -27   -5    5   16   -8   -6  }

{  -25    5   16   -3  -38   14   11   -3  -97    7   26    1   55  -10  -19    3  }

{    8    9   16    1   37   36   94  -38   -7    3  -47   11   -6  -13  -17   10  }

{    2   34   -5    1   -7   24  -25   -3    8   99  -28  -29    6  -43   21   11  }

{  -16  -27  -39 -109    6   10   16   24    3   19   10   24   -4   -7   -2   -3  }

{   -9  -10  -34    4   -9   -5  -29    5  -33  -26  -96   33   14    4   39  -14  }

{  -13    1    4   -9  -30  -17   -3  -64  -35   11   17   19  -86    6   36   14  }

{    8   -7   -5  -15    7  -30  -28  -87   31    4    4   33   61   -5  -17   22  }

{   -2   13   -6   -4   -2   28  -13  -14   -3   37  -15   -3   -2  107  -36  -24  }

{    4    9   11   31    4    9   16   19   12   33   32   94   12    0   34  -45  }

{    2   -2    8  -16    8    5   28  -17    6   -7   18  -45   40   36   97   -8  }

{    0   -2    0  -10   -1   -7   -3  -35   -1   -7   -2  -32   -6  -33  -16 -112  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 0, and lfnstIdx is equal to 2, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{  119  -30  -22   -3  -23   -2    3    2  -16    3    6    0   -3    2    1    0  }

{  -27 -101   31   17  -47    2   22    3   19   30   -7   -9    5    3   -5   -1  }

{    0   58   22  -15 -102    2   38    2   10  -13   -5    4   14   -1   -9    0  }

{   23    4   66  -11   22   89   -2  -26   13   -8  -38   -1   -9  -20   -2    8  }

{  -19   -5  -89    2  -26   76  -11  -17   20   13   18   -4    1  -15    3    5  }

{  -10   -1   -1    6   23   25   87   -7  -74    4   39   -5    0   -1  -20   -1  }

{  -17  -28   12   -8  -32   14  -53   -6  -68  -67   17   29    2    6   25    4  }

{    1  -24  -23    1   17   -7   52    9   50  -92  -15   27  -15  -10   -6    3  }

{   -6  -17   -2 -111    7  -17    8  -42    9   18   16   25   -4    2   -1   11  }

{    9    5   35    0    6   21   -9   34   44   -3  102   11   -7   13   11  -20  }

{    4   -5   -5  -10   15   19   -2    6    6  -12  -13    6   95   69  -29  -24  }

{   -6   -4   -9  -39    1   22    0  102  -19   19  -32   30  -16  -14   -8  -23  }

{    4   -4    7    8    4  -13  -18    5    0    0   21   22   58  -88  -54   28  }

{   -4   -7    0  -24   -7    0  -25    3   -3  -30    8  -76  -34    4  -80  -26  }

{    0    6    0   30   -6    1  -13  -23    1   20   -2   80  -44   37  -68    1  }

{    0    0   -1    5   -1   -7    1  -34   -2    3   -6   19    5  -38   11 -115  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 1, and lfnstIdx is equal to 1, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{ -111   39    4    3   44   11  -12   -1    7  -16   -5    2    3   -1    4    2  }

{  -47  -27   15   -1  -92   43   20   -2   20   39  -16   -5   10   -5  -13    2  }

{  -35  -23    4    4  -17  -72   32    6  -59   18   50   -6    0   40    0  -13  }

{   13   93  -27   -4  -48   13  -34    4  -52   11    1   10    3   16   -3    1  }

{  -11  -27    1    2  -47   -4  -36   10   -2  -85   14   29  -20   -2   57    4  }

{    0  -35   32   -2   26   60   -3  -17  -82    1  -30    0  -37   21    3   12  }

{  -17  -46  -92   14    7  -10  -39   29  -17   27  -28   17    1  -15  -13   17  }

{    4  -10  -23    4   16   58  -17   26   30   21   67    2  -13   59   13  -40  }

{    5  -20   32   -5    8   -3  -46   -7   -4    2  -15   24  100   44    0    5  }

{   -4   -1   38  -18   -7  -42  -63   -6   33   34  -23   15  -65   33  -20    2  }

{   -2  -10   35  -19    5    8  -44   14  -25   25   58   17    7  -84  -16  -18  }

{    5   13   18   34   11   -4   18   18    5   58   -3   42   -2  -10   85   38  }

{   -5   -7  -34  -83    2   -1   -4  -73    4   20   15  -12    4   -3   44   12  }

{    0    4   -2  -60    5    9   42   34    5  -14    9   80   -5   13  -38   37  }

{   -1    2    7  -57    3   -7    9   68   -9    6  -49  -20    6   -4   36  -64  }

{   -1    0  -12   23    1   -4   17  -53   -3    4  -21   72   -4   -8   -3  -83  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 1, and lfnstIdx is equal to 2, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{   88  -55    6   -3  -66   27    9   -2   11   11  -13    1   -2   -7    1    2  }

{  -58  -20   27   -2  -27   75  -29    0   47  -42  -11   11   -9   -3   19   -4  }

{  -51   23  -22    5  -63    3   37   -5    1   64  -35   -4   29  -31  -11   13  }

{  -27  -76   49   -2   40   14    9  -17  -56   36  -25    6   14    3   -6    8  }

{   19   -4  -36   22   52    7   36  -23   28  -17  -64   15   -5  -44   48    9  }

{   29   50   13  -10    1   34  -59    1  -51    4  -16   30   52  -33   24   -5  }

{  -12  -21  -74   43  -13   39   18   -5  -58  -35   27   -5   19   26    6   -5  }

{   19   38  -10   -5   28   66    0   -5   -4   19  -30  -26  -40   28  -60   37  }

{   -6   27   18   -5  -37  -18   12  -25  -44  -10  -38   37  -66   45   40   -7  }

{  -13  -28  -45  -39    0   -5  -39   69  -23   16  -12  -18  -50  -31   24   13  }

{   -1    8   24  -51  -15   -9   44   10  -28  -70  -12  -39   24  -18   -4   51  }

{   -8  -22  -17   33  -18  -45  -57  -27    0  -31  -30   29   -2  -13  -53   49  }

{    1   12   32   51   -8    8   -2  -31  -22    4   46  -39  -49  -67   14   17  }

{    4    5   24   60   -5  -14  -23   38    9    8  -34  -59   24   47   42   28  }

{   -1   -5  -20  -34    4    4  -15  -46   18   31   42   10   10   27   49   78  }

{   -3   -7  -22  -34   -5  -11  -36  -69   -1   -3  -25  -73    5    4    4  -49  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 2, and lfnstIdx is equal to 1, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{ -112   47   -2    2  -34   13    2    0   15   -7    1    0    8   -3   -1    0  }

{   29   -7    1   -1 -108   40    2    0  -45   13    4   -1    8   -5    1    0  }

{  -36  -87   69  -10  -17  -33   26   -2    7   14  -11    2    6    8   -7    0  }

{   28   -5    2   -2  -29   13   -2    0  103  -36   -4    1   48  -16   -4    1  }

{  -12  -24   15   -3   26   80  -61    9   15   54  -36    2    0   -4    6   -2  }

{   18   53   69  -74   14   24   28  -30   -6   -7  -11   12   -5   -7   -6    8  }

{    5   -1    2    0  -26    6    0    1   45   -9   -1    0 -113   28    8   -1  }

{  -13  -32   18   -2   15   34  -27    7  -25  -80   47   -1  -16  -50   28    2  }

{   -4  -13  -10   19   18   46   60  -48   16   33   60  -48    1    0    5   -2  }

{   15   33   63   89    8   15   25   40   -4   -8  -15   -8   -2   -6   -9   -7  }

{   -8  -24  -27   15   12   41   26  -29  -17  -50  -39   27    0   35  -67   26  }

{   -2   -6  -24   13   -1   -8   37  -22    3   18  -51   22  -23  -95   17   17  }

{   -3   -7  -16  -21   10   24   46   75    8   20   38   72    1    2    1    7  }

{    2    6   10   -3   -5  -16  -31   12    7   24   41  -16  -16  -41  -89   49  }

{    4    8   21   40   -4  -11  -28  -57    5   14   31   70    7   18   32   52  }

{    0    1    4   11   -2   -4  -13  -34    3    7   20   47   -6  -19  -42 -101  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 2, and lfnstIdx is equal to 2, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{  -99   39   -1    2   65  -20   -5    0  -15   -2    5   -1    0    3   -1    0  }

{   58   42  -33    3   33  -63   23   -1  -55   32    3   -5   21   -2   -8    3  }

{  -15   71  -44    5  -58  -29   25    3   62   -7   -4   -4  -19    4    0    1  }

{   46    5    4   -6   71  -12  -15    5   52  -38   13   -2  -63   23    3   -3  }

{  -14  -54  -29   29   25   -9   61  -29   27   44  -48    5  -27  -21   12    7  }

{   -3    3   69  -42  -11  -50  -26   26   24   63  -19   -5  -18  -22   12    0  }

{   17   16   -2    1   38   18  -12    0   62    1  -14    5   89  -42    8   -2  }

{   15   54   -8    6    6   60  -26   -8  -30   17  -38   22  -43  -45   42   -7  }

{   -6  -17  -55  -28    9   30   -8   58    4   34   41  -52  -16  -36  -20   16  }

{   -2   -1   -9  -79    7   11   48   44  -13  -34  -55    6   12   23   20  -11  }

{    7   29   14   -6   12   53   10  -11   14   59  -15   -3    5   71  -54   13  }

{   -5  -24  -53   15   -3  -15  -61   26    6   30  -16   23   13   56   44  -35  }

{    4    8   21   52   -1   -1   -5   29   -7  -17  -44  -84    8   20   31   39  }

{   -2  -11  -25   -4   -4  -21  -53    2   -5  -26  -64   19   -8  -19  -73   39  }

{   -3   -5  -23  -57   -2   -4  -24  -75    1    3    9  -25    6   15   41   61  }

{    1    1    7   18    1    2   16   47    2    5   24   67    3    9   25   88  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 3, and lfnstIdx is equal to 1, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{ -114   37    3    2  -22  -23   14    0   21  -17   -5    2    5    2   -4   -1  }

{  -19  -41   19   -2   85  -60  -11    7   17   31  -34    2  -11   19    2   -8  }

{   36  -25   18   -2  -42  -53   35    5   46  -60  -25   19    8   21  -33   -1  }

{  -27  -80   44   -3  -58    1  -29   19  -41   18  -12   -7   12  -17    7   -6  }

{  -11  -21   37  -10   44   -4   47  -12  -37  -41   58   18   10  -46  -16   31  }

{   15   47   10   -6  -16  -44   42   10  -80   25  -40   21  -23   -2    3  -14  }

{   13   25   79  -39  -13   10   31   -4   49   45   12   -8    3   -1   43    7  }

{   16   11  -26   13  -13  -74  -20   -1    5   -6   29  -47   26  -49   54    2  }

{   -8  -34  -26    7  -26  -19   29  -37    1   22   46   -9  -81   37   14   20  }

{   -6  -30  -42  -12   -3    5   57  -52   -2   37  -12    6   74   10    6  -15  }

{    5    9   -6   42  -15  -18   -9   26   15   58   14   43   23  -10  -37   75  }

{   -5  -23  -23   36    3   22   36   40   27   -4  -16   56  -25  -46   56  -24  }

{    1    3   23   73    8    5   34   46  -12    2   35  -38   26   52    2  -31  }

{   -3   -2  -21  -52    1  -10  -17   44  -19  -20   30   45   27   61   49   21  }

{   -2   -7  -33  -56   -4   -6   21   63   15   31   32  -22  -10  -26  -52  -38  }

{   -5  -12  -18  -12    8   22   38   36   -5  -15  -51  -63   -5    0   15   73  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 3, and lfnstIdx is equal to 2, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{ -102   22    7    2   66  -25   -6   -1  -15   14    1   -1    2   -2    1    0  }

{   12   93  -27   -6  -27  -64   36    6   13    5  -23    0   -2    6    5   -3  }

{  -59  -24   17    1  -62   -2   -3    2   83  -12  -17   -2  -24   14    7   -2  }

{  -33   23  -36   11  -21   50   35  -16  -23  -78   16   19   22   15  -30   -5  }

{    0  -38  -81   30   27    5   51  -32   24   36  -16   12  -24   -8    9    1  }

{   28   38    8   -9   62   32  -13    2   51  -32   15    5  -66   28    0   -1  }

{   11  -35   21  -17   30  -18   31   18  -11  -36  -80   12   16   49   13  -32  }

{  -13   23   22  -36  -12   64   39   25  -19   23  -36    9  -30  -58   33   -7  }

{   -9  -20  -55  -83    3   -2    1   62    8    2   27  -28    7   15  -11    5  }

{   -6   24  -38   23   -8   40  -49    0   -7    9  -25  -44   23   39   70   -3  }

{   12   17   17    0   32   27   21    2   67   11   -6  -10   89  -22  -12   16  }

{    2   -9    8   45    7   -8   27   35   -9  -31  -17  -87  -23  -22  -19   44  }

{   -1   -9   28  -24   -1  -10   49  -30   -8   -7   40    1    4   33   65   67  }

{    5  -12  -24  -17   13  -34  -32  -16   14  -67   -7    9    7  -74   49    1  }

{    2   -6   11   45    3  -10   33   55    8   -5   59    4    7   -4   44  -66  }

{   -1    1  -14   36   -1    2  -20   69    0    0  -15   72    3    4    5   65  }

},

# Conclusion

In this contribution, we propose to unify the zero-out range for all the 4×4 LFNST cases regardless of the transform block size. The proposed method cuts off the memory cost of matrices for 4×4 LFNST by half with marginal performance loss in the AI, RA, and LDB configurations. It is recommended to adopt this method in the next VVC WD and VTM software.

# Reference

[1]	F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

[2] H. Egilmez, M. Salehifar, X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signalling,” JVET-O2026, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 15th Meeting: Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).