| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0395 |
| --- | --- |

| Title: | CABAC zero word thresholds |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Steve Keating Karl Sharman | Email: | adrian.browne@sony.com steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This document describes a modified coding method for padding with cabac_zero_word.  The current specification uses a single constraint for the number of bins allowed in a picture based on the picture size and the size of the encoded stream.  Following a suggestion from the 15th JVET meeting in Gothenburg, an empirical study has been completed which compares the effect of the cabac_zero_word constraint on HM and VTM.  In addition, three further configurations of VTM were studied – one with dependent/trellis quantization disabled, one with certain coefficient coding thresholds disabled, and one with both dependent quantization and coefficient coding thresholds disabled.

It is proposed that the cabac_zero_word constraint is modified due to the integration of the dependent quantization tool. If, later, when profiles are defined, some profiles do not permit dependent quantization, the current lower levels could be used in order to simplify decoder design.

This contribution also demonstrates the effect of the coefficient coding threshold constraints (JVET-N0280 & JVET-O0052) on the bin-to-bit ratios.

# Introduction

Section 7.4.3.11 of the specification [1] states the following:

Let NumBytesInVclNalUnits be the sum of the values of NumBytesInNalUnit for all VCL NAL units of a coded picture.

Let BinCountsInNalUnits be the number of times that the parsing process function DecodeBin( ), specified in clause TBD, is invoked to decode the contents of all VCL NAL units of a coded picture.

Let the variable RawMinCuBits be derived as follows:

RawMinCuBits = MinCbSizeY * MinCbSizeY *
				( BitDepthY + 2 * BitDepthC / ( SubWidthC * SubHeightC ) )	(7-39)

The value of BinCountsInNalUnits shall be less than or equal to ( 32 ÷ 3 ) * NumBytesInVclNalUnits + ( RawMinCuBits * PicSizeInMinCbsY )  32.

NOTE – The constraint on the maximum number of bins resulting from decoding the contents of the coded slice NAL units can be met by inserting a number of cabac_zero_word syntax elements to increase the value of NumBytesInVclNalUnits. Each cabac_zero_word is represented in a NAL unit by the three-byte sequence 0x000003 (as a result of the constraints on NAL unit contents that result in requiring inclusion of an emulation_prevention_three_byte for each cabac_zero_word).

In section 7.3.2.11 of the specification it states that cabac_zero_word may be added at the end of each slice to maintain a ratio of bits to bins, such that a decoder has a limit with which to work.

During the Gothenburg meeting JVET-O0517 [3] was presented and it was agreed that the proposal should be further studied along with the formula and threshold value of the constraint [4].  Discussion during the presentation of JVET-O0520 [5] suggested that an issue with increased generation of GT1 flags might be caused by dependent/trellis quantization and that this problem might be resolved by a change to the cabac_zero_word formula.

In addition, a previous study, JVET-N0049 [6], identified that VTM4.0 had a higher bit to bin ratio than HM16.19 but did not identify any likely causes and the meeting notes state that further study was necessary [7]

# Study

VTM6.1 and HM16.20 were instrumented with code to log the number of bits and bins within each picture / slice.  In addition instrumentation was added to enable the number of bits and bins used within each CTU to be logged.  For the majority of this paper results at the CTU level (128x128 for VTM, 64x64 for HM) are used for the following reasons:

- The number of square CTUs is in excess of 100 times the number of frames in CTC. The greater number of data points increases the diversity of data analysed.

- Individual areas in pictures with different bit/bin characteristics can be identified.

- It is not safe to rely on the averaging effect over a large image. In fact, it is more likely that the initial tuning of CABAC relieves the bin-to-bit ratio, and therefore, as observed in the CTCs the problem is likely to be more prevalent for larger formats.

- Comparison between different classes is possible as each data point represents the same quantity of pixels

- The results on a per-CTU case can be scaled up to pictures.  Any constraint which is suitable for all CTUs in an image must also be applicable to the picture made up of these CTUs.

- It also turned out that the picture averages followed the trends of the individual CTUs.

VTM6.1 was studied in its standard configuration and also in the following configurations:

- With dependent quantization disabled (DepQuant=0)

- With limits on the maximum number of context coded bins per coefficient or coefficient sub-block defined in JVET-N0280 [8] and JVET-O0052 [9] removed.

- Both 1) and 2)

Each of these five options were then simulated at both CTC QPs (22,27,32,27) and low QPs (2, 7, 12, 17) for the number of frames defined in the CTC / low QP test conditions [1] for all intra, random access and low delay (B).

On a per frame basis, the following results were obtained:

| QPs under test | Percent of frames over cabac_zero_word_threshold |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM 6.1 | HM 16.20 | VTM 6.1 -DepQuant (1) | VTM 6.1 no bin limit (2) | VTM 6.1 -DepQuant no bin limit (3) |
| 22, 27, 32,37 | 0.001% | 0.000% | 0.000% | 0.003% | 0.000% |
| 2, 7, 12, 17 | 0.713% | 0.000% | 0.000% | 1.196% | 0.000% |

Table 1: Percent of frames over cabac_zero_word_threshold

From table 1, it can be seen that under CTC, the number of frames over the cabac_zero_word threshold is much greater for VTM 6.1 than HM 16.20 under low QP conditions.  However, if dependent quantization is disabled the number of frames over the threshold returns to zero.  Even when the coefficient bin limit is removed, thus increasing the number of frames over the threshold (when compared to VTM 6.1), disabling dependent quantization returns the number of frames to zero.

On a per CTU basis (64x64 for HM; 128x128 for VTM), the following results were obtained:

| QPs under test | Percent of CTUs over cabac_zero_word_threshold |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM 6.1 | HM 16.20 | VTM 6.1 -DepQuant (1) | VTM 6.1 no bin limit (2) | VTM 6.1 -DepQuant no bin limit (3) |
| 22, 27, 32, 37 | 0.226% | <0.001% | 0.000% | 0.229% | 0.000% |
| 2, 7, 12, 17 | 3.701% | 0.001% | 0.001% | 4.091% | 0.002% |

Table 2: Percent of CTUs over cabac_zero_word_threshold

Comparing table 1 and table 2, the same pattern of the number of frames or CTUs over the cabac_zero_word threshold can be observed.  The figures for VTM 6.1 without dependent quantization and HM 16.20 are similar for both frame and CTU based cases.  The percentages are higher for CTUs than for frames, but this can be explained by the averaging effect of many CTUs over a frame.

If plotted graphically, the trends in the results can be seen.  In the following figures the number of coded context bins is plotted against the number of bits in the output stream.  Points in the red zone represent CTUs which are over the cabac_zero_word threshold.  The colour of each dot is determined from HSB where the hue of the individual dots represents the dominant QP of all CTUs at that location, and the brightness represents the number of CTUs at that location.

Figure 1. Bins vs bits in encoded stream for VTM 6.1.

Figure 2. Bins vs bits in encoded stream for HM 16.20.

Figure 3. Bins vs bits in encoded stream for VTM 6.1 with dependent quantization disabled.

Figure 4. Bins vs bits in encoded stream for VTM 6.1 without coefficient context bin limits.

Figure 5. Bins vs bits in encoded stream for VTM 6.1 with dependent quantization disabled and without coefficient context bin limits.

It can be seen from the graphs for HM16.20 (figure 2) and VTM 6.1 without dependent quantization (figure 3) are similar and keep out of the red zone.  However, the standard VTM6.1 (figure 1) crosses the threshold for some CTUs between 10K bits and 40Kbits long.  Beyond this limit, the trend of the VTM6.1 graph is less than the 4÷3 bit to bin ratio.

The graph for VTM 6.1 without context bin limits (figure 4) is similar to the graph of VTM6.1 (figure 1) for lower bit rates, but for higher bits rates there is a greater spread of bin to bit ratios, with a higher average number of bins to bits, without exceeding the threshold.  The same increase is also visible when comparing VTM 6.1 without dependent quantization and context bin limits (figure 5) with VTM 6.1 without dependent quantization (figure 3).  These comparisons imply that the context bin limits imposed by JVET-N0280 and JVET-O0052 have more effect for CTUs with higher bit rates that do not exceed the bin to bit threshold, than for CTUs with more moderate bit rates that do exceed this threshold.

# Proposal

From the results presented above, it is proposed that if dependent quantization is not available no change to the threshold and formula for cabac_zero_word is required.

If dependent quantization is available, it is proposed that the formula for cabac_zero_word needs updating, and that it should be different from the formula when dependent quantization is not available.

In order to update the formula new values must be determined for the two terms in the formula, the scaling factor of the initial offset (currently 1/32) and the bin-to-bit ratio (currently 4÷3).  As figure 1 shows that CTUs which exceed the cabac_zero_word threshold are at lower numbers of bit encoded the offset value should be determined first.

Figure 6 shows the same data as figure 1with the data normalised for a bin to bit ratio of 4÷3.   The y-axis thus becomes the offset required in the cabac_zero_word formula. The horizontal lines represent offset ratios of 1/32 (the existing value, in red), 3/64 (in blue) and 1/16 (in green).  From the figure it can be seen that some CTUs exceed the offset ratio of 3/64 but none exceed 1/16.  It also shows that for larger numbers of encoded bits the required offset goes negative which implies that the 4÷3 ratio is too high.

Figure 6. Required bin offset vs bits in encoded stream for VTM 6.1 with a bin to bit ratio of 4÷3.

Figures 7 and 8 show the same data as figures 1 and 6 but with the data normalized for offset ratios of 3/64 and 1/16 respectively.  It can be seen from the graph that with an offset ratio of 1/16 the bin-to-bit ratio can be reduced to 5÷4 with all CTUs well below the threshold.

Figure 7. Bin to bit ratio vs bits in encoded stream for VTM 6.1 with an offset ratio of 3 / 64.

Figure 8. Bin to bit ratio vs bits in encoded stream for VTM 6.1 with an offset ratio of 1 / 16.

Therefore, if dependent quantization is available, it is proposed that the threshold for cabac_zero_word should be changed from:

( 32 ÷ 3 ) * NumBytesInVclNalUnits + ( RawMinCuBits * PicSizeInMinCbsY )  32

to:

10 * NumBytesInVclNalUnits + ( RawMinCuBits * PicSizeInMinCbsY )  16

i.e.

1.25 * NumBitsInVclNalUnits + ( RawMinCuBits * PicSizeInMinCbsY )  16

These changes are only because of the presence of dependent quantization, and they increase the bin/bit ratio, therefore burdening a decoder. Therefore, it is recommended that if, in the future, profiles/sub-profiles are defined that do not require dependent quantization, the original equations could still be used for those cases.

# Observation 1 – Role of context bin limiting

JVET-N0280 and JVET-O052 attempt to limit the context coded bins used for coefficient coding.  However, if analysed for their effect in minimizing the usage of cabac_zero_word, the results can be seen by considering table 3, by comparing figures 3 and 5 (dependent quantization disabled), and by comparing figures 9 and 10 (dependent quantization enabled).  Although the number of bins is reduced by these proposals, the effect is not in the region which generates cabac_zero_word, but in CTUs which generate longer bit streams with bin counts below the threshold.

| QPs under test | Percent of CTUs over cabac_zero_word threshold |  |  |  |
| --- | --- | --- | --- | --- |
|  | Dependent quantization enabled with proposed cabac_zero_word threshold |  | Dependent quantization disabled with original cabac_zero_word threshold |  |
|  | VTM 6.1 | VTM 6.1 no limit | VTM 6.1 | VTM 6.1 no limit |
| 22, 27, 32, 37 | 0.000% | 0.000% | 0.000% | 0.000% |
| 2, 7, 12, 17 | 0.000% | 0.000% | 0.001% | 0.002% |

Table 3: Percent of CTUs over cabac_zero_word threshold using proposed update

Figure 9. Bins vs bits in encoded stream for VTM 6.1 using updated cabac_zero_word threshold.

Figure 10. Bins vs bits in encoded stream for VTM 6.1 without coefficient context bin limits using updated cabac_zero_word threshold.

# Observation 2 – Discrepancy in VTM 6.1 and HM 16.20 versus specification

In the specification equation 7-39 is given as:

RawMinCuBits = MinCbSizeY * MinCbSizeY *
				( BitDepthY + 2 * BitDepthC / ( SubWidthC * SubHeightC ) )

i.e. multiply BitDepthC by 2 then divide by ( SubWidthC * SubHeightC ) using the defined operator precedence.

Whereas VTM 6.1 and HM 16.20 implement this as:

const int rawBits = paddedWidth * paddedHeight * (sps.getBitDepth(CHANNEL_TYPE_LUMA) +       2*(sps.getBitDepth(CHANNEL_TYPE_CHROMA)>>log2subWidthCxsubHeightC));

i.e. divide sps.getBitDepth(CHANNEL_TYPE_CHROMA) by subWidthCxsubHeightC then multiply by 2.

This discrepancy results in rawBits being 14/15 of the correct value when using 10 bit chroma samples.

The code has been patched on a branch to match the specification and it is strongly suggested that this is integrated into VTM 6.2 / VTM 7.0 / HM 16.21. A merge request has been made to JCT-VC’s HM.

All the results above are based on the VTM 6.1 / HM 16.20 not the patched branch.  However, for information tables 4 and 5 represents the same data as table 1 and 2, but with the patched code.  The magnitude of the difference between the tables does not materially effect the proposed changes.

| QPs under test | Percent of frames over original cabac_zero_word_threshold |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM 6.1 | HM 16.20 | VTM 6.1 -DepQuant (1) | VTM 6.1 no bin limit (2) | VTM 6.1 -DepQuant no bin limit (3) |
| 22, 27, 32, 37 | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| 2, 7, 12, 17 | 0.167% | 0.000% | 0.000% | 0.235% | 0.000% |

Table 4: Percent of frames over cabac_zero_word_threshold in patched branch.

| QPs under test | Percent of CTUs over original cabac_zero_word_threshold |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM 6.1 | HM 16.20 | VTM 6.1 -DepQuant (1) | VTM 6.1 no bin limit (2) | VTM 6.1 -DepQuant no bin limit (3) |
| 22, 27, 32, 37 | 0.154% | 0.000% | 0.000% | 0.156% | 0.000% |
| 2, 7, 12, 17 | 2.553% | <0.001% | <0.001% | 2.794% | <0.001% |

Table 5: Percent of CTUs over cabac_zero_word_threshold in patched branch.

# Conclusion

A study has been completed which analysed the context coded bin to bit ratio in VTM 6.1, HM 16.20 and VTM 6.1 with dependent quantization disabled, VTM 6.1 with coefficient bin limiting disabled and VTM 6.1 with both dependent quantization and coefficient bins limiting disabled in both CTC and low QP test conditions.

As a result of the study it is proposed that two different formulas are used for the threshold for padding with cabac_zero_word.  One formula should be used when dependent quantization is available, and the other (which is the same as the existing formula) should be used when dependent quantization is not available.

Additionally, it has been observed that the context bin limits introduced in JVET-O0520 and JVET-N0280 reduce the number of context bins, but tend to target CTUs which do not exceed the cabac_zero_word threshold.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “JVET common test conditions and software reference configurations”, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring

- “Versatile Video Coding (Draft 5)”, JVET-N1001-v8, B. Bross, J. Chen, and S. Liu.

- “Slice/tile level CABAC zero-word constraints”, JVET-O0517, A. Browne, S. Keating and K. Sharman.

- “Meeting Report of the 15th JVET Meeting (Gothenburg, SE, 3–12 July 2019)”, JVET-O2000, G. J. Sullivan and J.-R. Ohm.

- “GT1 flag coding”, JVET-O0520, A. Browne, S. Keating and K. Sharman.

- “AHG16/Non-CE7: A study of bin to bit ratio in VTM4.0 and HM16.19”, JVET-N0049, M.Zhou

- “Meeting Report of the 14th JVET Meeting”, JVET-N1000, G. J. Sullivan and J.-R. Ohm.

- “CE8: Residual Coding for Transform Skip Mode (CE8-4.3a, CE8-4.3b, CE8-4.4a, and CE8-4.4b)”, JVET-N0280, B. Bross, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand.

- “CE7-1: TB-level constraints on context-coded bins for coefficient coding”, JVET-O0052, T.-D. Chuang, C.-Y. Chen, S.-T. Hsiang, Y.-W. Huang, S.-M. Lei, M. Coban, M. Karczewicz.