| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0400 |
| --- | --- |

| Title: | AhG16/Non-CE8: Removal of shared merge list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang, Li Zhang, Hongbin Liu, Kai Zhang, Jizheng Xu, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | wangyang.cs@bytedance.com lizhang.idm@bytedance.com liuhongbin.01@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to increase the MV throughput of small blocks with IBC enabled. In the proposed method, the shared merge list is removed, and the derivation of spatial merge candidates for 4×4 blocks is skipped. Therefore, the pruning operations among BV candidates are totally removed. Additionally, updating of HMVP tables for certain block sizes (no greater than 16) is skipped as well. It is asserted that the proposed method could further reduce the complexity and increase the MV throughput. Simulation results reportedly show that BD-rate changes are {0.01%, -0.01%, -0.03%} and {0.06%, 0.02%, -0.11%} in {AI, RA, LDB} configurations for class F and class TGM, respectively.

Introduction

In current VVC, the block vector (BV) predictors for merge and AMVP mode in IBC share a common BV candidate list, which consists of the following elements:

- 2 spatial candidates from neighbouring positions (denoted by A1, B1 as in Figure 1): 1 pruning

- 5 HMVP entries: 2 prunings

- Zero vectors

Figure 1. Positions of spatial merge candidates.

To improve the throughput for small blocks, shared merge list was adopted in the 13rd JVET meeting [1] but has been removed from regular merge list in the 14th JVET meeting. In the current design, shared merge list is only used for IBC BV candidate list.

With the shared merge list, the merge sharing node will be decided for each CU inside a CTU during the parsing stage. A node is determined to be a merge sharing node if all the following three conditions are satisfied:

- The node is the parent node of leaf CUs.

- The node size is equal to or larger than 32.

- At least one of the child CU split from the node has a size smaller than 32 (i.e., equal to 16).

Figure 2 shows five examples of merge sharing nodes supported in the latest VVC working draft. The shared merging candidate list is generated for the dotted virtual CU (i.e., a merge sharing node).

Figure 2. Examples of merge sharing nodes.

Shared merge list could be removed due to the following reasons. Firstly, the location and size of a merge sharing node or a mask to indicate whether current node is the merge sharing node must be saved, which causes the latency of parsing. Secondly, all CUs must be checked whether it is one of the leaf CUs in a merge sharing node, which causes more complex logic dependency on ancestor CUs. Thirdly, it is possible that when meeting a merge sharing node, a BV candidate list is constructed while all CUs under this node are not coded with IBC mode. Thus, it is a waste of computational complexity. Finally, for a small region, it may contain one or more merge sharing nodes, therefore, BV candidate list may need to be constructed multiple times. Besides, the current VTM dose not support shared merge list at encoder side since each CU has to construct the BV candidate list individually.

To tackle above drawbacks and further improve the MV throughput, it is proposed to remove the shared merge list.

Proposed methods

In this contribution, it is proposed to remove the shared merge list for IBC and enable parallel processing for IBC merge candidate list construction on small blocks.

The shared merge list is totally removed, and derivation of spatial merge candidates for 4×4 blocks is skipped. Therefore, the pruning operations among BV candidates are totally removed. Additionally, updating of HMVP tables for certain block size (i.e. no greater than 16) is skipped as well.

Complexity analysis

Complexity analysis under two cases are provided in this subsection. The first case focuses on the shared merge region (SMR) itself. The second case provides additional data for a given small region which may include at least one shared merge region.

Complexity analysis for SMRs

Complexity analysis of proposed method and current VTM design is tabulated in Table 1. Here, the number of spatial neighbours to be accessed, the number of pruning, and the number of IBC merge list construction for each method (e.g. VTM design using shared merge list and proposed method) are summarized.

Table 1: Complexity analysis of VTM design and proposed method (for shared region)

| CU size, split type | Methods | Number of spatial neighbours to be accessed | Number of pruning | Number of BV list construction |
| --- | --- | --- | --- | --- |
| 8×8, QT | VTM | 2 | 3 | 1 |
|  | Proposed | 0 | 0 | 0 |
|  | Saving | 100% | 100% | 100% |
| 8×4/4×8, BT | VTM | 2 | 3 | 1 |
|  | Proposed | 0 | 0 | 0 |
|  | Saving | 100% | 100% | 100% |
| 16×4/4×16, TT | VTM | 2 | 3 | 1 |
|  | Proposed | 2 | 3 | 1 |
|  | Saving | 0% | 0% | 0% |

Complexity analysis for small regions with at least one SMR

The proposed method works well for all kinds of test cases, not only for the shared merge regions.

In one example, if one small region contains multiple SMRs as depicted in Figure 3(a), two BV candidate lists are still required to be constructed in current VVC. In addition, there are dependency among these two 8×4 sub-regions. While with the proposed method, 0 candidate list is required since only HMVP candidates are allowed and the decoded merge/AMVP index could directly be used to obtain the BV candidates in the HMVP table.

In yet another example, or one small region contains one shared merge region and one non-shared merge region. In this case, two BV candidate lists are still required to be constructed in current VVC with dependency among the left and right 8×4 sub-regions. However, with the proposed method, only 1 candidate list is needed for the right 8×4 sub-region. And there is almost no dependency with the left 8×4 sub-region since the BV of left 4×4 block could be directly obtained from the HMVP table with decoded BV index.

Table 2: Complexity analysis of VTM design and proposed method (for a given region)

| Region size | Methods | Number of spatial neighbours to be accessed | Number of pruning | Number of BV list construction | Dependency on its neighbours |
| --- | --- | --- | --- | --- | --- |
| 8×8 in Figure 3(a) | VTM | 2×2 | 3×2 | 1×2 | Y |
|  | Proposed | 0 | 0 | 0 | N |
|  | Saving | 100% | 100% | 100% |  |
| 8×8 in Figure 3(b) | VTM | 2×2 | 3×2 | 1×2 | Y |
|  | Proposed | 2 | 3 | 1 | N |
|  | Saving | 50% | 50% | 50% |  |

Figure 3. 8×8/16×4 region with certain partitions.

Simulation results

The proposed method has been implemented on top of VTM6.0 [2]. Simulations are performed following the common test conditions [3].

The results of the proposed method are shown in Table 3.

Table 3: Coding performance of proposed method

|  |  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class F | 0.01% | -0.01% |  | 0.03% |  | 99% | 96% |
| Class TGM | 0.06% | 0.04% |  | 0.04% |  | 100% | 101% |
|  |  | Random access Main10 |  |  |  |  |  |
|  |  |  | Over VTM-6.0 |  |  |  |  |
|  | Y | U |  |  | V | EncT | DecT |
| Class F | -0.01% | 0.05% |  |  | 0.07% | 99% | 99% |
| Class TGM | 0.02% | -0.03% |  |  | -0.06% | 100% | 99% |
|  |  | Low delay B Main10 |  |  |  |  |  |
|  |  |  | Over VTM-6.0 |  |  |  |  |
|  | Y | U |  |  | V | EncT | DecT |
| Class F | -0.03% | 0.02% |  |  | 0.64% | 100% | 101% |
| Class TGM | -0.11% | -0.12% |  |  | -0.03% | 98% | 94% |

Conclusions

It is proposed to remove the shared merge list for IBC. It is recommended adopting the proposed method to the next version of VVC.

Proposed changes to working draft

The suggested specification changes on top of JVET-O2001_vE is described as follows: deleted text is marked with strikethrough, and newly added text are highlighted in green.

7.4.9.2 Coding tree unit semantics

The CTU is the root node of the coding tree structure.

The array IsAvailable[ cIdx ][ x ][ y ] specifying whether the sample at ( x, y ) is available for use in the derivation process for neighbouring block availability as specified in clause 6.4.4 is initialized as follows for cIdx = 0..2, x = 0..CtbSizeY − 1, and y = 0..CtbSizeY − 1:

IsAvailable[ cIdx ][ x ][ y ] = FALSE		(7-123)

The array IsInSmr[ x ][ y ] specifying whether the sample at ( x, y ) is located inside a shared merging candidate list region, is initialized as follows for x = 0..CtbSizeY − 1 and y = 0..CtbSizeY − 1:

IsInSmr[ x ][ y ] = FALSE		(7-124)

7.4.9.4 Coding tree semantics

When all of the following conditions are true, IsInSmr[ x ][ y ] is set equal to TRUE for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

- IsInSmr[ x0 ][ y0 ] is equal to FALSE

- cbWidth * cbHeight / 4 is less than 32

- treeType is not equal to DUAL_TREE_CHROMA

When IsInSmr[ x0 ][ y0 ] is equal to TRUE. the arrays SmrX[ x ][ y ], SmrY[ x ][ y ], SmrW[ x ][ y ] and SmrH[ x ][ y ] are derived as follows for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

SmrX[ x ][ y ] = x0		(7-126)

SmrY[ x ][ y ] = y0		(7-127)

SmrW[ x ][ y ] = cbWidth		(7-128)

SmrH[ x ][ y ] = cbHeight		(7-129)

…

When all of the following conditions are true, IsInSmr[ x ][ y ] is set equal to TRUE for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

- IsInSmr[ x0 ][ y0 ] is equal to FALSE

- One of the following conditions is true:

- mtt_split_cu_binary_flag is equal to 1 and  cbWidth * cbHeight / 2 is less than 32

- mtt_split_cu_binary_flag is equal to 0 and  cbWidth * cbHeight / 4 is less than 32

- treeType is not equal to DUAL_TREE_CHROMA

When IsInSmr[ x0 ][ y0 ] is equal to TRUE. the arrays SmrX[ x ][ y ], SmrY[ x ][ y ], SmrW[ x ][ y ] and SmrH[ x ][ y ] are derived as follows for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

SmrX[ x ][ y ] = x0		(7-130)

SmrY[ x ][ y ] = y0		(7-131)

SmrW[ x ][ y ] = cbWidth		(7-132)

SmrH[ x ][ y ] = cbHeight		(7-133)

8.6.2.1 General

…

When CbWidth * CbHeight is larger than 16, IsInSmr[ xCb ][ yCb ] is equal to false, the updating process for the history-based block vector predictor list as specified in clause 8.6.2.6 is invoked with luma block vector bvL.

It is a requirement of bitstream conformance that the luma block vector bvL shall obey the following constraints:

- CtbSizeY is greater than or equal to ( ( yCb + ( bvL[ 1 ] >> 4 ) ) & ( CtbSizeY − 1 ) ) + cbHeight.

- IbcVirBuf[ 0 ][ ( x + (bvL[ 0 ] >> 4 ) ) & ( IbcVirBufWidth − 1 ) ][ ( y + (bvL[1] >> 4 ) ) & ( CtbSizeY − 1) ] shall not be equal to −1 for x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1.

8.6.2.2 Derivation process for IBC luma block vector prediction

This process is only invoked when CuPredMode[ 0 ][ xCb ][ yCb ] is equal to MODE_IBC, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma block vector in 1/16 fractional-sample accuracy bvL.

The variables xSmr, ySmr, smrWidth, and smrHeight are derived as follows:

xSmr = IsInSmr[ xCb ][ yCb ]  ?  SmrX[ xCb ][ yCb ]  :  xCb	(8-906)

ySmr = IsInSmr[ xCb ][ yCb ]  ?  SmrY[ xCb ][ yCb ]  :  yCb	(8-907)

smrWidth = IsInSmr[ xCb ][ yCb ]  ?  SmrW[ xCb ][ yCb ]  :  cbWidth	(8-908)

smrHeight = IsInSmr[ xCb ][ yCb ]  ?  SmrH[ xCb ][ yCb ]  :  cbHeight	(8-909)

The varaible IslgrBlk is set to (cbWidth * cbHeight is greater than 16 ? true : false).

The luma block vector bvL is derived by the following ordered steps:

- When IslgrBlk is true, tThe derivation process for spatial block vector candidates from neighbouring coding units as specified in clause 8.6.2.3 is invoked with the luma coding block location ( xCb, yCb ) set equal to ( xCbSmr, yCbSmr ), the luma coding block width cbWidth, and the luma coding block height cbHeight set equal to smrCbWidth and smrCbHeight as inputs, and the outputs being the availability flags availableFlagA1, availableFlagB1 and the block vectors bvA1 and bvB1.

- When IslgrBlk is true, tThe block vector candidate list, bvCandList, is constructed as follows:

i = 0
if( availableFlagA1 )
	bvCandList [ i++ ] = bvA1		(8-910)
if( availableFlagB1 )
	bvCandList [ i++ ] = bvB1

- When IslgrBlk is true, tThe variable numCurrCand is set equal to the number of merging candidates in the bvCandList.

- When numCurrCand is less than MaxNumIbcMergeCand and NumHmvpIbcCand is greater than 0, the derivation process of IBC history-based block vector candidates as specified in 8.6.2.4 is invoked with bvCandList, and IsLgrBlk, and numCurrCand as inputs, and modified bvCandList and numCurrCand as outputs.

- When numCurrCand is less than MaxNumIbcMergeCand, the following applies until numCurrCand is equal to MaxNumIbcMergeCand:

    - bvCandList[ numCurrCand ][ 0 ] is set equal to 0.

    - bvCandList[ numCurrCand ][ 1 ] is set equal to 0.

    - numCurrCand is increased by 1.

- The variable bvIdx is derived as follows:

bvIdx = general_merge_flag[ xCb ][ yCb ] ? merge_idx[ xCb ][ yCb ] : mvp_l0_flag[ xCb ][ yCb ]	(8-911)

- The following assignments are made:

bvL[ 0 ] = bvCandList[ mvIdx ][ 0 ]		(8-912)

bvL[ 1 ] = bvCandList[ mvIdx ][ 1 ]		(8-913)

8.6.2.3 Derivation process for IBC spatial block vector candidates

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are as follows:

- the availability flags availableFlagA1 and availableFlagB1 of the neighbouring coding units,

- the block vectors in 1/16 fractional-sample accuracy bvA1, and bvB1 of the neighbouring coding units,

For the derivation of availableFlagA1 and mvA1 the following applies:

- The luma location ( xNbA1, yNbA1 ) inside the neighbouring luma coding block is set equal to ( xCb − 1,  yCb + cbHeight − 1 ).

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the neighbouring luma location ( xNbA1, yNbA1 ), checkPredModeY set equal to TRUE, and cIdx set equal to 0 as inputs, and the output is assigned to the block availability flag availableA1.

- The variables availableFlagA1 and bvA1 are derived as follows:

- If availableA1 is equal to FALSE, availableFlagA1 is set equal to 0 and both components of  bvA1 are set equal to 0.

- Otherwise, availableFlagA1 is set equal to 1 and the following assignments are made:

bvA1 = MvL0[ xNbA1 ][ yNbA1 ]		(8-914)

For the derivation of availableFlagB1 and bvB1 the following applies:

- The luma location ( xNbB1, yNbB1 ) inside the neighbouring luma coding block is set equal to ( xCb + cbWidth − 1, yCb − 1 ).

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the neighbouring luma location ( xNbB1, yNbB1 ), checkPredModeY set equal to TRUE, and cIdx set equal to 0 as inputs, and the output is assigned to the block availability flag availableB1.

- The variables availableFlagB1 and bvB1 are derived as follows:

- If one or more of the following conditions are true, availableFlagB1 is set equal to 0 and both components of  bvB1 are set equal to 0:

  - availableB1 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB1, yNbB1 ) have the same block vectors.

- Otherwise, availableFlagB1 is set equal to 1 and the following assignments are made:

bvB1 = MvL0[ xNbB1 ][ yNbB1 ]		(8-915)

8.6.2.4 Derivation process for IBC history-based block vector candidates

Inputs to this process are:

- a block vector candidate list bvCandList,

- a variable isInSmr specifying whether the current coding unit is inside a shared merging candidate regiona variable to indicate non-small block IsLgrBlk,

- the number of available block vector candidates in the list numCurrCand.

Outputs to this process are:

- the modified block vector candidate list bvCandList,

- the modified number of motion vector candidates in the list numCurrCand.

The variables isPrunedA1 and isPrunedB1 are set both equal to FALSE.

For each candidate in smrHmvpIbcCandList[ hMvpIdx ] with index hMvpIdx = 1..smrNumHmvpIbcCand, the following ordered steps are repeated until numCurrCand is equal to MaxNumIbcMergeCand:

- The variable sameMotion is derived as follows:

  - If IsLgrBlk is true and all of the following conditions are true for any block vector candidate N with N being A1 or B1, sameMotion and isPrunedN are both set equal to TRUE:

  - hMvpIdx is less than or equal to 1.

  - The candidate HmvpIbcCandList[NumHmvpIbcCand − hMvpIdx] is equal to the block vector candidate N.

  - isPrunedN is equal to FALSE.

  - Otherwise, sameMotion is set equal to FALSE.

- When sameMotion is equal to FALSE, the candidate HmvpIbcCandList[NumHmvpIbcCand − hMvpIdx] is added to the block vector candidate list as follows:

bvCandList[ numCurrCand++ ] = HmvpIbcCandList[ NumHmvpIbcCand − hMvpIdx ]	(8-916)

Reference

- C.-C. Chen, Y.-C. Lin, M.-S. Chiang, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE4.3.1: Shared merging candidate list,” JVET-M0170, Marrakech, Morocco, Jan. 2019.

- VTM-6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- JVET-N1010, “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).