| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0416 |
| --- | --- |

| Title: | Non-CE4: Using integer MV in CIIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Chen, Yuwen He 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4855 Wei.Chen@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Proposed specification text change for test 1

#### General

- the variable biFlag is derived as follows

  - If predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1, biFlag is set equal to TRUE;

  - Otherwise, biFlag is set equal to FALSE.

- For X being each of 0 and 1, when predFlagLX[ xSbIdx ][ ySbIdx ] is equal to 1, the following applies:

  - The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in clause 8.5.6.2 with X and refIdxLX as inputs.

  - The motion vector offset mvOffset is set equal to refMvLX[ xSbIdx ][ xSbIdx ] − mvLX[ xSbIdx ][ ySbIdx ].

  - If cIdx is equal to 0, the following applies:

    - The array predSamplesLXL  is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xSb, ySb ), the coding subblock width sbWidth, the coding subblock height sbHeight in luma samples, the luma motion vector offset mvOffset, the refined luma motion vector refMvLX[ xSbIdx ][ xSbIdx ], the reference array refPicLXL, bdofFlag, hpelIfIdx, biFlag, and cIdx as inputs.

    - When cbProfFlagLX is equal to 1, the prediction refinement with optical flow process specified in clause 8.5.6.4 is invoked with sbWidth, sbHeight, predSamplesLXL and the motion vector difference array diffMvLX[ xIdx ][ yIdx ] as inputs and the refined array predSamplesLXL as outputs.

  - Otherwise, if cIdx is equal to 1, the following applies:

    - The array predSamplesLXCb is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xSb, ySb ), the coding subblock width sbWidth / SubWidthC, the coding subblock height sbHeight / SubHeightC, the chroma motion vector offset mvOffset, the refined chroma motion vector refMvLX[ xSbIdx ][ xSbIdx ], the reference array refPicLXCb, bdofFlag, hpelIfIdx, biFlag, and cIdx as inputs.

  - Otherwise (cIdx is equal to 2), the following applies:

    - The array predSamplesLXCr is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xSb, ySb ), the coding subblock width sbWidth / SubWidthC, the coding subblock height sbHeight / SubHeightC, the chroma motion vector offset mvOffset, the refined chroma motion vector refMvLX[ xSbIdx ][ xSbIdx ], the reference array refPicLXCr, bdofFlag, hpelIfIdx, biFlag, and cIdx as inputs.

#### Fractional sample interpolation process

##### General

Inputs to this process are:

- a luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture,

- a variable sbWidth specifying the width of the current coding subblock,

- a variable sbHeight specifying the height of the current coding subblock,

- a motion vector offset mvOffset,

- a refined motion vector refMvLX,

- the selected reference picture sample array refPicLX,

- the half sample interpolation filter index hpelIfIdx,

- the bi-directional optical flow flag bdofFlag,

- the bi-prediction falg biFlag,

- a variable cIdx specifying the colour component index of the current block.

Outputs of this process are:

- an (sbWidth + brdExtSize)x(sbHeight + brdExtSize) array predSamplesLX of prediction sample values.

- If cIdx is equal to 0, the following applies:

  - Otherwise, the  prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma sample 8-tap interpolation filtering process as specified in clause 8.5.6.3.2 with ( xIntL − ( brdExtSize > 0 ? 1 : 0 ), yIntL − ( brdExtSize > 0 ? 1 : 0 ) ), ( xFracL, yFracL ), ( xSbIntL, ySbIntL ), refPicLX, hpelIfIdx, sbWidth, sbHeight, biFlag and ( xSb, ySb ) as inputs.

##### Luma sample interpolation filtering process

Inputs to this process are:

- a luma location in full-sample units ( xIntL, yIntL ),

- a luma location in fractional-sample units ( xFracL, yFracL ),

- a luma location in full-sample units ( xSbIntL, ySbIntL ) specifying the top-left sample of the bounding block for reference sample padding relative to the top-left luma sample of the reference picture,

- the luma reference sample array refPicLXL,

- the half sample interpolation filter index hpelIfIdx,

- a variable sbWidth specifying the width of the current subblock,

- a variable sbHeight specifying the height of the current subblock,

- the bi-prediction falg biFlag,

- a luma location ( xSb, ySb ) specifying the top-left sample of the current subblock relative to the top-left luma sample of the current picture,

Output of this process is a predicted luma sample value predSampleLXL

When ciip_flag[ xSb][ ySb ] is equal to 1 and biFlag is equal to TRUE, the following applies:

xIntL = xIntL + ( xFracL >> 3) 		(8-xxx)

yIntL = yIntL + ( yFracL >> 3 ) 		(8-xxx)

xFracL = 0		(8-xxx)

yFracL = 0		(8-xxx)

#### General

The predSamplesL, predSamplesCb and predSamplesCr are derived by the following ordered steps:

- For N being each of A and B, the following applies:

- The reference picture consisting of an ordered two-dimensional array refPicLNL of luma samples and two ordered two-dimensional arrays refPicLNCb and refPicLNCr of chroma samples is derived by invoking the process specified in clause 8.5.6.2 with X set equal to predListFlagN and refIdxX set equal to refIdxN as input.

- The array predSamplesLNL is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the luma coding block width sbWidth set equal to cbWidth, the luma coding block height sbHeight set equal to cbHeight, the motion vector offset mvOffset set equal to ( 0, 0 ), the motion vector mvLX set equal to mvN and the reference array refPicLXL set equal to refPicLNL, the variable bdofFlag set euqal to FALSE, the variable biFlag set euqal to FALSE, and the variable cIdx is set equal to 0 as inputs.

- The array predSamplesLNCb is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the coding block width sbWidth set equal to cbWidth / SubWidthC, the coding block height sbHeight set equal to cbHeight / SubHeightC, the motion vector offset mvOffset set equal to ( 0, 0 ), the motion vector mvLX set equal to mvCN, and the reference array refPicLXCb set equal to refPicLNCb, the variable bdofFlag set euqal to FALSE, the variable biFlag set euqal to FALSE, and the variable cIdx is set equal to 1 as inputs.

- The array predSamplesLNCr is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the coding block width sbWidth set equal to cbWidth / SubWidthC, the coding block height sbHeight set equal to cbHeight / SubHeightC, the motion vector offset mvOffset set equal to ( 0, 0 ), the motion vector mvLX set equal to mvCN, and the reference array refPicLXCr set equal to refPicLNCr, the variable bdofFlag set euqal to FALSE, the variable biFlag set euqal to FALSE, and the variable cIdx is set equal to 2 as inputs.