| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0437_r1 |
| --- | --- |

| Title: | CE7-related: Modifications to transform skip coefficient sign flag coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung, Xiang Li, Xin Zhao, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 {cauyeung, xlxiangli, xinzzhao shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In VVC Draft 6, six contexts are used to encode coefficient sign flag in transform skip mode. This contribution proposes to use 10 contexts to improve coding efficiency.

Compared to VTM-6.0 at standard QPs with QP in [22, 27, 32, 37], experimental results show -0.02% AI, -0.01% RA, -0.02% LB luma BD-rate for Class F, and -0.03% AI, -0.01% RA, -0.04% LB luma BD-rate for Class TGM.

Compared to VTM-6.0 at low QPs with QP in [2, 7, 12, 17], experimental results show -0.05% AI, -0.11% RA, -0.03% LB luma BD-rate for Class F, and -0.06% AI, -0.03% RA, -0.12% LB luma BD-rate for Class TGM.

Compared to VTM-6.0 at standard QPs with QP in [22, 27, 32, 37] with the proposed at 1.75 bins/coefficient, experimental results show 0.10% AI, 0.10% RA, 0.11% LB luma BD-rate for Class F, and 0.24% AI, 0.19% RA, 0.08% LB luma BD-rate for Class TGM. In contrast to VTM-6.0 with 1.75 bins/coefficient, experimental results show 0.11% AI, 0.07% RA, 0.05% LB luma BD-rate for Class F, and 0.26% AI, 0.20% RA, -0.02% LB luma BD-rate for Class TGM.

Compared to VTM-6.0 at low QPs with QP in [2, 7, 12, 17] with the proposed at 1.75 bins/coefficient, experimental results show 0.28% AI, 0.27% RA, 0.09% LB luma BD-rate for Class F, and 0.47% AI, 0.45% RA, 0.46% LB luma BD-rate for Class TGM. In contrast to VTM-6.0 with 1.75 bins/coefficient, experimental results show 0.30% AI, 0.38% RA, 0.03% LB luma BD-rate for Class F, and 0.46% AI, 0.49% RA, 0.41% LB luma BD-rate for Class TGM.

This revision is updated to report all low QP test results.

# Introduction

This contribution proposes a method to improve the coding efficiency of coefficient sign flag in transform skip mode. This is a part of three contributions intended to improve the coding efficiency in transform skip mode at 1.75 bins per coefficients at standard QPs and low QPs.

In VVC Draft 6 [1], the variable ctxInc for selecing the ctxInc of coeff_sign_flag[ n ] can be derived from the following variables:

leftSign = ( xC = = 0 ) ? 0 : CoeffSignLevel[ xC − 1 ][ yC ]

aboveSign = ( yC = = 0 ) ? 0 : CoeffSignLevel[ xC ][ yC − 1 ]

The ctxInc is derived as follows:

- If  leftSign is equal to 0 and aboveSign is equal to 0, or if leftSign is equal to  −aboveSign, the following applies:

ctxIncOffset = 0

- Otherwise, if leftSign is greater than or equal to 0 and aboveSign is greater than or equal to 0, the following applies:

ctxIncOffset = 1

- Otherwise, the following applies:

ctxIncOffset = 2

Then the ctxInc is derived as the following:

ctxInc = ctxIncOffset + ( intra_bdpcm_flag  = =  0  ?  0  :  3 )

The value of the context increment offset, ctxIncOffset, can also be determined using Table 1. As shown in Table 1, VVC Draft 6 [1] divides all possible outcomes of the pair (leftSign, aboveSign) into three classes. Each class is assigned with a unique context increment offset number between 0 to 2.

Table 1: A table to select ctxIncOffset for coeff_sign_flag based on (leftSigFlag, aboveSigFlag) in VVC Draft 6 [1].

| (leftSign, aboveSign) | ctxIncOffset |
| --- | --- |
| (-1,-1), (-1,0), (0,-1) | 2 |
| (-1,1), (0,0), (1,-1) | 0 |
| (1,1), (1,0), (0,1) | 1 |

# Proposed transform skip coefficient sign flag coding

In this contribution, the value of the offset context increment, OffsetCtxInc, can be determined by Table 2, where

leftSign = ( xC = = 0 ) ? 0 : CoeffSignLevel[ xC − 1 ][ yC ]

aboveSign = ( yC = = 0 ) ? 0 : CoeffSignLevel[ xC ][ yC − 1 ]

diagSign = ( xC = = 0  ||  yC = = 0 ) ? 0 : CoeffSignLevel[ xC − 1 ][ yC – 1 ]

In this disclosure, the context index of coeff_sign_flag [ xC ][ yC ] is based on the diagSign, leftSign and aboveSign as defined in the following.

As shown in Table 2, this contribution divides all possible outcomes of the tuple

(leftSign, aboveSign, diagSign)

into five classes. Each class is assigned with a unique offsetCtxInc number between 0 to 4. Then the ctxInc is derived as the following:

ctxInc = ctxIncOffset + ( intra_bdpcm_flag  = =  0  ?  0  :  3 )

Table 2: A table to select context increment offset, ctxIncOffset, based on (leftSign, aboveSign) and diagSign.

|  | diagSign |  |  |
| --- | --- | --- | --- |
| (leftSign, aboveSign), | -1 | 0 | 1 |
| (-1,-1), (-1,0), (0, -1) | 2 | 2 | 4 |
| (-1, 1), (1, -1) | 3 | 0 | 4 |
| (0,0) | 4 | 0 | 3 |
| (1,1), (1,0), (0, 1) | 3 | 1 | 1 |

In this contribution, the number of context models for coding coeff_sign_flag[ n ] with transform skip mode is increased from 6 to 10.

# Results

The testing condition of the proposed follows the CE7 [2] testing conditions using standard and low QP configurations as defined in CE7 [2]. In particular, the anchor results are encoded at 2 bins per coefficient in transform skip mode. The proposed is tested at both 2 and 1.75 bins per coefficient in transform skip mode.

As reference, the results of VTM-6.0 [3] at 2 bins per coefficient is compared with VTM-6.0 [3] at 1.75 bins per coefficient in Table 3 for standard QPs and Table 4 for low QPs.  to show the coding losses at 1.75 bins per coefficient.

A summary of all results for Class F and Class TGM is in the following Table:

Table 3: A comparison of VTM-6.0 at 2 and 1.75 bins per coefficient with standard QP values.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.02% | 99% | 99% |
| Class A2 | 0.00% | -0.02% | 0.03% | 100% | 101% |
| Class B | 0.00% | 0.02% | 0.00% | 100% | 99% |
| Class C | 0.02% | -0.03% | 0.04% | 100% | 98% |
| Class E | 0.00% | 0.10% | -0.01% | 99% | 95% |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 98% |
| Class D | 0.01% | 0.00% | -0.01% | 99% | 96% |
| Class F | 0.11% | 0.10% | 0.15% | 99% | 100% |
| Class TGM | 0.26% | 0.22% | 0.23% | 97% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.03% | 99% | 99% |
| Class A2 | 0.02% | 0.14% | 0.09% | 99% | 99% |
| Class B | 0.01% | 0.10% | 0.01% | 100% | 100% |
| Class C | 0.00% | 0.06% | 0.03% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.08% | 0.03% | 99% | 99% |
| Class D | -0.03% | -0.48% | 0.02% | 100% | 99% |
| Class F | 0.07% | 0.10% | 0.17% | 99% | 99% |
| Class TGM | 0.20% | 0.14% | 0.13% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.11% | 0.04% | 99% | 99% |
| Class C | -0.03% | -0.04% | -0.20% | 100% | 99% |
| Class E | -0.09% | 0.46% | 0.57% | 100% | 99% |
| Overall | -0.03% | 0.15% | 0.10% | 99% | 99% |
| Class D | 0.03% | 0.31% | 0.80% | 100% | 98% |
| Class F | 0.05% | 0.29% | 0.62% | 99% | 99% |
| Class TGM | -0.02% | -0.03% | -0.03% | 100% | 102% |

Table 4: A comparison of VTM-6.0 at 2 and 1.75 bins per coefficient with low QP values.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.04% | 0.05% | 99% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.01% | -0.01% | 0.00% | 100% | 100% |
| Class C | 0.01% | 0.00% | 0.01% | 100% | 98% |
| Class E | 0.05% | 0.00% | 0.02% | 99% | 98% |
| Overall | 0.02% | 0.01% | 0.01% | 100% | 99% |
| Class D | 0.00% | -0.01% | 0.01% | 99% | 98% |
| Class F | 0.30% | 0.14% | 0.14% | 98% | 99% |
| Class TGM | 0.46% | 0.42% | 0.41% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.00% | 0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.01% | 99% | 100% |
| Class B | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | -0.01% | 0.01% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | -0.03% | 0.04% | 99% | 98% |
| Class F | 0.38% | 0.00% | 0.12% | 99% | 99% |
| Class TGM | 0.49% | 0.40% | 0.40% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.01% | 99% | 99% |
| Class C | 0.00% | 0.00% | -0.02% | 100% | 99% |
| Class E | 0.03% | -0.02% | 0.02% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class D | -0.02% | 0.03% | -0.02% | 99% | 98% |
| Class F | 0.03% | 0.09% | 0.05% | 99% | 99% |
| Class TGM | 0.41% | 0.32% | 0.42% | 99% | 96% |

## Standard QP results at 2 bins per coefficient

In this result, the anchor and the test are encoded at 2 bins per coefficient in transform skip mode.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.01% | 101% | 101% |
| Class A2 | 0.00% | 0.00% | 0.02% | 100% | 101% |
| Class B | 0.00% | 0.04% | -0.02% | 100% | 97% |
| Class C | 0.01% | -0.09% | 0.06% | 100% | 94% |
| Class E | 0.01% | 0.06% | -0.04% | 100% | 96% |
| Overall | 0.00% | 0.00% | 0.01% | 100% | 97% |
| Class D | 0.00% | 0.03% | -0.08% | 99% | 98% |
| Class F | -0.02% | -0.03% | -0.03% | 99% | 99% |
| Class TGM | -0.03% | -0.01% | -0.02% | 98% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.00% | 99% | 99% |
| Class A2 | 0.01% | 0.11% | 0.07% | 99% | 99% |
| Class B | -0.03% | 0.06% | 0.02% | 99% | 98% |
| Class C | 0.01% | 0.10% | 0.03% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.07% | 0.03% | 99% | 99% |
| Class D | -0.01% | -0.28% | 0.04% | 100% | 100% |
| Class F | -0.01% | 0.02% | -0.08% | 99% | 98% |
| Class TGM | -0.01% | -0.06% | -0.09% | 99% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.17% | -0.14% | 99% | 100% |
| Class C | -0.03% | -0.05% | -0.10% | 99% | 94% |
| Class E | -0.13% | 0.52% | 0.41% | 99% | 96% |
| Overall | -0.03% | 0.18% | 0.01% | 99% | 97% |
| Class D | 0.00% | -0.14% | 0.12% | 100% | 97% |
| Class F | -0.02% | 0.24% | 0.52% | 99% | 98% |
| Class TGM | -0.04% | -0.08% | -0.01% | 100% | 103% |

## Low QP results at 2 bins per coefficient

In this result, the anchor and the test are encoded at 2 bins per coefficient in transform skip mode.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.01% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Class C | 0.00% | -0.01% | 0.00% | 100% | 101% |
| Class E | 0.01% | -0.02% | 0.01% | 100% | 98% |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Class D | -0.01% | 0.02% | -0.01% | 100% | 101% |
| Class F | -0.05% | -0.03% | -0.03% | 99% | 99% |
| Class TGM | -0.06% | -0.04% | -0.05% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.01% | 0.00% | 0.01% | 100% | 101% |
| Class C | -0.01% | -0.01% | -0.02% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Class D | -0.03% | -0.03% | -0.01% | 99% | 97% |
| Class F | -0.11% | -0.18% | -0.15% | 99% | 99% |
| Class TGM | -0.03% | 0.00% | 0.01% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.01% | -0.02% | 99% | 99% |
| Class C | -0.01% | 0.01% | 0.03% | 100% | 97% |
| Class E | 0.00% | -0.03% | 0.02% | 100% | 100% |
| Overall | -0.01% | 0.00% | 0.01% | 100% | 99% |
| Class D | -0.03% | 0.02% | -0.09% | 100% | 98% |
| Class F | -0.03% | -0.11% | -0.07% | 100% | 98% |
| Class TGM | -0.12% | 0.03% | 0.06% | 99% | 98% |

## Standard QP results at 1.75 bins per coefficient

In this result, the anchor is encoded at 2 bins per coefficient and the test is encoded at 1.75 bins per coefficient in transform skip mode.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.03% | 100% | 101% |
| Class A2 | 0.01% | 0.00% | -0.01% | 101% | 102% |
| Class B | 0.00% | 0.02% | -0.05% | 100% | 98% |
| Class C | 0.00% | -0.02% | 0.07% | 100% | 98% |
| Class E | 0.00% | 0.08% | -0.05% | 99% | 96% |
| Overall | 0.00% | 0.02% | 0.00% | 100% | 99% |
| Class D | 0.02% | 0.02% | -0.06% | 100% | 97% |
| Class F | 0.10% | 0.10% | 0.03% | 98% | 98% |
| Class TGM | 0.24% | 0.20% | 0.19% | 97% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | -0.05% | 99% | 99% |
| Class A2 | 0.01% | 0.03% | 0.06% | 99% | 99% |
| Class B | 0.00% | 0.00% | -0.05% | 99% | 99% |
| Class C | -0.02% | -0.10% | 0.02% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.01% | 99% | 99% |
| Class D | -0.05% | -0.48% | 0.02% | 100% | 99% |
| Class F | 0.10% | 0.07% | 0.13% | 99% | 99% |
| Class TGM | 0.19% | 0.13% | 0.10% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.04% | -0.19% | 99% | 100% |
| Class C | -0.04% | -0.03% | -0.33% | 99% | 98% |
| Class E | -0.06% | 0.18% | 0.08% | 99% | 97% |
| Overall | -0.03% | 0.05% | -0.17% | 99% | 98% |
| Class D | 0.02% | 0.00% | -0.10% | 100% | 94% |
| Class F | 0.11% | -0.03% | 0.28% | 99% | 96% |
| Class TGM | 0.08% | 0.03% | 0.13% | 100% | 102% |

## Low QP results at 1.75 bins per coefficient

In this result, the anchor is encoded at 2 bins per coefficient and the test is encoded at 1.75 bins per coefficient in transform skip mode.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.03% | 0.05% | 99% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.01% | 0.00% | 0.02% | 100% | 99% |
| Class C | 0.01% | 0.00% | 0.02% | 100% | 98% |
| Class E | 0.06% | 0.02% | 0.03% | 100% | 98% |
| Overall | 0.02% | 0.01% | 0.02% | 100% | 99% |
| Class D | -0.01% | -0.02% | -0.01% | 100% | 95% |
| Class F | 0.28% | 0.15% | 0.15% | 99% | 98% |
| Class TGM | 0.47% | 0.36% | 0.36% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.01% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.01% | -0.01% | 0.00% | 100% | 101% |
| Class C | -0.01% | -0.02% | 0.00% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | 0.00% | 100% | 100% |
| Class D | 0.00% | -0.05% | -0.01% | 99% | 96% |
| Class F | 0.27% | 0.04% | 0.10% | 99% | 97% |
| Class TGM | 0.45% | 0.37% | 0.38% | 99% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.01% | -0.01% | 100% | 97% |
| Class E | 0.04% | -0.09% | 0.00% | 100% | 100% |
| Overall | 0.01% | -0.02% | 0.00% | 100% | 99% |
| Class D | -0.01% | 0.03% | -0.05% | 99% | 95% |
| Class F | 0.09% | 0.07% | 0.09% | 99% | 96% |
| Class TGM | 0.46% | 0.36% | 0.38% | 99% | 96% |

# Proposed Text

The proposed method is described in the form of text modifications relative to the specification text of VVC Draft 6 [1] shown below. The text changes are highlighted in yellow.

9.3.4.2.10	Derivation process of ctxInc for the syntax element coeff_sign_flag for transform skip mode

Inputs to this process are the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC )

Output of this process is the variable ctxInc.

The variables rightSign and belowSign are set as follows:

leftSign = ( xC = = 0 ) ? 0 : CoeffSignLevel[ xC − 1 ][ yC ]	(9-64)

aboveSign = ( yC = = 0 ) ? 0 : CoeffSignLevel[ xC ][ yC − 1 ]	(9-65)

diagSign = ( xC = = 0  ||  yC = = 0 ) ? 0 : CoeffSignLevel[ xC − 1 ][ yC – 1 ] 	(9-66)

The variable ctxInc is derived as follows:

- If leftSign is equal to 0 and aboveSign is equal to 0, or if leftSign is equal to  −aboveSign, the following applies:

ctxIncOffset = (diagSign == 0)  ?  0  :

((leftSign != 0)  ?  3 + (diagSign >= 0)  :  4 − (diagSign >= 0)) 	(9-67)

- Otherwise, if rightSign is greater than or equal to 0 and belowSign is greater than or equal to 0, the following applies:

ctxIncOffset = diagSign >= 0 ? 1 : 3		(9-68)

- Otherwise, the following applies:

ctxIncOffset = diagSign <= 0 ? 2 : 4		(9-69)

The context index increment, ctxInc, is derived as the following:

ctxInc = ctxIncOffset + ( intra_bdpcm_flag  = =  0  ?  0  :  5 )

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001-vE, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- H. Schwarz, M. Coban, C. Auyeung, “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding,” JVET-O2027-v3, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-6.0.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).