| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0439 |
| --- | --- |

| Title: | Non-CE4: Motion pruning with alternative half-pel interpolation filter flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, XiangLi, Xiaozhong Xu, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 guichunli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, method of motion pruning is proposed to consider alternative half-pel interpolation filter flag when comparing motion information. The proposed method was implemented based on VTM-6.0, and simulations were conducted with common test conditions. It is reported that the proposed method has BD-rate of 0.01% on RA, and -0.03% on LB.

# Introduction

Adaptive motion vector resolution (AMVR) with Half-pel precision was previously adopted in VVC. It also includes an alternative half-pel interpolation filter (HPEL IF) to be used when MV resolution is half-pel precision. In current VTM-6.0 software, there’s a useAltHpelIf flag included in motion information to indicate whether the alternative HPEL IF is used or not. But in motion information comparison during motion information pruning process, this useAltHpelIf flag is not considered.

# Proposed Method

Considering that the alternative HPEL IF flag affects the motion compensation behavior, when same MV is used but alternative HPEL IF flag is different for two merge candidates, those candidates lead to different motion compensation results. To resolve this issue, it is proposed to also compare the alternative HPEL IF flag when comparing motion information in motion pruning process.

Figure 1- Same MV with differnt Alt HPEL IF flag leads to different MC output

# Experimental Results

The proposed method is implemented based on VTM-6.0. Simulations were conducted following JVET common test conditions. VTM-6.0 is used as anchor.

Table 3.1 Simulation Results

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.04% | 0.02% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.02% | 0.03% | 0.03% | 100% | 100% |  |  |  |  |  |
| Class B | 0.01% | 0.07% | -0.06% | 100% | 99% | 0.00% | -0.05% | -0.12% | 100% | 101% |
| Class C | 0.00% | 0.03% | 0.06% | 99% | 98% | -0.04% | 0.02% | 0.13% | 100% | 98% |
| Class E |  |  |  |  |  | -0.05% | 0.47% | 0.41% | 100% | 100% |
| Overall | 0.01% | 0.03% | 0.01% | 100% | 99% | -0.03% | 0.10% | 0.10% | 100% | 100% |
| Class D | -0.02% | -0.07% | 0.03% | 99% | 98% | 0.01% | 0.28% | -0.07% | 100% | 99% |
| Class F | 0.00% | -0.02% | -0.01% | 99% | 99% | -0.02% | 0.24% | 0.51% | 99% | 99% |

From the simulation results, compared with VTM-6.0 anchor, it was observed that there’s BD-rate change of 0.01% on Random Access configuration, and -0.03% for Low Delay B configuration. There’s no impact on encoding time and decoding time.

# Conclusion

It was observed that the proposed method of considering the alternative half-pel interpolation filter flag for motion information comparison has BD-rate change of 0.01% on RA and -0.03% on LDB.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-N1010 Marrakech, MA, Mar 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-6.0

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).