| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0450-v2 |
| --- | --- |

| Title: | AHG17: On Sample Aspect Ratio for RPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Sühring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution deals with sample aspect ratio signalling for RPR. The proposal is to add a flag indicating that the SAR of the CVS is not constant and in addition two options for correct SAR derivation at a resolution changing point are proposed:

Option 1: Implicit derivation by scaling the initial SAR, or

Option 2: Explicit signalling through a SAR SEI message.

Revision 1 of this document updates Option 2 according to the BoG recommendations by removing the persistence signalling and defining the scope as in the following.

From JVET-P0968: “The persistency scope is the set of AUs starting from the current until (exclusive) the next AU associated with the same SEI message.”

# Introduction

Reference Picture Resampling (RPR) allows an encoder to change the coded picture size outside the boundaries of a coded video sequence, e.g. in live communication scenarios without RAPs or in streaming scenarios at RAPs with leading pictures. In such a case reference pictures are to be resampled to the correct resolution, i.e. the resolution of the current picture.

However, in some scenarios, it could be that the horizontal and vertical scaling factors at a resampling point are not equal, i.e. the aspect ratio of the coded/cropped picture changes as when switching from one resolution to another with varying picture aspect ratio: e.g. 4Kx2K to 1080p to 480p as common in popular streaming service bitrate ladders. For further examples, see ISO/IEC 23009-19 CMAF sections C.4 and C.5.

In these cases, the sample aspect ratio needs to change to maintain the aspect ratio of the presented video. However, the SAR signalling in VVC (and predecessors) assumes a constant SAR per CVS (as indicated in the VUI) so that when RPR decouples the resolution change from the CVS, the SAR signalling needs to be adjusted to allow changes of the SAR within the CVS.

Therefore, this document proposes to add a flag indicating that the indicated sample aspect ratio in the SPS VUI is constant over the CVS and to allow at resolution change either to:

- Option A: implicit derivation of the new SampleAspectRatio value

- Option B: explicit SEI signalling of the new SampleAspectRatio value.

# Proposal

Common VUI syntax:

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_constant_flag | u(1) |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = 255 ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| […] |  |
| } |  |

aspect_ratio_info_present_flag equal to 1 specifies that aspect_ratio_idc is present. aspect_ratio_info_present_flag equal to 0 specifies that aspect_ratio_idc is not present.

aspect_ratio_constant_flag equal to 1 specifies that the value of aspect_ratio_idc, SarWidth and SarHeight is constant in the CLVS. aspect_ratio_constant_flag equal to 0 specifies that the value of aspect_ratio_idc, SarWidth and SarHeight is not constant in the CLVS and that sample aspect ratio SEI messages may be present in the CLVS indicating new sample aspect ratio. aspect_ratio_constant_flag equal to 0 does not impose this requirement.

aspect_ratio_idc, when not equal to 255, indicates the nominal sample aspect ratio of the luma samples. Its semantics are as specified for SampleAspectRatio parameter in Rec. ITU-T H.273 | ISO/IEC 23091-2. When the aspect_ratio_idc syntax element is not present, the value of aspect_ratio_idc is inferred to be equal to 0. Values of aspect_ratio_idc that are specified as reserved for future use in Rec. ITU-T H.273 | ISO/IEC 23091-2 shall not be present in bitstreams conforming to this version of this Specification. Decoders shall interpret values of aspect_ratio_idc that are reserved for future use in Rec. ITU-T H.273 | ISO/IEC 23091-2 as equivalent to the value 0.

sar_width, when present, indicates the nominal horizontal size of the sample aspect ratio (in arbitrary units).

sar_height, when present, indicates the nominal vertical size of the sample aspect ratio (in the same arbitrary units as sar_width).

When present, sar_width and sar_height shall be relatively prime or equal to 0. When aspect_ratio_idc is equal to 0 or sar_width is equal to 0 or sar_height is equal to 0, the sample aspect ratio is unspecified in this Specification.

Proposal Option A: Implicit SAR derivation

The sample aspect ratio is derived as follows. When aspect_ratio_constant_flag is equal to 1, the SampleAspectRatio is derived from the nominal sample aspect ratio by setting equal to SarWidth:SarHeight.

Otherwise, aspect_ratio_constant_flag is equal to 0, the value of SampleAspectRatio is set equal to SarWidth*HorFactor:SarHeight*VerFactor, where HorFactor and VerFactor are derived as follow:

HorFactor = PicOutputWidthL/pic_width_max_in_luma_samples

VerFactor = PicOutputHeightL/pic_height_max_in_luma_samples

Proposal Option B: Explicit SAR signalling with SEI message

The sample aspect ratio is derived from the nominal sample aspect ratio as follows.

Otherwise, aspect_ratio_constant_flag is equal to 0, the value of SampleAspectRatio derived based on the sample aspect ratio indication SEI message.

Sample aspect ratio SEI message

The sample aspect ratio SEI message provides information about the sample aspect ratio of the samples of the decoded picures. The scope of the sample aspect ratio SEI message are the pictures of the CLVS from the access unit containing the SEI message, inclusive, up to but not including the first the next access unit, in decoding order, that contains a sample aspect ratio SEI message in decoding order.

| sample_aspect_ratio( payloadSize ) { | Descriptor |
| --- | --- |
| sei_aspect_ratio_idc | u(8) |
| if( sei_aspect_ratio_idc = = EXTENDED_SAR ) { |  |
| sei_sar_width | u(16) |
| sei_sar_height | u(16) |
| } |  |
| } |  |

sei_aspect_ratio_idc, when not equal to 255, indicates the sample aspect ratio of the luma samples. Its semantics are as specified for SampleAspectRatio parameter in Rec. ITU-T H.273 | ISO/IEC 23091-2. When the sei_aspect_ratio_idc syntax element is not present, the value of aspect_ratio_idc is inferred to be equal to 0. Values of sei_aspect_ratio_idc that are specified as reserved for future use in Rec. ITU-T H.273 | ISO/IEC 23091-2 shall not be present in bitstreams conforming to this version of this Specification. Decoders shall interpret values of sei_aspect_ratio_idc that are reserved for future use in Rec. ITU-T H.273 | ISO/IEC 23091-2 as equivalent to the value 0.

sei_sar_width, when present, indicates the horizontal size of the sample aspect ratio (in arbitrary units).

sei_sar_height, when present, indicates the vertical size of the sample aspect ratio (in the same arbitrary units as sei_sar_width).

When present, sei_sar_width and sei_sar_height shall be relatively prime or equal to 0. When sei_aspect_ratio_idc is equal to 0 or sei_sar_width is equal to 0 or sei_sar_height is equal to 0, the sample aspect ratio is unspecified in this Specification.

It is a requirement of bitstream conformance that all sample aspect ratio SEI messages in the CLVS have the same value of the syntax elements sei_aspect_ratio_idc, sei_sar_width and sei_sar_height when aspect_ratio_constant_flag in the SPS that is referred to by all slices in the access units of the CLVS is equal to 1.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).