| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0460 |
| --- | --- |

| Title: | Non-CE8: Minimum QP for Palette Escape Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao Seethal Paluri Seung Hwan Kim LG Electronics. | Tel: Email: | 1-201-816-2000 jie.zhao@lge.com seethal.paluri@lge.com seunghwan3.kim@lge.com |
| Source: |  |  |  |

_____________________________

# Abstract

In palette mode escape coding, there is no transform, quantized escape components are signalled directly. This is similar to transform skip mode. In transform skip mode, there is a limit to the minimum QP allowed. This document proposes to limit the minimum Qp used in escape mode coding to be consistent with Transform skip minimum Qp.

Results of Dual Tree Off, Low QP (2, 7, 12, 17), YUV respectively are:

- AI  0.04%, -0.36%, -0.38%; RA -0.33%, -0.31%, -0.32%; LD 0.03%, -0.18%, -0.18%

Results of Dual Tree On, Low QP (2, 7, 12, 17), YUV respectively are:

- AI  0.03%,	-0.46%,	-0.45%; RA -0.16%, -0.33%	, -0.34%; LD 0.01%, -0.17%, -0.18%

# Introduction

Limiting the minimum allowed QP for transform skip [1] is adopted in VVC WD6. This prevents cases in which the effective quantization step size for transform skip becomes smaller than 1.

The minimum Qp for transform skip, denoted as QpPrimeTsMin, is set as 6* (internalBitDepth – inputBitDepth)) at encoder side, and min_qp_prime_ts_ minus4 is signalled in SPS

min_qp_prime_ts_minus4 specifies the minimum allowed quantization parameter for transform skip mode as follows:

QpPrimeTsMin = 4 + min_qp_prime_ts_ minus4		(7-12)

When transform skip is used, the QP is derived as

qP = Max( QpPrimeTsMin, Qp′Y )

Currently in VTM6, transform skip is only applied to luma.

# Proposed

In palette mode escape coding, there is no transform, quantized escape components are signalled directly. This is similar to transform skip mode. Therefore it is asserted that to prevent the effective quantization step size for palette escape coding becomes smaller than 1, the same limitation of minimum QP for transform skip should be applied to palette escape coding. For chroma, the QpPrimeTsMinChroma can be set same as QpPrimeTsMin or signalled and derived in a similar way as QpPrimeTsMin, e.g. signal min_qp_prime_chroma_ts_minus4 in SPS, and derive QpPrimeChromaTsMin as follows.

QpPrimeChromaTsMin = 4 + min_qp_prime_chroma_ts_ minus4

The proposed changes of palette qP derivation are:

#### 8.4.5.3 Decoding process for palette mode

…

- Otherwise (bIsEscapeSample is equal to 1 and cu_transquant_bypass_flag is equal to 0), the following ordered steps apply:

- The quantization parameter qP is derived as follows:

- If cIdx is equal to 0,

qP = Max( 0 QpPrimeTsMin, Qp′Y )		(8-272)

- Otherwise, if cIdx is equal to 1,

qP = Max( 0 QpPrimeTsMinChroma, Qp′Cb )		(8-273)

- Otherwise (cIdx is equal to 2),

qP = Max( 0 QpPrimeTsMinChroma, Qp′Cr )		(8-274)

# Experimental Result

The proposed palette minimum QP limitation has no impact on CTC. Tests are done using CE8 base software which is VTM6 with some bug fixes, and tested at low QP 2, 7, 12, 17. It shows there is small gain at low Qp.

Table 3.1 Dual Tree Off Result

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 CE8 base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.08% | -0.94% | -0.98% | 100% | 100% |
| TGM 720p | 0.03% | -0.30% | -0.30% | 100% | 99% |
| Animation | 0.00% | -0.01% | -0.01% | 100% | 99% |
| Mixed content | 0.06% | -0.27% | -0.31% | 101% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.04% | -0.36% | -0.38% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  | Over VTM6.0 CE8 base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.57% | -0.55% | -0.54% | 99% | 99% |
| TGM 720p | -0.45% | -0.41% | -0.43% | 100% | 98% |
| Animation | 0.00% | -0.01% | -0.01% | 99% | 99% |
| Mixed content | -0.41% | -0.38% | -0.40% | 100% | 99% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | -0.33% | -0.31% | -0.32% | 100% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  | Over VTM6.0 CE8 base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.01% | -0.30% | -0.29% | 100% | 100% |
| TGM 720p | 0.11% | -0.19% | -0.17% | 100% | 99% |
| Animation | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Mixed content | 0.05% | -0.33% | -0.34% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.03% | -0.18% | -0.18% | 100% | 100% |

Table 3.2 Dual Tree On Result

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 CE8 base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.01% | -0.97% | -0.93% | 100% | 100% |
| TGM 720p | 0.05% | -0.25% | -0.24% | 100% | 99% |
| Animation | 0.04% | -0.06% | -0.09% | 100% | 100% |
| Mixed content | 0.05% | -0.75% | -0.75% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.03% | -0.46% | -0.45% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  | Over VTM6.0 CE8 base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.26% | -0.60% | -0.57% | 98% | 98% |
| TGM 720p | -0.22% | -0.41% | -0.49% | 99% | 98% |
| Animation | 0.01% | -0.05% | -0.05% | 100% | 99% |
| Mixed content | -0.24% | -0.35% | -0.34% | 100% | 98% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | -0.16% | -0.33% | -0.34% | 99% | 98% |
|  |  |  | Low delay B Main10 |  |  |
|  | Over VTM6.0 CE8 base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.06% | -0.23% | -0.24% | 99% | 100% |
| TGM 720p | 0.06% | -0.20% | -0.21% | 100% | 100% |
| Animation | 0.00% | -0.01% | -0.01% | 100% | 100% |
| Mixed content | 0.05% | -0.35% | -0.34% | 99% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.01% | -0.17% | -0.18% | 100% | 100% |

# Conclusion

This contribution proposes to limit the minimum QP allowed for escape coding to be consistent to Transform skip minimum Qp and prevent the effective quantization step size for palette escape coding becomes smaller than 1. It is proposed to adopt this Qp limitation to VVC working draft.

# Reference

- “Non-CE8: Minimum QP for Transform Skip Mode,” Document of Joint Video Experts Team (JVET), JVET-O0919, Gothenburg, SE, July 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-O0010, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).