| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0470_v2 |
| --- | --- |

| Title: | Non-CE5: Simplification of Cross Component Adaptive Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li, Xin Zhao, Liang Zhao, Yixin Du, Shan Liu | Tel: Email: | xlxiangli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Cross Component Adaptive Loop Filter (CC-ALF) was proposed in CE5 to enhance chroma components with luma reconstructed samples as input. In this contribution, simplification of CC-ALF is proposed by reducing the bit depth of multipliers so that CC-ALF filter processing can be achieved by multiplications of 4-bit and 8-bit values. Simulation results are reported as follows. Compared to CE5-2.1, minor BD rate difference is observed with 2%-3% decoding time saving.

- Over VTM-6, BD rate changes are 0.12%, -7.35%, -5.84% for AI, 0.18%, -10.42%, -9.83% for RA, 0.13%, -15.70%, -11.91% for LDB.

- Over CE5-2.1, BD rate changes are -0.01%, 0.26%, 0.20% for AI, 0.02%, 0.30%, 0.28% for RA, -0.04%, -0.08%, -0.19% for LDB.

# Introduction

In CE5-2.1 (earlier JVET-O0636), Cross Component Adaptive Loop Filter (CC-ALF) was proposed to enhance chroma components by adding offsets to chroma samples after ALF, where the offsets are derived from a high-pass filter with reconstructed luma samples as input. The filtering process is as follows

ΔIix,y=x0,y0∈SiI0xC+x0,yC+y0cix0,y0,
where ΔIix,y denotes chroma offset, I0xC+x0,yC+y0 indicates luma reconstruction sample, and cix0,y0 represents filter coefficient. For the filter, a diamond shape is used with eighteen filter coefficients, among which fourteen coefficients are different.

In CE5-2.1, the bit-depth of filter coefficients is 8-bit. When internal bit-depth is 10-bit, eighteen 8-bit by 10-bit multipliers are needed to filter each chroma sample. With the consideration that some filter coefficients are the same, the number of multipliers may be reduced to fourteen but the bit-depth of some multipliers is also increased.

# Proposed method

To simplify the filtering process of CC-ALF, the bit-depth of multiplier is reduced to 4-bit by 8-bit with following changes.

- CC-ALF coefficients are constrained to 4 bits, i.e., in the range of (-8, 7), inclusive.

- Before feeding into CC-ALF, luma reconstructed samples are first shifted to 8-bit dynamic range with rounding.

The proposed method not only reduces computational complexity in hardware implementation, but is also more friendly for software SIMD optimization. In fact, multiplications of signed 8-bit and unsigned 8-bit can performed at the same time with SIMD, which is much more efficient than 16-bit by 16-bit multiplications.

In addition, as CC-ALF filter coefficients are within 4 bits, 4-bit fixed length coding is used instead of 3rd order exp-Golomb binarization uek(v). In fact, for 4-bit values, 4-bit fixed length coding is not only simpler, but also more efficient than 3rd order exp-Golomb binarization.

# Experimental results

The proposed method has been implemented on top of CE5-2.1. The simulation results are summarized as follows, where the anchor on the left side is VTM-6, the anchor on the right side is CE5-2.1.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6 |  |  |  |  | Over CE5-2.1-noshift |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | -3.44% | -4.04% | 104% | 99% | 0.00% | 0.05% | 0.12% | 100% | 96% |
| Class A2 | 0.09% | -7.38% | -8.03% | 102% | 100% | -0.05% | 1.40% | 1.04% | 100% | 97% |
| Class B | 0.10% | -10.53% | -8.52% | 102% | 101% | 0.00% | 0.04% | 0.06% | 101% | 97% |
| Class C | 0.12% | -4.19% | -3.91% | 101% | 97% | 0.00% | -0.01% | 0.00% | 100% | 98% |
| Class E | 0.18% | -10.12% | -3.52% | 102% | 98% | -0.01% | 0.04% | -0.06% | 100% | 96% |
| Overall | 0.12% | -7.35% | -5.84% | 102% | 99% | -0.01% | 0.26% | 0.20% | 100% | 97% |
| Class D | 0.13% | -2.91% | -3.42% | 101% | 98% | 0.02% | -0.17% | -0.14% | 100% | 97% |
| Class F | 0.18% | -6.54% | -6.58% | 100% | 100% | 0.00% | 0.04% | 0.06% | 100% | 98% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6 |  |  |  |  | Over CE5-2.1-noshift |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.16% | -5.51% | -5.14% | 103% | 101% | 0.02% | 0.17% | 0.21% | 100% | 99% |
| Class A2 | 0.20% | -10.36% | -14.67% | 102% | 101% | -0.01% | 1.52% | 1.27% | 100% | 98% |
| Class B | 0.23% | -17.67% | -14.10% | 102% | 100% | 0.02% | -0.05% | -0.03% | 100% | 97% |
| Class C | 0.12% | -5.11% | -4.37% | 101% | 96% | 0.04% | -0.08% | -0.01% | 101% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.18% | -10.42% | -9.83% | 102% | 99% | 0.02% | 0.30% | 0.28% | 100% | 98% |
| Class D | 0.11% | -4.58% | -4.69% | 102% | 102% | 0.00% | -0.15% | -0.23% | 100% | 99% |
| Class F | 0.21% | -4.60% | -4.46% | 102% | 100% | 0.03% | 0.14% | 0.17% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6 |  |  |  |  | Over CE5-2.1-noshift |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.18% | -21.38% | -19.28% | 102% | 102% | 0.00% | 0.23% | -0.12% | 100% | 98% |
| Class C | 0.15% | -8.46% | -6.81% | 101% | 106% | 0.03% | -0.33% | -0.39% | 100% | 98% |
| Class E | 0.02% | -15.90% | -6.41% | 104% | 105% | -0.22% | -0.28% | -0.03% | 101% | 99% |
| Overall | 0.13% | -15.70% | -11.91% | 103% | 104% | -0.04% | -0.08% | -0.19% | 101% | 98% |
| Class D | 0.18% | -8.24% | -6.31% | 101% | 100% | 0.00% | -0.14% | -0.32% | 101% | 100% |
| Class F | 0.28% | -9.91% | -7.71% | 101% | 102% | 0.15% | 0.68% | -0.20% | 100% | 98% |

When the method of shifting chroma gain to luma in CE5-2.1 is enabled, the simulation results are as follows.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6 |  |  |  |  | Over CE5-2.1-withshift |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.19% | 5.45% | 3.88% | 103% | 100% | 0.00% | 0.08% | 0.14% | 101% | 100% |
| Class A2 | -2.09% | 1.27% | 0.35% | 101% | 98% | -0.05% | 1.61% | 1.18% | 101% | 98% |
| Class B | -0.74% | -1.70% | 0.93% | 100% | 99% | 0.00% | 0.06% | 0.02% | 100% | 98% |
| Class C | -0.76% | 3.97% | 4.52% | 101% | 97% | -0.01% | -0.01% | 0.01% | 100% | 97% |
| Class E | -0.47% | -2.78% | 4.68% | 101% | 100% | -0.01% | 0.04% | -0.04% | 100% | 99% |
| Overall | -1.00% | 1.07% | 2.75% | 101% | 99% | -0.01% | 0.30% | 0.22% | 100% | 98% |
| Class D | -0.73% | 5.18% | 4.73% | 101% | 99% | 0.02% | -0.19% | -0.12% | 100% | 100% |
| Class F | -0.73% | -0.98% | -1.24% | 100% | 100% | 0.00% | 0.07% | 0.05% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6 |  |  |  |  | Over CE5-2.1-withshift |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.82% | 5.59% | 4.30% | 101% | 100% | -0.01% | 0.30% | 0.15% | 100% | 99% |
| Class A2 | -2.07% | 0.27% | -4.88% | 101% | 100% | -0.03% | 1.50% | 1.49% | 100% | 98% |
| Class B | -0.53% | -6.38% | -2.33% | 101% | 100% | 0.01% | -0.03% | -0.09% | 101% | 98% |
| Class C | -0.75% | 5.54% | 6.45% | 101% | 98% | 0.03% | -0.21% | -0.12% | 100% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.16% | 0.52% | 0.83% | 101% | 100% | 0.00% | 0.29% | 0.26% | 100% | 98% |
| Class D | -0.62% | 5.69% | 6.22% | 102% | 103% | 0.02% | -0.39% | -0.18% | 101% | 99% |
| Class F | -0.94% | 1.96% | 2.43% | 101% | 101% | -0.02% | 0.13% | 0.08% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6 |  |  |  |  | Over CE5-2.1-withshift |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.52% | -9.21% | -6.61% | 101% | 101% | 0.02% | 0.09% | 0.03% | 101% | 99% |
| Class C | -0.71% | 3.62% | 5.54% | 101% | 104% | 0.02% | -0.25% | 0.08% | 100% | 99% |
| Class E | -0.13% | -5.59% | 7.24% | 103% | 103% | 0.11% | 0.01% | -0.33% | 101% | 100% |
| Overall | -0.49% | -4.03% | 0.90% | 102% | 102% | 0.04% | -0.04% | -0.05% | 101% | 99% |
| Class D | -0.51% | 4.64% | 7.43% | 101% | 100% | 0.06% | -0.26% | 0.10% | 101% | 101% |
| Class F | -0.85% | -2.81% | 0.01% | 101% | 103% | -0.01% | -0.07% | 0.25% | 100% | 99% |

It can be observed from the results that the proposed simplification has minor efficiency impact when compared to CE5-2.1. Moreover, less decoding time than CE5-2.1 is obtained due to SIMD implementation.

# Spec text changes

The proposed text changes are on top of CE5-2.1 are as follows. The changes are highlighted.

Changes to syntax table and semantics

| alf_data( ) { | Descriptor |
| --- | --- |
| … |  |
| if ( alf_cross_component_cb_filter_signal_flag ) |  |
| for ( j = 0; j < 14; j++ ) |  |
| alf_cross_component_cb_coeff_plus8[ j ] | u(4) |
| alf_cross_component_cb_coeff_abs[ j ] | uek(v) |
| if( alf_cross_component_cb_coeff_abs[ j ] ) |  |
| alf_cross_component_cb_coeff_sign[ j ] | u(1) |
| if ( alf_cross_component_cr_ filter_signal_flag ) |  |
| for ( j = 0; j < 14; j++ ) |  |
| alf_cross_component_cb_coeff_plus8[ j ] | u(4) |
| alf_cross_component_cr_coeff_abs[ j ] | uek(v) |
| if( alf_cross_component_cr_coeff_abs[ j ] ) |  |
| alf_cross_component_cr_coeff_sign[ j ] | u(1) |
| … |  |
| } |  |

alf_cross_component_cb_coeff_plus8[ j ] minus 8 specifies the value of the j-th cross component Cb filter coefficient. When alf_cross_component_cb_coeff_plus8[ j ] is not present, j-th cross component Cb filter coefficient inferred to be equal to 0.

alf_cross_component_cr_coeff_plus8[ j ] minus 8 specifies the value of the j-th cross component Cr filter coefficient. When alf_cross_component_cr_coeff_plus8[ j ] is not present, j-th cross component Cr filter coefficient inferred to be equal to 0.

Changes to filtering process

  - The variable shift1 and offset1 are derived as follows

shift1 =  BitDepthY – 8

offset1  =  1 << shift1>>1

shift2 =  7 – (BitDepthC – 8)

  - The variable sum is derived as follows:

sum = 	f[ 0 ] * ((recPictureL[ hx, vy + yM2 ] + offset1) >> shift1) +
	f[ 1 ] * ((recPictureL[ hx+xM1, vy + yM1 ] + offset1) >> shift1) +
	f[ 2 ] * ((recPictureL[ hx, vy + yM1 ] + offset1) >> shift1) +
	f[ 3 ] * ((recPictureL[ hx+xP1, vy + yM1 ] + offset1) >> shift1) +
	f[ 4 ] * ((recPictureL[ hx+xM2, vy ] + offset1) >> shift1) +
	f[ 5 ] * ((recPictureL[ hx+xM1, vy ] + offset1) >> shift1) +
	f[ 6 ] * ((recPictureL[ hx, vy ] + offset1) >> shift1) +
	f[ 7 ] * ((recPictureL[ hx+xP1, vy ] + offset1) >> shift1) +
	f[ 4 ] * ((recPictureL[ hx+xP2, vy ] + offset1) >> shift1) +	(8-1289)
	f[ 4 ] * ((recPictureL[ hx+xM2, vy+yP1 ] + offset1) >> shift1) + 
	f[ 8 ] * ((recPictureL[ hx+xM1, vy+yP1 ] + offset1) >> shift1) +
	f[ 9 ] * ((recPictureL[ hx, vy+yP1 ] + offset1) >> shift1) +
	f[ 10 ] * ((recPictureL[ hx+xP1, vy+yP1 ] + offset1) >> shift1) +
	f[ 4 ] * ((recPictureL[ hx+xP2, vy+yP1 ] + offset1) >> shift1) +
	f[ 11 ] * ((recPictureL[ hx+xM1, vy + yP2 ] + offset1) >> shift1) +
	f[ 12 ] * ((recPictureL[ hx, vy + yP2 ] + offset1) >> shift1) +
	f[ 13 ] * ((recPictureL[ hx+xP1, vy + yP2 ] + offset1) >> shift1) +
	f[ 0 ] * ((recPictureL[ hx, vy + yP3 ] + offset1) >> shift1)

sum = curr + ( sum + 64 (1<<shift2>>1)) >> 7 shift2 )	(8-1290)

# Conclusions

In this contribution, CC-ALF is simplified by reducing the bit-depth of multipliers used in CC-ALF filtering process from 8-bit by 10-bit to 4-bit by 8-bit when internal bit-depth is 10. Simulation results show that the proposed method shows almost the same coding efficiency to CE5-2.1 with less complexity in both hardware and software implemenation. It is proposed to adopt this simplification.

# References

- F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).