| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0471 |
| --- | --- |

| Title: | AHG12 Explicit Signalling of Uniform and Non-Uniform Subpicture Grid in the SPS. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seethal Paluri Jie Zhao Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email | seethal.paluri@lge.com jie.zhao@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics USA |  |  |

_____________________________

# Abstract

This contribution proposes to explicitly signal uniform and non-uniform subpicture grid information in the existing subpicture framework of the current working draft (WD 6) of the Versatile Video Coding (VVC). It is asserted that the explicit signalling using the uniform and non-uniform approach would be more aligned with the tiles structure and it would also facilitate ease of signalling larger subpicture grids. A flag is introduced in the Sequence Parameter Set (SPS), which indicates if the uniform signaling is used. If uniform signalling is used, then the dimensions of the subpicture grid and height are explicitly signalled. If non-uniform signalling is used the number of subpicture columns and rows are first signalled and then for each column and row, the subpicture grid width and height is explicitly signalled.

# Introduction

The subpicture design introduced in the Working Draft (WD) 6 [1] of the Versatile Video Coding (VVC) is meant to aid uses cases such as viewport-dependent 360-degree streaming. The subpicture design is currently signalled in the Sequence Parameter Sets (SPS). In the existing structure, the subpicture grid width and height are signalled. Following this a subpicture grid index is signalled for every grid position. The subpicture height and width information is used to compute the number of subpictures and for each subpicture, two flags are signalled namely, subpicture_treated_as_pic_flag and loop_filter_across_subpic_enabled_flag. Table 1 below shows the syntax and semantics associated with subpictures.

Table 1 Shows an excerpt of syntax from the SPS.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

Where the semantics is described as:

subpics_present_flag equal to 1 indicates that subpicture parameters are present in the present in the SPS RBSP syntax. subpics_present_flag equal to 0 indicates that subpicture parameters are not present in the present in the SPS RBSP syntax.

NOTE 2 – When a bitstream is the result of a sub-bitstream extraction process and contains only a subset of the subpictures of the input bitstream to the sub-bitstream extraction process, it might be required to set the value of subpics_present_flag equal to 1 in the RBSP of the SPSs.

max_subpics_minus1 plus 1 specifies the maximum number of subpictures that may be present in the CVS. max_subpics_minus1 shall be in the range of 0 to 254. The value of 255 is reserved for future use by ITU-T | ISO/IEC.

subpic_grid_col_width_minus1 plus 1 specifies the width of each element of the subpicture identifier grid in units of 4 samples. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / 4) ) bits.

The variable NumSubPicGridCols is derived as follows:

NumSubPicGridCols = ( pic_width_max_in_luma_samples + subpic_grid_col_width_minus1 * 4 + 3 ) /
		( subpic_grid_col_width_minus1 * 4 + 4 )

subpic_grid_row_height_minus1 plus 1 specifies the height of each element of the subpicture identifier grid in units of 4 samples. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / 4) ) bits.

The variable NumSubPicGridRows is derived as follows:

NumSubPicGridRows = ( pic_height_max_in_luma_samples + subpic_grid_row_height_minus1 * 4 + 3 ) /
		( subpic_grid_row_height_minus1 * 4 + 4 )

subpic_grid_idx[ i ][ j ] specifies the subpicture index of the grid position (i, j). The length of the syntax element is Ceil( Log2( max_subpics_minus1 + 1 ) ) bits.

The variables SubPicTop[ subpic_grid_idx[ i ][ j ] ], SubPicLeft[ subpic_grid_idx[ i ][ j ] ], SubPicWidth[ subpic_grid_idx [ i ][ j ] ], SubPicHeight[ subpic_grid_idx[ i ][ j ] ], and NumSubPics are derived as follows:

NumSubPics = 0
for( i = 0; i. < NumSubPicGridRows; i++ ) {
	for( j = 0; j < NumSubPicGridCols; j++ ) {
		if ( i = = 0)
			SubPicTop[ subpic_grid_idx[ i ][ j ] ] = 0
		else if( subpic_grid_idx[ i ][ j ]  !=  subpic_grid_idx[ i – 1 ][ j ] ) {
			SubPicTop[ subpic_grid_idx[ i ][ j ] ] = i
			SubPicHeight[ subpic_grid_idx[ i − 1][ j ] ] = i − SubPicTop[ subpic_grid_idx[ i − 1 ][ j ] ]
		}
		if ( j = = 0)
			SubPicLeft[ subpic_grid_idx[ i ][ j ] ] = 0	
		else if (subpic_grid_idx[ i ][ j ] != subpic_grid_idx[ i ][ j − 1 ] ) {
			SubPicLeft[ subpic_grid_idx[ i ][ j ] ] = j
			SubPicWidth[ subpic_grid_idx[ i ][ j ] ] = j − SubPicLeft[ subpic_grid_idx[ i ][ j − 1 ] ]
		}
		if ( i = = NumSubPicGridRows − 1)
			SubPicHeight[ subpic_grid_idx[ i ][ j ] ] = i − SubPicTop[ subpic_grid_idx[ i − 1 ][ j ] ] + 1			if (j = = NumSubPicGridRows − 1)
			SubPicWidth[ subpic_grid_idx[ i ][ j ] ] = j − SubPicLeft[ subpic_grid_idx[ i ][ j − 1 ] ] + 1
		if( subpic_grid_idx[ i ][ j ] > NumSubPics)
			NumSubPics = subpic_grid_idx[ i ][ j ]
	}
}

subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CVS is treated as a picture in the decoding process excluding in-loop filtering operations. subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to 0.

loop_filter_across_subpic_enabled_flag[ i ] equal to 1 specifies that in-loop filtering operations may be performed across the boundaries of the i-th subpicture in each coded picture in the CVS. loop_filter_across_subpic_enabled_flag[ i ] equal to 0 specifies that in-loop filtering operations are not performed across the boundaries of the i-th subpicture in each coded picture in the CVS. When not present, the value of loop_filter_across_subpic_enabled_pic_flag[ i ] is inferred to be equal to 1.

It is a requirement of bitstream conformance that the following constraints apply:

- For any two subpictures subpicA and subpicB, when the index of subpicA is less than the index of subpicB, any coded NAL unit of subPicA shall succeed any coded NAL unit of subPicB in decoding order.

- The shapes of the subpictures shall be such that each subpicture, when decoded, shall have its entire left boundary and entire top boundary consisting of picture boundaries or consisting of boundaries of previously decoded subpictures.

NumSubPics = 0
for( i = 0; i. < NumSubPicGridRows; i++ ) {
	for( j = 0; j < NumSubPicGridCols; j++ ) {
		if ( i = = 0)
			SubPicTop[ subpic_grid_idx[ i ][ j ] ] = 0
		else if( subpic_grid_idx[ i ][ j ]  !=  subpic_grid_idx[ i – 1 ][ j ] ) {
			SubPicTop[ subpic_grid_idx[ i ][ j ] ] = i
			SubPicHeight[ subpic_grid_idx[ i − 1][ j ] ] = i − SubPicTop[ subpic_grid_idx[ i − 1 ][ j ] ]
		}
		if ( j = = 0)
			SubPicLeft[ subpic_grid_idx[ i ][ j ] ] = 0	
		else if (subpic_grid_idx[ i ][ j ] != subpic_grid_idx[ i ][ j − 1 ] ) {
			SubPicLeft[ subpic_grid_idx[ i ][ j ] ] = j
			SubPicWidth[ subpic_grid_idx[ i ][ j ] ] = j − SubPicLeft[ subpic_grid_idx[ i ][ j − 1 ] ]
		}
		if ( i = = NumSubPicGridRows − 1)
			SubPicHeight[ subpic_grid_idx[ i ][ j ] ] = i − SubPicTop[ subpic_grid_idx[ i − 1 ][ j ] ] + 1

if (j = = NumSubPicGridRows − 1)
			SubPicWidth[ subpic_grid_idx[ i ][ j ] ] = j − SubPicLeft[ subpic_grid_idx[ i ][ j − 1 ] ] + 1
		if( subpic_grid_idx[ i ][ j ] > NumSubPics)
			NumSubPics = subpic_grid_idx[ i ][ j ]
	}
}

Figure 1 Shows an example of the subpicture structure

Figure 1 above shows an example of a subpicture grid that may be generated. The common subpicture grid indices are combined to form one subpicture.

# Proposal

In this document, we assert that the grid spacing used for the subpictures may be customized to provide greater flexibility to facilitate signalling of larger subpictures with greater ease. Here, we introduce signaling of non-uniform subpicture grid syntax. It is asserted that allowing the subpicture grid information to be either uniformly (i.e., existing structure) or non-uniformly can be useful as (i) it is more aligned with the tiles structure in the picture parameter set (PPS), and (ii) it could be a more efficient way of signalling larger subpictures as fewer subpicture grid indices need to be signalled.

Table 2 Shows an excerpt from the Subpicutre design

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| … |  |
| sps_seq_parameter_set_id | ue(v) |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| uniform_subpic_grid_flag | u(1) |
| if( uniform_ subpic_grid_flag ) { |  |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| }else { |  |
| num_subpic_grid_col_minus1 | ue(v) |
| num_subpic_grid_row_minus1 | ue(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| subpic_grid_rows_height_minus1[ i ] | ue(v) |
| for( i = 0; i < NumSubPicGridCols; i++ ) |  |
| subpic_grid_cols_width_minus1[ i ] | ue(v) |
| } |  |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

Where the semantics may be described as follows:

uniform_subpic_grid_flag equal to 1 specifies that tile column boundaries and likewise tile row boundaries are distributed uniformly across the picture and signalled using the syntax elements subpic_grid_col_width_minus1 and subpic_grid_row_height_minus1. uniform_subpic_spacing_flag equal to 0 specifies that tile column boundaries and likewise tile row boundaries may or may not be distributed uniformly across the picture and signalled using the syntax elements num_subpic_grid_col_minus1 and num_ subpic_grid_row_minus1 and a list of syntax element pairs subpic_grid_row_height_minus1[ i ] and subpic_col_width_minus1[ i ]. When not present, the value of uniform_subpic_grid_flag is inferred to be equal to 1.

subpics_present_flag equal to 1 indicates that subpicture parameters are present in the present in the SPS RBSP syntax. subpics_present_flag equal to 0 indicates that subpicture parameters are not present in the present in the SPS RBSP syntax.

NOTE 2 – When a bitstream is the result of a sub-bitstream extraction process and contains only a subset of the subpictures of the input bitstream to the sub-bitstream extraction process, it might be required to set the value of subpics_present_flag equal to 1 in the RBSP of the SPSs.

max_subpics_minus1 plus 1 specifies the maximum number of subpictures that may be present in the CVS. max_subpics_minus1 shall be in the range of 0 to 254. The value of 255 is reserved for future use by ITU-T | ISO/IEC.

subpic_grid_col_width_minus1 plus 1 specifies the width of each element of the subpicture identifier grid in units of 4 samples. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / 4) ) bits.

If uniform_subpic_grid_flag is equal to 1, the variable NumSubPicGridCols is derived as follows:

NumSubPicGridCols = ( pic_width_max_in_luma_samples + subpic_grid_col_width_minus1 * 4 + 3 ) /
		( subpic_grid_col_width_minus1 * 4 + 4 )

subpic_grid_row_height_minus1 plus 1 specifies the height of each element of the subpicture identifier grid in units of 4 samples. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / 4) ) bits.

If uniform_subpic_grid_flag is equal to 1, the variable NumSubPicGridRows is derived as follows:

NumSubPicGridRows = ( pic_height_max_in_luma_samples + subpic_grid_row_height_minus1 * 4 + 3 ) /
		( subpic_grid_row_height_minus1 * 4 + 4 )

num_subpic_grid_col_minus1 plus 1 specifies the value of the variable NumSubPicGridCols.

num_subpic_grid_row_minus1 plus 1 specifies the value of the variable NumSubPicGridRows.

subpic_grid_rows_height_minus1[i] plus 1 specifies the height of the i-th row.

subpic_grid_cols_width_minus1[i] plus 1 specifies the width of the i-th column.

subpic_grid_idx[ i ][ j ] specifies the subpicture index of the grid position (i, j). The length of the syntax element is Ceil( Log2( max_subpics_minus1 + 1 ) ) bits.

The variables SubPicTop[ subpic_grid_idx[ i ][ j ] ], SubPicLeft[ subpic_grid_idx[ i ][ j ] ], SubPicWidth[ subpic_grid_idx [ i ][ j ] ], SubPicHeight[ subpic_grid_idx[ i ][ j ] ], and NumSubPics are derived as follows:

NumSubPics = 0
for( i = 0; i. < NumSubPicGridRows; i++ ) {
	for( j = 0; j < NumSubPicGridCols; j++ ) {
		if ( i = = 0)
			SubPicTop[ subpic_grid_idx[ i ][ j ] ] = 0
		else if( subpic_grid_idx[ i ][ j ]  !=  subpic_grid_idx[ i – 1 ][ j ] ) {
			SubPicTop[ subpic_grid_idx[ i ][ j ] ] = i
			SubPicHeight[ subpic_grid_idx[ i − 1][ j ] ] = i − SubPicTop[ subpic_grid_idx[ i − 1 ][ j ] ]
		}
		if ( j = = 0)
			SubPicLeft[ subpic_grid_idx[ i ][ j ] ] = 0	
		else if (subpic_grid_idx[ i ][ j ] != subpic_grid_idx[ i ][ j − 1 ] ) {
			SubPicLeft[ subpic_grid_idx[ i ][ j ] ] = j
			SubPicWidth[ subpic_grid_idx[ i ][ j ] ] = j − SubPicLeft[ subpic_grid_idx[ i ][ j − 1 ] ]
		}
		if ( i = = NumSubPicGridRows − 1)
			SubPicHeight[ subpic_grid_idx[ i ][ j ] ] = i − SubPicTop[ subpic_grid_idx[ i − 1 ][ j ] ] + 1

if (j = = NumSubPicGridRows − 1)
			SubPicWidth[ subpic_grid_idx[ i ][ j ] ] = j − SubPicLeft[ subpic_grid_idx[ i ][ j − 1 ] ] + 1
		if( subpic_grid_idx[ i ][ j ] > NumSubPics)
			NumSubPics = subpic_grid_idx[ i ][ j ]
	}
}

subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CVS is treated as a picture in the decoding process excluding in-loop filtering operations. subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to 0.

loop_filter_across_subpic_enabled_flag[ i ] equal to 1 specifies that in-loop filtering operations may be performed across the boundaries of the i-th subpicture in each coded picture in the CVS. loop_filter_across_subpic_enabled_flag[ i ] equal to 0 specifies that in-loop filtering operations are not performed across the boundaries of the i-th subpicture in each coded picture in the CVS. When not present, the value of loop_filter_across_subpic_enabled_pic_flag[ i ] is inferred to be equal to 1.

It is a requirement of bitstream conformance that the following constraints apply:

- For any two subpictures subpicA and subpicB, when the index of subpicA is less than the index of subpicB, any coded NAL unit of subPicA shall succeed any coded NAL unit of subPicB in decoding order.

- The shapes of the subpictures shall be such that each subpicture, when decoded, shall have its entire left boundary and entire top boundary consisting of picture boundaries or consisting of boundaries of previously decoded subpictures.

The variables SubPicIdx, SubPicLeftBoundaryPos, SubPicTopBoundaryPos, SubPicRightBoundaryPos, and SubPicBotBoundaryPos are derived as follows:

SubPicIdx = CtbToSubPicIdx[ CtbAddrBsToRs[ FirstCtbAddrBs[ SliceBrickIdx[ 0 ] ] ] ]
if( subpic_treated_as_pic_flag[ SubPicIdx ] ) {

if(uniform_subpic_grid_flag){
	SubPicLeftBoundaryPos = SubPicLeft[ SubPicIdx ] * ( subpic_grid_col_width_minus1 + 1 ) * 4
	SubPicRightBoundaryPos = ( SubPicLeft[ SubPicIdx ] + SubPicWidth[ SubPicIdx ] ) *
				( subpic_grid_col_width_minus1 + 1 ) * 4	
	SubPicTopBoundaryPos = SubPicTop[ SubPicIdx ] * ( subpic_grid_row_height_minus1 + 1 )* 4
	SubPicBotBoundaryPos = ( SubPicTop[ SubPicIdx ] + SubPicHeight[ SubPicIdx ] ) *
				( subpic_grid_row_height_minus1 + 1 ) * 4

} else {
 SubPicLeftBoundaryPos = SubPicLeft[ SubPicIdx ] * ( subpic_grid_cols_width_minus1[ SubPicIdx]+ 1 ) * 4
	SubPicRightBoundaryPos = ( SubPicLeft[ SubPicIdx ] + SubPicWidth[ SubPicIdx ] ) *
				( subpic_grid_cols_width_minus1 [SubPicIdx] + 1 ) * 4	
	SubPicTopBoundaryPos = SubPicTop[ SubPicIdx ] * ( subpic_grid_rows_height_minus1[SubPicIdx] + 1 )* 4
	SubPicBotBoundaryPos = ( SubPicTop[ SubPicIdx ] + SubPicHeight[ SubPicIdx ] ) *
				( subpic_grid_rows_height_minus1[SubPicIdx] + 1 ) * 4

}

}

The list CtbToSubPicIdx[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in picture raster scan to a subpicture index, is derived as follows:

for( ctbAddrRs = 0; ctbAddrRs < PicSizeInCtbsY; ctbAddrRs++ ) {
	posX = ctbAddrRs % PicWidthInCtbsY * CtbSizeY 
	posY = ctbAddrRs / PicWidthInCtbsY * CtbSizeY 
	CtbToSubPicIdx[ ctbAddrRs ] = −1
	for( i = 0; CtbToSubPicIdx[ ctbAddrRs ] < 0  && i < NumSubPics; i++ ) {
	if(uniform_subpic_grid_flag) {

if( ( posX  >=  SubPicLeft[ i ] * ( subpic_grid_col_width_minus1 + 1 ) * 4 )  &&
				( posX < ( SubPicLeft[ i ] + SubPicWidth[ i ] ) *
						( subpic_grid_col_width_minus1 + 1 ) * 4 )  &&	
				( posY  >=  SubPicTop[ i ] *
						( subpic_grid_row_height_minus1 + 1 ) * 4 )  &&
				( posY < ( SubPicTop[ i ] + SubPicHeight[ i ] ) *
						( subpic_grid_row_height_minus1 + 1 ) * 4 ) )
			CtbToSubPicIdx[ ctbAddrRs ] = i
	}else {

if( ( posX  >=  SubPicLeft[ i ] * ( subpic_grid_cols_width_minus1[i] + 1 ) * 4 )  &&
				( posX < ( SubPicLeft[ i ] + SubPicWidth[ i ] ) *
						( subpic_grid_cols_width_minus1[i] + 1 ) * 4 )  &&	
				( posY  >=  SubPicTop[ i ] *
						( subpic_grid_rows_height_minus1[i] + 1 ) * 4 )  &&
				( posY < ( SubPicTop[ i ] + SubPicHeight[ i ] ) *
						( subpic_grid_rows_height_minus1[i] + 1 ) * 4 ) )
			CtbToSubPicIdx[ ctbAddrRs ] = i
	}
}

# References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, Document of Joint Video Experts Team (JVET), JVET-O2001, 15th Meeting: Gothenburg, SE, 3-12 July 2019.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).