| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0475 |
| --- | --- |

| Title: | Non-CE8: Simplification of palette predictor update for small CUs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Ted Hsieh, Vadim Seregin, Marta Karczewicz | Tel: Email: | +1-858-8452163 yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

- Method 1

- Method 2

7.4.9.6	Palette coding semantics

In the following semantics, the array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. The array indices xC, yC specify the location ( xC, yC ) of the sample relative to the top-left luma sample of the picture. The array index startComp specifies the first colour component of the current palette table. startComp equal to 0 indicates the Y component; startComp equal to 1 indicates the Cb component; startComp equal to 2 indicates the Cr component. numComps specifies the number of colour components in the current palette table.

The predictor palette consists of palette entries from previous coding units that are used to predict the entries in the current palette.

The variable palette_max_size specifies the maximum allowed palette size and is set equal to 31.The variable PredictorPaletteSize[ startComp ] specifies the size of the predictor palette for the first colour component of the current palette table startComp. PredictorPaletteSize is derived as specified in clause 8.4.5.3.

The variable PalettePredictorEntryReuseFlags[ i ] equal to 1 specifies that the i-th entry in the predictor palette is reused in the current palette. PalettePredictorEntryReuseFlags[ i ] equal to 0 specifies that the i-th entry in the predictor palette is not an entry in the current palette. All elements of the array PalettePredictorEntryReuseFlags[ i ] are initialized to 0.

palette_predictor_run is used to determine the number of zeros that precede a non-zero entry in the array PalettePredictorEntryReuseFlags.

It is a requirement of bitstream conformance that the value of palette_predictor_run shall be in the range of 0 to ( PredictorPaletteSize − predictorEntryIdx ), inclusive, where predictorEntryIdx corresponds to the current position in the array PalettePredictorEntryReuseFlags. The variable NumPredictedPaletteEntries specifies the number of entries in the current palette that are reused from the predictor palette. The value of NumPredictedPaletteEntries shall be in the range of 0 to palette_max_size, inclusive.

num_signalled_palette_entries specifies the number of entries in the current palette that are explicitly signalled for the first colour component of the current palette table startComp.

When num_signalled_palette_entries is not present, it is inferred to be equal to 0.

The variable CurrentPaletteSize[ startComp ] specifies the size of the current palette for the first colour component of the current palette table startComp and is derived as follows:

CurrentPaletteSize[ startComp ] = NumPredictedPaletteEntries + num_signalled_palette_entries	(7-155)

The value of CurrentPaletteSize[ startComp ] shall be in the range of 0 to palette_max_size, inclusive.

new_palette_entries[ cIdx ][ i ] specifies the value for the i-th signalled palette entry for the colour component cIdx.

The variable PredictorPaletteEntries[ cIdx ][ i ] specifies the i-th element in the predictor palette for the colour component cIdx.

The variable CurrentPaletteEntries[ cIdx ][ i ] specifies the i-th element in the current palette for the colour component cIdx and is derived as follows:

numPredictedPaletteEntries = 0

numPredictorentriesUsed = ( cbWidth * cbHeight < PredictorPaletteSize[ startComp ])? cbWidth * cbHeight : PredictorPaletteSize[ startComp ]
for( i = 0; i < numPredictorentriesUsed; i++ )
	if( PalettePredictorEntryReuseFlags[ i ] ) {
		for( cIdx =startComp; cIdx < ( startComp + numComps ); cIdx++ )
			CurrentPaletteEntries[ cIdx ][ numPredictedPaletteEntries ] = PredictorPaletteEntries[ cIdx ][ i ]
		numPredictedPaletteEntries++
		}

for( cIdx = startComp; cIdx < (startComp + numComps); cIdx++)	(7-156)
	for( i = 0; i < num_signalled_palette_entries[startComp]; i++ )
		CurrentPaletteEntries[ cIdx ][ numPredictedPaletteEntries + i ] = new_palette_entries[ cIdx ][ i ]

palette_escape_val_present_flag equal to 1 specifies that the current coding unit contains at least one escape coded sample. escape_val_present_flag equal to 0 specifies that there are no escape coded samples in the current coding unit. When not present, the value of palette_escape_val_present_flag is inferred to be equal to 1.

The variable MaxPaletteIndex specifies the maximum possible value for a palette index for the current coding unit. The value of MaxPaletteIndex is set equal to CurrentPaletteSize[ startComp ] − 1 + palette_escape_val_present_flag.

#### 8.4.5.3	Decoding process for palette mode

Inputs to this process are:

- a location ( xCb, yCb ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

- a variable startComp specifies the first colour component in the palette table,

- a variable cIdx specifying the colour component of the current block,

- two variables nCbW and nCbH specifying the width and height of the current block, respectively.

Output of this process is an array recSamples[ x ][ y ], with x = 0.. nCbW − 1, y = 0.. nCbH − 1 specifying reconstructed sample values for the block.

Depending on the value of cIdx, the variables nSubWidth and nSubHeight are derived as follows:

- If cIdx is equal to 0, nSubWidth is set to 1 and nSubHeight is set to 1.

- Otherwise, nSubWidth is set to SubWidthC and nSubHeight is set to SubHeightC.

The ( nCbW x nCbH ) block of the reconstructed sample array recSamples at location ( xCb, yCb ) is represented by recSamples[ x ][ y ] with x = 0..nCTbW − 1 and y = 0..nCbH − 1, and the value of recSamples[ x ][ y ] for each x in the range of 0 to nCbW − 1, inclusive, and each y in the range of 0 to nCbH − 1, inclusive, is derived as follows:

- The variables xL and yL are derived as follows:

xL = palette_transpose_flag  ?  x * nSubHeight  :  x * nSubWidth	(8-268)

yL = palette_transpose_flag  ?  y * nSubWidth  :  y * nSubHeight	(8-269)

- The variable bIsEscapeSample is derived as follows:

- If PaletteIndexMap[ xCb + xL ][ yCb + yL ] is equal to MaxPaletteIndex and palette_escape_val_present_flag is equal to 1, bIsEscapeSample is set equal to 1.

- Otherwise, bIsEscapeSample is set equal to 0.

- If bIsEscapeSample is equal to 0, the following applies:

recSamples[ x ][ y ] = CurrentPaletteEntries[ cIdx ][ PaletteIndexMap[ xCb + xL ][ yCb + yL ] ]	(8-270)

- Otherwise, if cu_transquant_bypass_flag is equal to 1, the following applies:

recSamples[ x ][ y ] = PaletteEscapeVal[ cIdx ][ xCb + xL ][ yCb + yL ]	(8-271)

- Otherwise (bIsEscapeSample is equal to 1 and cu_transquant_bypass_flag is equal to 0), the following ordered steps apply:

- The quantization parameter qP is derived as follows:

- If cIdx is equal to 0,

qP = Max( 0, Qp′Y )		(8-272)

- Otherwise, if cIdx is equal to 1,

qP = Max( 0, Qp′Cb )		(8-273)

- Otherwise (cIdx is equal to 2),

qP = Max( 0, Qp′Cr )		(8-274)

- The variables bitDepth is derived as follows:

bitDepth = ( cIdx  = =  0 )  ?  BitDepthY  :  BitDepthC	(8-275)

- The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

- The following applies:

tmpVal = ( PaletteEscapeVal[ cIdx ][ xCb + xL ][ yCb + yL ] *
			levelScale[ qP%6 ] )  <<  ( qP / 6 ) + 32 )  >>  6	(8-276)

recSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, tmpVal )	(8-277)

Define PaletteMaxPredictorSize as the maximum allowed predictor palette size, which is set equal to 63, when one of the following conditions is true:

- cIdx is equal to 0 and numComps is equal to 1;

- cIdx is equal to 2;

- cbWidth * cbHeight >= PaletteMaxPredictorSize

, the variable PredictorPaletteSize[startComp] and the array PredictorPaletteEntries are derived or modified as follows:

for( i = 0; i < CurrentPaletteSize[ startComp ]; i++ )
	for( cIdx = startComp; cIdx < (startComp + numComps); cIdx++ )
		newPredictorPaletteEntries[ cIdx ][ i ] = CurrentPaletteEntries[ cIdx ][ i ]
newPredictorPaletteSize = CurrentPaletteSize[ startComp ]
for( i = 0; i < PredictorPaletteSize  &&  newPredictorPaletteSize < PaletteMaxPredictorSize; i++ )
	if( !PalettePredictorEntryReuseFlags[ i ] ) {
		for( cIdx = startComp; cIdx < (startComp + numComps); cIdx++ )	(8-278)
			newPredictorPaletteEntries[ cIdx ][ newPredictorPaletteSize ] =
				PredictorPaletteEntries[ cIdx ][ i ]
		newPredictorPaletteSize++
	}
for( cIdx = startComp; cIdx < ( startComp + numComps ); cIdx++ )
	for( i = 0; i < newPredictorPaletteSize; i++ )
		PredictorPaletteEntries[ cIdx ][ i ] = newPredictorPaletteEntries[ cIdx ][ i ]
PredictorPaletteSize[ startComp ] = newPredictorPaletteSize

It is a requirement of bitstream conformance that the value of PredictorPaletteSize[ startComp ] shall be in the range of 0 to PaletteMaxPredictorSize, inclusive.

# Reference

- Y.-H. Chao, C.-H. Hung, W.-J. Chien, V. Seregin, M. Karczewicz, Y.-C. Sun, T.-S. Chang, J. Lou ,W. Zhu, L. Zhang, J. Xu, K. Zhang, H. Liu, “CE8-2.1: Palette mode in HEVC” , JVET-O0119, 15th meeting, Gothenburg, SE, July 2019.

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools”, JVET-O2028, 15th meeting, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)