| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0480-v2 |
| --- | --- |

| Title: | AHG 17: On simplification of subpicture design |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Sühring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

At the last-meeting, a a fine granular chess-board like mapping of blocks to subpictures was adopted as a starting point. This contribution proposes asserted simplifications of the current subpicture design and provides solutions for identified drawbacks of the current design. It is proposed to:

- Move subpictures syntax from SPS to PPS and align with tile/slice structures.

- Add appropriate scanning process to derive SubPicBrickId.

- Adjust slice header with subpicture based addressing mechanism.

- Relax the requirement on AUD.

Revision 1 corrects the condition of the AUD constraint.

# Introduction

It is asserted that the current design of subpictures is more complex than required. The following drawbacks have been identified:

- The subpictures are defined within the SPS with a grid that is not necessarily aligned with CTUs or tile sizes and a mapping of a chess-board like pattern to tiles and slices needs to be derived. There is no envisioned use-case for slices to contain more than a subpicture. Therefore, it is considered that the granularity for this mapping should be based on slices.

- Signalling all subpictures information at SPS level would make difficult to dynamically change the order of subpicture IDs signalled within the slice headers, which is useful in some scenarios.

- During the last meeting the AUD was mandated to be able to identify the start of an AU due to the addressing scheme currently used which is required for “merging/extraction” use-cases with subpictures although for typical use-cases where no subpictures are used, this is not required.

# Proposal

The proposal consists of multiple parts:

- Move subpictures syntax from SPS to PPS and align with tile/slice structures.

- Add appropriate scanning process to derive SubPicBrickId.

- Adjust slice header with subpicture based addressing mechanism.

- Relax the requirement on AUD.

The design would allow HEVC-like detection of AU start (SH flag, no AUD) when no subpictures are used.

Subpicture Id association in PPS may change in CVS to facilitate subpicture based streaming cases.

It is asserted that the design provides sufficient flexibility, while keeping the goal of allowing simple rewriting and merging operations.

The following summerizes the specification changes in the attached document for convenience:

- Move subpictures syntax to the PPS and align with tile/slice structures

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| […] |  |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_num_sub_pics_minus1 | ue(v) |
| […] | ue(v) |

sps_num_sub_pics_minus1 plus 1 specifies the number of subpictures partitioning the pictures of the CVS. sps_num_sub_pics_minus1 shall be in the range of 0 to 254. The value of 255 is reserved for future use by ITU-T | ISO/IEC.

It is a requirement of bitstream conformance that when sps_num_sub_pics_minus1 is greater than 0, the values of the tiling sytax elements [Ed. RS: list according to meeting decision] are the same in all PPS that are active within a CVS. sps_num_sub_pics_minus1 equal to 0 does not impose such a constraint.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| … |  |
| loop_filter_across_bricks_enabled_flag | u(1) |
| if( loop_filter_across_bricks_enabled_flag ) |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| pps_num_sub_pics_minus1 | ue(v) |
| if( rect_slice_flag ) { |  |
| signalled_sub_pic_id_flag | u(1) |
| if( signalled_sub_pic_id_flag ) { |  |
| signalled_sub_pic_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| pps_sub_pic_id[ i ] | u(v) |
| } |  |
| } |  |
| for( i = 0; i <= pps_num_sub_pics_minus1; i++ ) { |  |
| sub_pic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_sub_pic_enabled_flag[ i ] | u(1) |
| } |  |
| … |  |
| } |  |

pps_num_sub_pics_minus1 plus 1 specifies the number of subpictures partitioning the picture. The value of pps_num_sub_pics_minus1 shall be constant in the CVS. pps_num_sub_pics_minus1 shall be in the range of 0 to 254. The value of 255 is reserved for future use by ITU-T | ISO/IEC.

It is a requirement of bitstream conformance that the value of pps_num_sub_pics_minus1 is equal to the value of pps_num_sub_pics_minus1 in all active PPS in the bitstream.

It is a requirement of bitstream conformance that when pps_num_sub_pics_minus1 is greater than 0, the values of the tiling sytax elements [Ed. RS: list according to meeting decision] are the same in all PPS that are active within a CVS. pps_num_sub_pics_minus1 equal to 0 does not impose such a constraint.

signalled_sub_pic_id_flag equal to 1 specifies that the subpicture ID for each slice is signalled. signalled_sub_pic_id_flag equal to 0 specifies that subpicture IDs are not signalled. When rect_slice_flag is equal to 0, the value of signalled_sub_pic_id_flag is inferred to be equal to 0.

signalled_sub_pic_id_length_minus1 plus 1 specifies the number of bits used to represent the syntax element sub_pic_id[ i ] when present, and the syntax element sh_sub_pic_id in slice headers. The value of signalled_sub_pic_id_length_minus1 shall be in the range of 0 to 15, inclusive. When not present, the value of signalled_sub_pic_id_length_minus1 is inferred to be equal to Ceil( Log2( Max( 2, num_sub_pics_minus1 + 1 ) ) ) − 1.

It is a requirement of bitstream conformance that when signalled_sub_pic_id_flag is equal to 0 num_sub_pics_minus1 shall be equal to num_slices_in_pic_minus1.

pps_sub_pic_id[ i ] specifies the subpicture identifier of the i-th slice of a coded picture. The length of the pps_sub_pic_id[ i ] syntax element is signalled_sub_pic_id_length_minus1 + 1 bits. When not present, the value of pps_sub_pic_id[ i ] is inferred to be equal to i, for each i in the range of 0 to num_sub_pics_minus1, inclusive.

sub_pic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CVS is treated as a picture in the decoding process excluding in-loop filtering operations. sub_pic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of sub_pic_treated_as_pic_flag[ i ] is inferred to be equal to 0.

loop_filter_across_sub_pic_enabled_flag[ i ] equal to 1 specifies that in-loop filtering operations may be performed across the boundaries of the i-th subpicture in each coded picture in the CVS. loop_filter_across_sub_pic_enabled_flag[ i ] equal to 0 specifies that in-loop filtering operations are not performed across the boundaries of the i-th subpicture in each coded picture in the CVS. When not present, the value of loop_filter_across_subpic_enabled_pic_flag[ i ] is inferred to be equal to 1.

It is a requirement of bitstream conformance that the following constraints apply:

- For any two subpictures subpicA and subpicB, when the index of subpicA is less than the index of subpicB, any coded NAL unit of subPicA shall succeed any coded NAL unit of subPicB in decoding order.

- The shapes of the subpictures shall be such that each subpicture, when decoded, shall have its entire left boundary and entire top boundary consisting of picture boundaries or consisting of boundaries of previously decoded subpictures.

It is a requirement of bitstream conformance that when pps_num_sub_pic_minus1 is not equal to 0, the tiling information and subpictures sizes are static in the CVS.

- Add appropriate scanning process to derive SubPicBrickId:

6.5.1 CTB raster scanning, tile scanning, brick scanning, and subpicture scanning processes

The list SubPicBrickId[ sliceIdx ] for sliceIdx ranging from 0 to num_slices_in_pic_minus1, inclusive, specifying the conversion from slice index to SubPicBrick ID and the list FirstBrickIdInSubPic[ subPicId] for subPicId in the range from 0 to max_sub_pics_minus1, inclusive, specifying the conversion from subPic ID to brick ID of the first slice in a sub picture, NumBricksInSubPic, specifying the number of bricks per subpicture are derived as follows:

SubPicBrickId[ 0 ] = 0
for ( i = 0, i <= max_sub_pics_minus1, i++) {
	FirstBrickIdInSubPic[ i ] = 0
	NumBricksInSubPic[ i ] = 0
}
FirstBrickIdInSubPic[ pps_sub_pic_id[ 0 ] ] = 0
NumBricksInSubPic[ pps_sub_pic_id[ 0 ] ] = NumBricksInSlice[ 0 ]
for( sliceIdx = 1, sliceIdx <= num_slices_in_pic_minus1 ; sliceIdx ++ ) {	(6-XX)
	if ( pps_sub_pic_id[ sliceIdx ] != pps_sub_pic_id[ sliceIdx − 1 ] )
			FirstBrickIdInSubPic[ pps_sub_pic_id[ sliceIdx ] ] = TopLeftBrickIdx[ sliceIdx ]
	SubPicBrickId[ sliceIdx ] = NumBricksInSubPic[ pps_sub_pic_id[ sliceIdx ] ] 	
	NumBricksInSubPic[ pps_sub_pic_id[ sliceIdx ] ] += NumBricksInSlice[ sliceIdx ]
}

- Relax the requirement on AUD:

##### 7.4.2.4.3 Order of NAL units and coded pictures and their association to layer access units and access units

When sps_num_sub_pic_minus1 is not equal to 0, each access unit starts with an access unit delimiter NAL unit. There shall be at most one access unit delimiter NAL unit in any layer access unit.

- Adjust slice header with subpicture based addressing mechanism

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| first_slice_in_sub_pic_flag | u(1) |
| if( rect_slice_flag ) |  |
| sh_sub_pic_id | u(v) |
| if( (rect_slice_flag ¦ ¦ NumBricksInPic > 1) && !first_slice_in_sub_pic_flag) |  |
| slice_address | u(v) |
| … |  |
| } |  |

first_slice_in_sub_pic_flag equal to 1 specifies that the slice is the first slice of the subpicture in decoding order. first_slice_in_sub_pic_flag equal to 0 specifies that the slice is not the first slice of the subpicture in decoding order.

sh_sub_pic_id specifies the sub picture identifier of the slice. When not present, the value of sh_sub_pic_id is inferred to be equal to 0. The length of slice_address is signalled_sub_pic_id_length_minus1 + 1 bits.

slice_address specifies the slice address of the slice. When not present, the value of slice_address is inferred to be equal to 0.

If rect_slice_flag is equal to 0, the following applies:

- The slice address is the brick ID as specified by Equation (7-59).

- The length of slice_address is Ceil( Log2 ( NumBricksInPic ) ) bits.

- The value of slice_address shall be in the range of 0 to NumBricksInPic − 1, inclusive.

Otherwise (rect_slice_flag is equal to 1), the following applies:

- The slice address is the subPic brick ID as specified by Equation 6-XX.

- The length of slice_address is Ceil( Log2 ( NumBricksInSubPic[ j ] ) ) bits.

It is a requirement of bitstream conformance that the following constraints apply:

- The value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit of the same subpicture of the same coded picture.

- When rect_slice_flag is equal to 0, the slices of a subpicture shall be in increasing order of their slice_address values.

- The shapes of the slices of a picture shall be such that each brick, when decoded, shall have its entire left boundary and entire top boundary consisting of a picture boundary or consisting of boundaries of previously decoded brick(s).

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).