| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0487 |
| --- | --- |

| Title: | CE8-related : Binarization of palette escape value |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Jie Zhao Junghak Nam Jungah Choi Seung Hwan Kim Jaehyun Lim | Tel: Email: | +82-2-6912-6561 sunmi.yoo@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In VVC, the escape value is binarized with Exponential-Golomb with k equal to 3 (EG3). In the worst case, the bin length becomes 18 for a sample, which is more than fixed length coding (FL) method. Therefore, it is proposed that the escape value is binarized with truncated binary coding (TB) with a quantized maximum value. The experimental results reportedly show that the proposed scheme achieves -0.06% (AI) / 0.00% (RA) / -0.04% (LDB), respectively, when dual tree is set to 1 and under the test conditions in CE8. In the dual tree off case, the test results are -0.10% (AI) / -0.08% (RA) / 0.09% (LDB), correspondingly.

Introduction

At the last meeting, palette coding is adopted in the current VVC [1]. The palette coding is beneficial when there are limited pixels and the texture is homogeneous in a CU when the coded image is computer generated. If there is a pixel which has different values from other pixels, it is coded not as a regular palette entry, but an escape value. The sample values of each component of the escape value are coded and transmitted after quantized. Each escape sample value is binarized with EG3, which has the bin length from 4 to 18 when the sample value is 0 to (1 << bitdepth) – 1, where the bitdepth in the current VTM is set to 10. Then the binarized symbol is bypass coded. In the worst case, the escape value is coded with 54 bits, whereas it needs only 30 bits when the pixel is coded with FL coding. Furthermore, since the Qp for the escape value is always bdoffset applied, the pixel cannot be restored completely when the initial Qp is set equal to 4 (which means the quantization step is equal to 1), even if the input bitdepth of the target sequence is 10. Therefore, the efficient escape value coding is provided in this proposal.

Proposed method

Truncated binary coding (TB) for escape value

In the proposed method, the escape value is binarized with TB, with a maximum value (cMax) equal to Q((1 << bitdepth) – 1), where Q(x) indicates quantization. If the Qp is equal to 4, the input and output value of Q(x) are the same. Once the bitdepth and the Qp are transmitted, the quantized cMax can be easily calculated, or it can be pre-calculated and referenced by being tabulated for each Qp (0 to 63). Moreover, when the cMax is not equal to power of 2, some values can be represented with the less bin length by 1 when using TB instead of FL. The bin length comparison between EG3 and TB for the regular Qps (22, 27, 32, 37) are shown in Table 1. Using TB can be more efficient since EG3 is biased to the low values.

Table 1 Required bin length according to Qps

| QP | Maximum value | Bin length after binarized with synVal equal to 0 to maximum value |  |
| --- | --- | --- | --- |
|  |  | EG3 | TB |
| 22 | 128 | 4~12 | 7~8 |
| 27 | 72 | 4~10 | 6~7 |
| 32 | 40 | 4~8 | 5~6 |
| 37 | 23 | 4~6 | 4~5 |

Input bitdpeth indication in SPS

As mentioned above, the escape values of 10-bit input sequence cannot be fully represented when the bdoffset is applied to the Qp. To prevent the problem, it is necessary to send the input bitdepth. It can be denoted in one of two methods like below.

- To send input_bit_depth_luma_minus8 and input_bit_depth_chroma_minus8 explicitly in SPS

- Define min_qp_prime_ts_chroma_minus4 and derive the input bitdepth from min_qp_prime_ts_minus4 and min_qp_prime_ts_chroma_minus4.

In the proposed method, it is implemented as number 1.

Experimental results

The effects of the proposed method are shown below. The tests are under the test condition described in JVET-O2028. Additionally, the low Qp test is accompanied as well.

CTC Qps (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -0.13% | -0.07% | -0.12% | 101% | 99% | -0.40% | -0.52% | -0.57% | 100% | 100% |
| TGM | -0.09% | -0.08% | -0.03% | 101% | 100% | 0.13% | -0.10% | -0.07% | 101% | 100% |
| A | -0.03% | 0.02% | 0.01% | 100% | 101% | 0.02% | 0.01% | 0.02% | 101% | 100% |
| M | -0.01% | -0.07% | -0.02% | 100% | 100% | -0.21% | -0.39% | -0.43% | 100% | 99% |
| CC | 0.00% | 0.02% | -0.01% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | -0.06% | -0.04% | -0.04% | 100% | 100% | -0.10% | -0.23% | -0.24% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -0.18% | -0.16% | -0.18% | 100% | 99% | -0.24% | -0.25% | -0.25% | 100% | 97% |
| TGM | 0.06% | -0.14% | -0.24% | 100% | 99% | -0.04% | -0.06% | -0.03% | 100% | 98% |
| A | 0.11% | 0.06% | 0.02% | 100% | 100% | -0.01% | -0.02% | -0.02% | 100% | 100% |
| M | 0.06% | -0.09% | -0.07% | 100% | 99% | -0.06% | -0.08% | -0.30% | 100% | 99% |
| CC | 0.00% | 0.00% | 0.05% | 100% | 102% | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Overall | 0.00% | -0.08% | -0.11% | 100% | 99% | -0.08% | -0.10% | -0.13% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -0.06% | -0.08% | -0.14% | 100% | 99% | -0.01% | 0.06% | -0.10% | 99% | 99% |
| TGM | -0.07% | -0.17% | 0.56% | 100% | 99% | 0.30% | -0.02% | 0.55% | 99% | 100% |
| A | 0.02% | 0.05% | 0.04% | 100% | 99% | -0.06% | 0.03% | 0.08% | 100% | 101% |
| M | -0.09% | -0.08% | -0.44% | 100% | 98% | 0.19% | 0.11% | -0.41% | 99% | 99% |
| CC | 0.01% | -0.07% | 0.02% | 100% | 99% | 0.00% | 0.00% | 0.01% | 99% | 99% |
| Overall | -0.04% | -0.08% | 0.03% | 100% | 99% | 0.09% | 0.04% | 0.05% | 99% | 99% |

Low Qps (2, 7, 12, 17)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -2.02% | -2.59% | -2.58% | 100% | 101% | -3.67% | -3.67% | -3.81% | 100% | 100% |
| TGM | -0.77% | -0.93% | -0.86% | 100% | 101% | -1.11% | -1.24% | -1.18% | 99% | 100% |
| A | -0.24% | -0.55% | -0.47% | 100% | 100% | -0.20% | -0.85% | -0.79% | 100% | 100% |
| M | -1.69% | -1.95% | -2.01% | 100% | 100% | -1.45% | -1.45% | -1.50% | 100% | 100% |
| CC | 0.01% | -0.02% | 0.00% | 99% | 100% | 0.00% | 0.01% | -0.01% | 100% | 100% |
| Overall | -1.06% | -1.35% | -1.33% | 100% | 100% | -1.50% | -1.66% | -1.68% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -0.94% | -1.15% | -1.27% | 100% | 100% | -1.81% | -2.08% | -2.12% | 101% | 100% |
| TGM | -0.28% | -0.10% | -0.30% | 99% | 101% | -0.43% | -0.51% | -0.72% | 99% | 100% |
| A | 0.04% | -0.10% | -0.09% | 100% | 100% | -0.12% | -0.08% | -0.12% | 100% | 100% |
| M | -0.95% | 0.26% | 0.11% | 100% | 100% | -0.58% | -0.56% | -0.70% | 100% | 100% |
| CC | 0.00% | 0.00% | -0.02% | 100% | 100% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | -0.47% | -0.28% | -0.39% | 100% | 100% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -1.30% | -1.41% | -1.42% | 99% | 100% | -1.78% | -1.77% | -1.79% | 100% | 100% |
| TGM | -0.17% | -0.40% | -0.38% | 101% | 103% | -0.32% | -0.31% | -0.30% | 99% | 101% |
| A | -0.05% | -0.16% | -0.12% | 100% | 100% | -0.54% | -0.12% | -0.08% | 100% | 100% |
| M | -0.54% | -0.09% | 0.05% | 100% | 101% | -0.32% | -0.41% | -0.52% | 100% | 101% |
| CC | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | -0.68% | -0.62% | -0.63% | 100% | 100% |

References

- Y.-H. Chao, C.-H. Hung, W.-J. Chien, V. Seregin, M. Karczewicz, Y.-C. Sun, T.-S. Chang, J. Lou, W. Zhu, L. Zhang, J. Xu, K. Zhang, H. Liu, “CE8-2.1: Palette mode in HEVC”, JVET-O0119,

Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).