| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0501 |
| --- | --- |

| Title: | Non-CE6: Cleanup on MTS related HLS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Shan Liu | Tel: Email: | xinzzhao@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In VVC Draft 6, there are three MTS related coding tools, including intra implicit MTS, intra explicit MTS and inter explicit MTS, and the enabling/disabling of these three tools are controlled by SPS flags. According to VVC Draft 6, in order to enable inter explicit MTS for inter prediction residuals, either implicit or explicit MTS must be enabled for intra prediction residuals, which is not necessary. In this contribution, it is proposed to decouple the HLS controlling of intra (either implicit or explicit) MTS and inter explicit MTS, such that intra MTS and inter explicit MTS can be enabled/disabled individually.

# Problem Statement

In VVC Draft 6, the MTS related coding tools, including intra implicit MTS, intra explicit MTS and inter explicit MTS, are controlled by three SPS flags, as specified below,

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| sps_mts_enabled_flag | u(1) |
| if( sps_mts_enabled_flag ) { |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| } |  |
| …… |  |
| } |  |

The enabling of intra implicit MTS, intra explicit MTS and inter explicit MTS are described in the following table.

| Tool | Enabling condition |
| --- | --- |
| Intra implicit MTS | sps_mts_enabled_flag == 1 sps_explicit_mts_intra_enabled_flag == 0 |
| Intra explicit MTS | sps_mts_enabled_flag == 1 sps_explicit_mts_intra_enabled_flag == 1 |
| Inter explicit MTS | sps_mts_enabled_flag == 1 sps_explicit_mts_inter_enabled_flag == 1 |

It is noted that, to enable inter explicit MTS, either implicit or explicit MTS must be enabled for intra prediction residuals (sps_mts_enabled_flag==1).

# Proposed spec text change

To separate the HLS controlling of intra (either implicit or explicit) MTS and inter explicit MTS, the following spec text changes are proposed. The added texts are highlighted in green, and deleted texts are marked by red strikethrough.

#### 7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| sps_mts_enabled_flag | u(1) |
| if( sps_mts_enabled_flag ) { |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| } |  |
| sps_intra_mts_selection | ue(v) |
| sps_inter_mts_selection | ue(v) |
| …… |  |
| } |  |

#### 7.4.3.3	Sequence parameter set RBSP semantics

sps_mts_enabled_flag equal to 1 specifies that sps_explicit_mts_intra_enabled_flag is present in the sequence parameter set RBSP syntax and that sps_explicit_mts_inter_enabled_flag is present in the sequence parameter set RBSP syntax. sps_mts_enabled_flag equal to 0 specifies that sps_explicit_mts_intra_enabled_flag is not present in the sequence parameter set RBSP syntax and that sps_explicit_mts_inter_enabled_flag is not present in the sequence parameter set RBSP syntax.

sps_explicit_mts_intra_enabled_flag equal to 1 specifies that tu_mts_idx may be present in the transform unit syntax for intra coding units. sps_explicit_mts_intra_enabled_flag equal to 0 specifies that tu_mts_idx is not present in the transform unit syntax for intra coding units. When not present, the value of sps_explicit_mts_intra_enabled_flag is inferred to be equal to 0.

sps_explicit_mts_inter_enabled_flag equal to 1 specifies that tu_mts_idx may be present in the transform unit syntax for inter coding units. sps_explicit_mts_inter_enabled_flag equal to 0 specifies that tu_mts_idx is not present in the transform unit syntax for inter coding units. When not present, the value of sps_explicit_mts_inter_enabled_flag is inferred to be equal to 0.

sps_intra_mts_selection equal to 0 or 1 specifies that that tu_mts_idx is not present in the transform unit syntax for intra coding units. sps_intra_mts_selection equal to 2 specifies that tu_mts_idx may be present in the transform unit syntax for intra coding units. sps_intra_mts_selection equal to 0 further specifies that implicit MTS is not applied for intra prediction residuals and sps_intra_mts_selection equal to 1 further specifies that implicit MTS is applied for intra prediction residuals.

sps_inter_mts_selection equal to 0 or 1 specifies that tu_mts_idx is not present in the transform unit syntax for inter coding units. sps_inter_mts_selection equal to 2 specifies that tu_mts_idx may be present in the transform unit syntax for inter coding units. sps_inter_mts_selection equal to 0 further specifies that implicit MTS is not applied for inter prediction residuals and sps_inter_mts_selection equal to 1 further specifies that implicit MTS is applied for inter prediciton residuals.

#### 7.3.8.10	Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| …… |  |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize ) |  |
| transform_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag sps_intra_mts_selection = =2 ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag sps_inter_mts_selection = =2 ) ) && ( !transform_skip_flag[ x0 ][ y0 ] ) ) |  |
| tu_mts_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( !transform_skip_flag[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] && !( tu_cbf_cb[ x0 ][ y0 ] && tu_joint_cbcr_residual_flag[ x0 ][ y0 ] )) { |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| } |  |

### 8.7.4	Transformation process for scaled transform coefficients

#### 8.7.4.1	General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

……

The variable implicitMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitMtsEnabled is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and sps_intra_mts_selection is not equal to 0

- cu_sbt_flag is equal to 1 and Max( nTbW, nTbH ) is less than or equal to 32 and sps_inter_mts_selection is not equal to 0

- sps_explicit_mts_intra_enabled_flag sps_intra_mts_selection is equal to 01 and CuPredMode[ 0 ][ xTbY ][ yTbY ] is equal to MODE_INTRA and lfnst_idx[ x0 ][ y0 ] is equal to 0 and intra_mip_flag[ x0 ][ y0 ] is equal to 0

- Otherwise, implicitMtsEnabled is set equal to 0.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived as follows:

- If cIdx is greater than 0, trTypeHor and trTypeVer are set equal to 0.

- Otherwise, if implicitMtsEnabled is equal to 1, the following applies:

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT or sps_explicit_mts_intra_enabled_flag is equal to 0 and CuPredMode[ 0 ][ xTbY ][ yTbY ] is equal to MODE_INTRA, trTypeHor and trTypeVer are derived as follows:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? 1 : 0	(8-975)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? 1 : 0	(8-976)

- Otherwise (cu_sbt_flag is equal to 1), trTypeHor and trTypeVer are specified in Table 8-15 depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag.

- Otherwise, trTypeHor and trTypeVer are specified in Table 8-14 depending on tu_mts_idx[ xTbY ][ yTbY ].

# Conclusions

In this contribution, modifications on MTS related HLS are proposed. With the proposed method, inter explicit MTS and intra MTS (either implicit or explicit) control flags are decoupled and they can be controlled separately. It is recommended to adopt the proposed changes into next version of VTM software and VVC spec text.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).