| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0507 |
| --- | --- |

| Title: | Non-CE3: ISP simplifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xiang Li, Xin Zhao, Shan Liu | Tel: Email: | {leolzhao, xlxiangli, xinzzhao, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In VVC Draft 6, two partitioning methods are employed, which are CU partitioning and ISP. It is reported that they may lead to same partitions, which introduces redundancy. In this contribution, two methods are proposed. The first method only allows ISP mode for a block when current multi-type tree depth plus 2 is larger than the maximum allowed multi-type tree depth, or width or height of current CU is smaller than 16. The second method allows ISP mode only when width or height of current CU is smaller than 16, wherein ISP will generate sub-partitions with width or height smaller than 4. It is reported that the first method achieves 0.02% and 0.01% luma BD-rate changes with 98% and 100% encoding time for AI and RA configurations, respectively, and the second method achieves 0.04% and 0.03% luma BD-rate changes with 98% and 100% encoding time for AI and RA configurations, respectively.

Introduction

The Intra Sub-Partitions (ISP) coding mode divides luma intra-predicted blocks vertically or horizontally into 2 or 4 sub-partitions depending on the block size dimensions, as shown in Table 1-1. Figure 1-1 and Figure 1-2 show examples of the two possibilities. All sub-partitions at least have 16 luma samples. For chroma components, ISP is not applied.

At most, two bins are signaled for ISP. The first bin indicates whether ISP is used. If ISP is used, another bin is further signaled to indicate the direction of ISP.

For each of these sub-partitions, residual signal is generated by entropy decoding the coefficients sent by the encoder and then inverse quantizing and inverse transforming them. Then, the sub-partition is intra predicted and finally the corresponding reconstructed samples are obtained by adding the residual to the prediction. Therefore, the reconstructed values of each sub-partition will be available to generate the prediction of the next one. All sub-partitions share the same intra mode.

Table 1-1: Number of sub-partitions depending on the block size

| Block Size | Number of Sub-Partitions |
| --- | --- |
| 4×4 | Not divided |
| 4×8 and 8×4 | 2 |
| All other cases | 4 |

Figure 1-1: Example of division of 4×8 and 8×4 blocks

Figure 1-2: Example of division of all blocks except 4×8, 8×4 and 4x4 blocks

Proposed methods

Method #1

It is observed that ISP and two-level BT split may lead to same partitioning, which is redundant and increases encoding complexity. Therefore, it is proposed to only allow ISP mode for a block when current multi-type tree depth plus 2 is larger than the maximum allowed multi-type tree depth, or width or height of current CU is smaller than 16.

Method #2

It is proposed to allow ISP mode only when width or height of current CU is smaller than 16, wherein ISP will generate sub-partitions with width or height smaller than 4 and these sub-partitions can’t be achieved by CU partitioning.

Experimental results

The proposed method in this contribution is implemented on top of VTM6.0 software. The simulations are performed on Linux cluster using AI and RA configurations under the common test condition described in JVET-N1010 [1]. Two tests are conducted to evaluate the performance of the proposed methods.

Table 3-1: Simulation results of method #1 under CTC.

|  | All Intra Main 10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.13% | -0.27% | 98% | 101% | 0.01% | 0.03% | 0.03% | 100% | 100% |
| Class A2 | 0.04% | 0.01% | 0.03% | 99% | 100% | 0.02% | 0.07% | 0.14% | 100% | 100% |
| Class B | 0.04% | 0.03% | -0.07% | 98% | 101% | 0.01% | 0.15% | 0.03% | 100% | 100% |
| Class C | 0.00% | -0.08% | 0.16% | 99% | 99% | 0.01% | 0.06% | 0.02% | 100% | 100% |
| Class E | 0.02% | -0.07% | 0.01% | 98% | 100% |  |  |  |  |  |
| Overall | 0.02% | -0.04% | -0.02% | 98% | 100% | 0.01% | 0.09% | 0.05% | 100% | 100% |
| Class D | -0.01% | 0.00% | -0.17% | 99% | 100% | -0.09% | -0.20% | -0.16% | 100% | 100% |
| Class F | 0.03% | -0.08% | -0.07% | 100% | 100% | 0.00% | -0.01% | 0.02% | 100% | 100% |

Table 3-2: Simulation results of method #2 under CTC.

|  | All Intra Main 10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.14% | -0.22% | 97% | 99% | 0.03% | 0.04% | 0.06% | 99% | 100% |
| Class A2 | 0.06% | -0.02% | -0.01% | 98% | 100% | 0.06% | 0.07% | -0.02% | 99% | 100% |
| Class B | 0.05% | 0.01% | -0.02% | 99% | 100% | 0.01% | 0.12% | 0.00% | 100% | 100% |
| Class C | 0.03% | -0.08% | 0.11% | 98% | 101% | 0.03% | 0.12% | 0.01% | 100% | 102% |
| Class E | 0.04% | 0.02% | 0.03% | 98% | 100% |  |  |  |  |  |
| Overall | 0.04% | -0.04% | -0.02% | 98% | 100% | 0.03% | 0.09% | 0.01% | 100% | 100% |
| Class D | 0.02% | -0.04% | -0.17% | 98% | 102% | -0.02% | -0.21% | -0.16% | 99% | 101% |
| Class F | 0.04% | -0.07% | -0.08% | 98% | 98% | 0.01% | -0.02% | -0.01% | 99% | 100% |

Spec text

The following proposed text modifications of the proposed method are based on JVET-O2001-vE. The added texts are highlighted in green, and deleted texts are marked by red strikethrough.

Spec text of method #1

#### 9.3.4.2	Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, mttDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA? 1 : 0 |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ¦ ¦ sps_palette_enabled_flag) { |  |
| if( treeType != DUAL_TREE_CHROMA && !( ( ( cbWidth = = 4 && cbHeight = = 4 ) ¦ ¦ modeType = = MODE_TYPE_INTRA ) && !sps_ibc_enabled_flag ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && modeType = = MODE_TYPE_ALL ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( slice_type != I && ( CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 && cu_skip_flag[ x0 ][ y0 ] = = 0 ) ) ) ) && cbWidth <= 64 && cbHeight <= 64 && modeType != MODE_TYPE_INTER && sps_ibc_enabled_flag && treeType != DUAL_TREE_CHROMA ) |  |
| pred_mode_ibc_flag | ae(v) |
| if( ( ( ( slice_type = = I ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 ) ¦ ¦ sps_ibc_enabled_flag ) && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) ¦ ¦ ( slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && !sps_ibc_enabled_flag && CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) ) && sps_palette_enabled_flag && cbWidth <= 64 && cbHeight <= 64 && && cu_skip_flag[ x0 ][ y0 ] = = 0 && modeType != MODE_INTER ) |  |
| pred_mode_plt_flag | ae(v) |
| } |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_PLT ) { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( pred_mode_plt_flag ) { |  |
| if( treeType = = DUAL_TREE_LUMA ) |  |
| palette_coding( x0, y0, cbWidth, cbHeight, 0, 1 ) |  |
| else /* SINGLE_TREE */ |  |
| palette_coding( x0, y0, cbWidth, cbHeight, 0, 3 ) |  |
| } else { |  |
| if( sps_bdpcm_enabled_flag && cbWidth <= MaxTsSize && cbHeight <= MaxTsSize ) |  |
| intra_bdpcm_flag | ae(v) |
| if( intra_bdpcm_flag ) |  |
| intra_bdpcm_dir_flag | ae(v) |
| else { |  |
| if( sps_mip_enabled_flag && ( Abs( Log2( cbWidth ) − Log2( cbHeight ) ) <= 2 ) && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_mip_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_mip_flag[ x0 ][ y0 ] ) |  |
| intra_mip_mode[ x0 ][ y0 ] | ae(v) |
| else { |  |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) && (mttDepth + 2 > MaxMttDepthY ¦ ¦ cbWidth < 16 ¦ ¦ cbHeight < 16)) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Spec text of method #2

#### 9.3.4.2	Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA? 1 : 0 |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ¦ ¦ sps_palette_enabled_flag) { |  |
| if( treeType != DUAL_TREE_CHROMA && !( ( ( cbWidth = = 4 && cbHeight = = 4 ) ¦ ¦ modeType = = MODE_TYPE_INTRA ) && !sps_ibc_enabled_flag ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && modeType = = MODE_TYPE_ALL ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( slice_type != I && ( CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 && cu_skip_flag[ x0 ][ y0 ] = = 0 ) ) ) ) && cbWidth <= 64 && cbHeight <= 64 && modeType != MODE_TYPE_INTER && sps_ibc_enabled_flag && treeType != DUAL_TREE_CHROMA ) |  |
| pred_mode_ibc_flag | ae(v) |
| if( ( ( ( slice_type = = I ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 ) ¦ ¦ sps_ibc_enabled_flag ) && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) ¦ ¦ ( slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && !sps_ibc_enabled_flag && CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) ) && sps_palette_enabled_flag && cbWidth <= 64 && cbHeight <= 64 && && cu_skip_flag[ x0 ][ y0 ] = = 0 && modeType != MODE_INTER ) |  |
| pred_mode_plt_flag | ae(v) |
| } |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_PLT ) { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( pred_mode_plt_flag ) { |  |
| if( treeType = = DUAL_TREE_LUMA ) |  |
| palette_coding( x0, y0, cbWidth, cbHeight, 0, 1 ) |  |
| else /* SINGLE_TREE */ |  |
| palette_coding( x0, y0, cbWidth, cbHeight, 0, 3 ) |  |
| } else { |  |
| if( sps_bdpcm_enabled_flag && cbWidth <= MaxTsSize && cbHeight <= MaxTsSize ) |  |
| intra_bdpcm_flag | ae(v) |
| if( intra_bdpcm_flag ) |  |
| intra_bdpcm_dir_flag | ae(v) |
| else { |  |
| if( sps_mip_enabled_flag && ( Abs( Log2( cbWidth ) − Log2( cbHeight ) ) <= 2 ) && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_mip_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_mip_flag[ x0 ][ y0 ] ) |  |
| intra_mip_mode[ x0 ][ y0 ] | ae(v) |
| else { |  |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) && (cbWidth < 16 ¦ ¦ cbHeight < 16)) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Conclusion

This contribution proposed two methods to simplify ISP by removing the redundancy between CU partitioning and ISP. It is reported that method #1 achieves 0.02% and 0.01% luma BD-rate changes with 98% and 100% encoding time for AI and RA configurations, and method #2 achieves 0.04% and 0.03% luma BD-rate changes with 98% and 100% encoding time for AI and RA configurations, respectively. It is suggested to adopt the proposed method.

References

- F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).