| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0534 |
| --- | --- |

| Title: | AHG16/Non-CE5: On deblocking at ALF virtual boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng, Li Zhang, Kai Zhang, Hongbin Liu, Hsiao Chiang Chuang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

Due to the adoption of deblocking filter at 4×4 luma grid, deblocking may be applied on a horizontal edge coinciding with a ALF virtual boundary (VB). Therefore, samples below the ALF VB shall be filtered together with those above the ALF VB. In this case, the previous pipeline processing is asserted to be influenced and extra computations for samples below the ALF VB would be necessary. In this contribution, it is proposed to disable the luma deblocking on horizontal edges coinciding with the ALF virtual boundaries. The proposed modification was tested under the JVET CE5 test conditions, and the overall BD-rate changes for {Y, Cb, Cr} are listed as follows.

| Anchor 1 (CTC) | AI | 0.00% | 0.00% | 0.00% |
| --- | --- | --- | --- | --- |
|  | RA | 0.01% | 0.07% | 0.02% |
|  | LDB | 0.00% | 0.24% | 0.01% |
|  | LDP | 0.03% | -0.07% | 0.30% |
| Anchor 2 (ALF off) | AI | 0.00% | 0.00% | 0.00% |
|  | RA | 0.01% | -0.02% | -0.04% |
|  | LDB | 0.02% | -0.04% | 0.04% |
|  | LDP | -0.01% | -0.01% | 0.05% |

# Problem statement

In practical real-time products, the decoding process can be done CTU by CTU in a raster scanning order in a CTU-pipelined way. Therefore, line buffers are required for CTU-based in-loop filtering because processing one CTU row requires pixels from the above CTU row. In VVC [1], thanks to the concept of ALF VB, the line buffer requirement in VVC is kept unchanged compared to that in HEVC. The ALF VB is defined as a line by shifting the horizontal CTU boundary with 4 samples for the luma component and 2 for the chroma component. Samples above the ALF VB are filtered together before the lower CTU comes, and the samples below the ALF VB cannot be filtered until the lower CTU comes. For example, the first 128×124 samples are cascaded filtered by deblocking, SAO and ALF, before the cascaded in-loop filtering of the lower 128×128 samples. However, due to recent adoption of 4×4 grid luma deblocking filter, the deblocking of samples below the ALF VB have to be handled along with the samples above the ALF VB, which violates the original purpose of ALF VB.

Figure 1. Illustration of CTU-based in-loop filtering when a deblocking edge coincides with the ALF VB

Figure 1 illustrates the CTU-based deblocking, SAO and ALF processing when a deblocking edge coincides with the ALF VB.

Before the adoption of 4×4 grid luma deblocking, samples in line K ~ line N could be deblocking filtered, along vertical boundaries then along horizontal boundaries, together with samples in CTU1 (such as sample in line O ~ line Q) since there is no dependency between samples at the two sides of the ALF VB in the deblocking stage.

The situation is changed after the adoption of 4×4 grid luma deblocking. When deblocking filtering is applied on samples in line K along the horizontal boundary (ALF VB in the figure), samples in line H ~ line M after deblocking filtering along vertical boundaries are required. Since samples above the ALF VB are inaccessible when the samples below the ALF VB are processed, the deblocking process along the horizontal boundary on samples in line K must be done when samples above ALF VB ( such as samples in line A ~ line J) are processed. Besides, the deblocking process along vertical boundaries on samples in line K ~ line M also must be done when samples above ALF VB are processed, because those filtered samples are required to do the deblocking filtering along the horizontal boundary on samples in line K.

Therefore, in VVC WD6, samples below the ALF VB may be deblocking filtered together with the samples above the ALF VB (i.e., deblocking on lines K ~ line M need to be done along with filtering on lines A ~ line J), which violates the original intention of ALF VB that the samples below an ALF VB should be processed together with the lower CTU.

# Proposed solution

In this contribution, it is proposed to disable the luma deblocking on a horizontal edge coinciding with an ALF virtual boundary. However, the 4×4 grid luma deblocking on other boundaries are kept as the same with VTM-6.0. The proposed method meets the requirement of ALF VB that the samples below the ALF VB cannot be filtered until the lower CTU comes. And it might not be critical in visual quality to just turn off the 4×4 grid luma deblocking on a small portion of horizontal boundaries.

# Simulation results

The proposed method was implemented on top of VTM-6.0 [2] and tested under JVET CE5 conditions [3] with either ALF on or ALF off. Simulation results reportedly are compared with Anchor 1 (VTM-6.0 CTC) and Anchor 2 (VTM-6.0 with ALF switched off) and provided in Table 1 (CTC) and Table 2 (ALF off), respectively.

Table 1: BD-Rate results for CTC (anchor 1, ALF on)

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | -0.01% | 0.00% | 100% | 99% |
| Class B | -0.01% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.01% | 0.01% | 100% | 99% |
| Class F | -0.01% | 0.00% | 0.01% | 100% | 101% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | -0.01% | 100% | 99% |
| Class A2 | 0.03% | 0.11% | 0.04% | 99% | 98% |
| Class B | 0.00% | 0.10% | -0.03% | 100% | 101% |
| Class C | 0.01% | 0.05% | 0.08% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.07% | 0.02% | 100% | 100% |
| Class D | -0.03% | -0.05% | -0.03% | 101% | 101% |
| Class F | -0.03% | 0.01% | 0.00% | 101% | 101% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.09% | -0.04% | 100% | 99% |
| Class C | -0.05% | -0.04% | -0.14% | 101% | 101% |
| Class E | 0.01% | 0.85% | 0.28% | 101% | 100% |
| Overall | 0.00% | 0.24% | 0.01% | 101% | 100% |
| Class D | -0.02% | 0.05% | 0.29% | 101% | 101% |
| Class F | 0.01% | 0.04% | 0.62% | 100% | 100% |
|  |  |  | Low delay P |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.04% | 0.26% | 100% | 100% |
| Class C | 0.04% | -0.03% | 0.20% | 100% | 101% |
| Class E | 0.08% | -0.31% | 0.49% | 100% | 101% |
| Overall | 0.03% | -0.07% | 0.30% | 100% | 100% |
| Class D | 0.04% | -0.12% | 0.08% | 99% | 99% |
| Class F | -0.03% | -0.10% | 0.30% | 100% | 100% |

Table 2: BD-Rate results for ALF off (anchor 2, ALF off)

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | -0.01% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.05% | 100% | 101% |
| Class A2 | 0.03% | 0.01% | 0.04% | 100% | 99% |
| Class B | 0.02% | -0.05% | -0.08% | 100% | 100% |
| Class C | 0.00% | -0.02% | -0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.02% | -0.04% | 100% | 100% |
| Class D | -0.02% | 0.13% | -0.03% | 100% | 101% |
| Class F | 0.00% | -0.04% | 0.04% | 99% | 100% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.06% | -0.06% | 100% | 100% |
| Class C | 0.02% | -0.01% | 0.25% | 99% | 99% |
| Class E | 0.03% | -0.23% | -0.06% | 100% | 102% |
| Overall | 0.02% | -0.04% | 0.04% | 100% | 100% |
| Class D | 0.01% | 0.15% | -0.21% | 98% | 99% |
| Class F | -0.02% | 0.10% | -0.15% | 99% | 98% |
|  |  |  | Low delay P |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.02% | 0.09% | 100% | 101% |
| Class C | -0.04% | 0.07% | -0.05% | 99% | 98% |
| Class E | 0.03% | -0.11% | 0.11% | 99% | 97% |
| Overall | -0.01% | -0.01% | 0.05% | 100% | 99% |
| Class D | -0.05% | 0.00% | 0.12% | 100% | 100% |
| Class F | 0.03% | 0.30% | -0.31% | 100% | 102% |

# Conclusions

This contribution proposes to disable the luma deblocking on horizontal boundary coinciding with the ALF virtual boundary, in order to align the CTU-pipelined processing for deblocking, SAO and ALF. It is recommended adopting the proposed method.

# Proposed spec changes

The proposed changes on top of VVC draft 6 [1] are described as below.

#### 8.8.3.3. Derivation process of transform block boundary

Inputs to this process are:

- a location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a variable nCbW specifying the width of the current coding block,

- a variable nCbH specifying the height of the current coding block,

- a variable cIdx specifying the colour component of the current coding block,

- a variable filterEdgeFlag,

- a two-dimensional (nCbW)x(nCbH) array edgeFlags,

- two-dimensional (nCbW)x(nCbH) arrays maxFilterLengthQs and maxFilterLengthPs,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered.

Outputs of this process are:

- the modified two-dimensional (nCbW)x(nCbH) array edgeFlags,

- the modified two-dimensional (nCbW)x(nCbH) arrays maxFilterLengthQs, maxFilterLengthPs.

Depending on edgeType, the arrays edgeFlags, maxFilterLengthPs and maxFilterLengthQs are derived as follows:

- The variable gridSize is set as follows:

gridSize = cIdx  = =  0 ? 4 : 8		(8-1024)

- If edgeType is equal to EDGE_VER, the following applies:

- The variable numEdges is set equal to Max( 1, nCbW / gridSize ).

- For xEdge = 0..numEdges − 1 and y = 0..nCbH − 1, the following applies:

- The horizontal position x inside the current coding block is set equal to xEdge * gridSize.

- The value of edgeFlags[ x ][ y ] is derived as follows:

    - If pps_loop_filter_across_virtual_boundaries_disabled_flag equal to 1 and ( xCb + x ) is equal to PpsVirtualBoundariesPosX[ n ] for any n = 0..pps_num_ver_virtual_boundaries − 1, edgeFlags[ x ][ y ] is set equal to 0.

    - Otherwise, if x is equal to 0, edgeFlags[ x ][ y ] is set equal to filterEdgeFlag.

    - Otherwise, if the location ( xCb + x , yCb + y ) is at a transform block edge, edgeFlags[ x ][ y ] is set equal to 1.

- When edgeFlags[ x ][ y ] is equal to 1,the following applies:

    - If cIdx is equal to 0, the following applies:

    - The value of maxFilterLengthQs[ x ][ y ] is derived as follows:

    - If the width in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or less than 4 or the width in luma samples of the transform block at luma location ( xCb + x − 1, yCb + y ) is equal to or less than 4, maxFilterLengthQs[ x ][ y ] is set equal to 1.

    - Otherwise, if the width in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or greater than 32, maxFilterLengthQs[ x ][ y ] is set equal to 7.

    - Otherwise, maxFilterLengthQs[ x ][ y ] is set equal to 3.

    - The value of maxFilterLengthPs[ x ][ y ] is derived as follows:

    - If the width in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or less than 4 or the width in luma samples of the transform block at luma location ( xCb + x − 1, yCb + y ) is equal to or less than 4 , maxFilterLengthPs[ x ][ y ] is set equal to 1.

    - Otherwise, if the width in luma samples of the transform block at luma location ( xCb + x − 1, yCb + y ) is equal to or greater than 32, maxFilterLengthPs[ x ][ y ] is set equal to 7.

    - Otherwise, maxFilterLengthPs[ x ][ y ] is set equal to 3.

    - Otherwise (cIdx is not equal to 0), the values of maxFilterLengthPs[ x][ y ] and maxFilterLengthQs[ x ][ y ] are derived as follows:

    - If the width in chroma samples of the transform block at chroma location ( xCb + x, yCb + y ) and the width at chroma location ( xCb + x − 1,  yCb + y ) are both equal to or greater than 8, maxFilterLengthPs[ x ][ y ] and maxFilterLengthQs[ x ][ y ] are set equal to 3.

    - Otherwise, maxFilterLengthPs[ x ][ y ] and maxFilterLengthQs[ x ][ y ] are set equal to 1.

- Otherwise (edgeType is equal to EDGE_HOR), the following applies:

- The variable numEdges is set equal to Max( 1, nCbH / gridSize ).

- For yEdge = 0..numEdges − 1 and x = 0..nCbW − 1, the following applies:

- The vertical position y inside the current coding block is set equal to yEdge * gridSize.

- The value of edgeFlags[ x ][ y ] is derived as follows:

    - If pps_loop_filter_across_virtual_boundaries_disabled_flag equal to 1 and ( yCb + y ) is equal to PpsVirtualBoundariesPosY[ n ] for any n = 0..pps_num_hor_virtual_boundaries − 1, edgeFlags[ x ][ y ] is set equal to 0.

    - Otherwise, if cIdx is equal to 0 and (yCb + y ) % CtbSizeY is equal to (CtbSizeY - 4), edgeFlags[ x ][ y ] is set equal to 0.

    - Otherwise, if y is equal to 0, edgeFlags[ x ][ y ] is set equal to filterEdgeFlag.

    - Otherwise, if the location ( xCb + x , yCb + y ) is at a transform block edge, edgeFlags[ x ][ y ] is set equal to 1.

- When edgeFlags[ x ][ y ] is equal to 1,the following applies:

    - If cIdx is equal to 0, the following applies:

    - The value of maxFilterLengthQs[ x ][ y ] is derived as follows:

    - If the height in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or less than 4 or the height in luma samples of the transform block at luma location ( xCb + x, yCb + y − 1 ) is equal to or less than 4, maxFilterLengthQs[ x ][ y ] is set equal to 1.

    - Otherwise, if the height in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or greater than 32, maxFilterLengthQs[ x ][ y ] is set equal to 7.

    - Otherwise, maxFilterLengthQs[ x ][ y ] is set equal to 3.

    - The value of maxFilterLengthPs[ x ][ y ] is derived as follows:

    - If the height in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or less than 4 or the height in luma samples of the transform block at luma location ( xCb + x, yCb + y − 1 ) is equal to or less than 4, maxFilterLengthPs[ x ][ y ] is set equal to 1.

    - Otherwise, if the height in luma samples of the transform block at luma location ( xCb + x, yCb + y − 1 ) is equal to or greater than 32, maxFilterLengthPs[ x ][ y ] is set equal to 7.

    - Otherwise, maxFilterLengthPs[ x ][ y ] is set equal to 3.

    - Otherwise (cIdx is not equal to 0), the values of maxFilterLengthPs[ x][ y ] and maxFilterLengthQs[ x ][ y ] are derived as follows:

    - If all of the following conditions are true, maxFilterLengthPs[ x ][ y ] and maxFilterLengthQs[ x ][ y ] are set equal to 3:

    - The height in chroma samples of the transform block at chroma location ( xCb + x, yCb + y ) and the height at chroma location ( xCb + x,  yCb + y − 1 ) are both equal to or greater than 8.

    - ( yCb + y ) % CtbHeightC is greater than 0, i.e. the horizontal edge do not overlap with the upper chroma CTB boundary.

    - Otherwise, maxFilterLengthPs[ x ][ y ] and maxFilterLengthQs[ x ][ y ] are set equal to 1.

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019, Gothenburg, SE.

VTM-6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0.

C.-Y. Chen, A. Norkin, “Description of Core Experiment 5 (CE5): Loop filtering”, JVET-O2025, July 2019, Gothenburg, SE.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).