| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0540-v1 |
| --- | --- |

| Title: | Non-CE6: Disabling MTS for 64xN and Nx64 CUs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jason Jung Dongcheol Kim Geonjung Ko Ju-Hyung (John) Son Jin Sam Kwak | Tel: Email: | +82-31-712-0523 jason.jung@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

- Abstract

In this contribution, the block size condition for applying MTS (Multiple Transform Selection) is proposed when the maximum transform size is equal to 32. In VVC Draft 6, the maximum transform size is signalled in the SPS (Sequence Parameter Set) and it can be configured to either 32 or 64. When the maximum transform size is equal to 32, 64xN and Nx64 (N>=4) CUs are partitioned into 32xMin(32, N) and Min(32, N)x32 TUs, respectively. For the partitioned TUs, MTS is allowed and MTS index (tu_mts_index) is coded. However, when the maximum transform size is equal to 64, MTS is not allowed to 64xN and Nx64 (N>=4) CUs. This contribution proposes to disallow MTS for 64xN and Nx64 (N>=4) CUs when the maximum transform size is equal to 32. In the proposed method, only DCT-II and transform skip mode are allowed and MTS index is not coded for 64xN and Nx64 (N>=4) CUs. This would harmonize the maximum CU block size that can utilize MTS as 32x32 irrespective of the signaled maximum transform size. It is reportedly shown that the proposed method achieves -0.02% (AI), -0.02% (RA) and -0.05% (LDB) BD-rates with same encoding and decoding times when the maximum transform size is configured to 32 in VTM-6.0. Additionally, when inter MTS is enabled (--MTS=3), the proposed method achieves 0.03% (RA) and -0.xx% (LDB) with 93% and xx% encoding times.

Introduction

In VVC Draft 6 [1], the maximum transform size is signalled in the SPS and it can be 32 or 64. When the maximum transform size is equal to 32, 64xN and Nx64 CUs (N>=4) are implicitly partitioned in the transform tree and MTS can be applied to the partitioned TUs. Figure 1 shows an example of applying MTS to 64x64 CU. For 64x64 CU, MTS is allowed to the implicitly partitioned 32x32 TUs and MTS index (tu_mts_idx) can be signalled for each TU to indicate which transform kernels are applied in horizontal and vertical directions.

Figure 1. An example of applying MTS to 64x64 CU in VVC draft 6

When the maximum transform size is equal to 64, it is noted that MTS is not allowed to 64xN and Nx64 (N>=4) CUs. Therefore, for the same 64xN and Nx64 (N>=4) CUs, MTS is selectively applied according to the maximum transform size signalled.

Proposed Method

In this contribution, it is proposed to disable MTS for 64xN and Nx64 CUs when the maximum transform size is equal to 32. Figure 2 shows an example of coding TUs for 64x64 CU. For the partitioned 32x32 TUs, {DCT-II, DCT-II} and transform skip mode are allowed and transform_skip_flag can be coded. It is noted that MTS index, tu_mts_idx is not coded and it is inferred to be equal to 0.

Figure 2. An example of coding TUs for 64x64 CU in the proposed method

Specification text change

7.3.8.10  Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| ⋯ |  |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize ) |  |
| transform_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) && ( !transform_skip_flag[ x0 ][ y0 ] ) && ( CbWidth[ chType ][ x0 ][ y0 ] <= 32 ) && ( CbHeight[ chType ][ x0 ][ y0 ] <= 32 ) ) |  |
| tu_mts_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| ⋯ |  |
| } |  |

Experimental Results

The proposed method was implemented on top of VTM-6.0 [2] and tested under the common test condition (CTC) [3] with the maximum transform size set to 32. Table 1 and 2 show coding performances of the proposed method with intra MTS enabled and both intra and inter MTS enabled, respectively.

Table 1. Coding performance of the proposed method with intra MTS enabled (MTS=1)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0+MaxTbSizeY32 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |
| Class A1 | -0.04% | -0.06% | -0.05% | 100% | 100% |  |
| Class A2 | -0.01% | -0.09% | -0.02% | 100% | 100% |  |
| Class B | -0.01% | -0.03% | 0.05% | 100% | 100% |  |
| Class C | 0.00% | 0.01% | -0.01% | 100% | 100% |  |
| Class E | -0.03% | -0.08% | -0.20% | 100% | 101% |  |
| Overall | -0.02% | -0.04% | -0.03% | 100% | 100% |  |
| Class D | 0.00% | 0.02% | 0.03% | 99% | 100% |  |
| Class F | 0.02% | -0.01% | -0.03% | 101% | 100% |  |
|  | Random access Main10 |  |  |  |  |  |
|  | Over VTM-6.0+MaxTbSizeY32 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |
| Class A1 | -0.06% | -0.26% | -0.28% | 100% | 100% |  |
| Class A2 | -0.01% | 0.05% | -0.12% | 100% | 100% |  |
| Class B | -0.02% | -0.14% | -0.03% | 100% | 100% |  |
| Class C | 0.01% | 0.02% | 0.00% | 100% | 100% |  |
| Class E |  |  |  |  |  |  |
| Overall | -0.02% | -0.08% | -0.09% | 100% | 100% |  |
| Class D | 0.01% | -0.03% | 0.00% | 100% | 100% |  |
| Class F | -0.03% | 0.04% | -0.22% | 100% | 100% |  |
|  | Low delay B Main10 |  |  |  |  |  |
|  | Over VTM-6.0+MaxTbSizeY32 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.01% | 0.01% | 0.11% | 100% | 100% |  |
| Class C | -0.03% | -0.16% | -0.16% | 100% | 100% |  |
| Class E | -0.15% | -0.23% | -1.17% | 100% | 100% |  |
| Overall | -0.05% | -0.11% | -0.30% | 100% | 100% |  |
| Class D | 0.03% | 0.46% | 0.29% | 100% | 100% |  |
| Class F | -0.07% | -0.29% | 0.51% | 100% | 100% |  |

Table 2. Coding performance of the proposed method with both intra and inter MTS enabled (MTS=3)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0+MaxTbSizeY32+MTS3 |  |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |  |
| Class A1 | 0.02% |  | -0.44% |  | -0.61% | 91% | 100% |  |
| Class A2 | 0.10% |  | -0.12% |  | -0.29% | 92% | 100% |  |
| Class B | 0.03% |  | -0.14% |  | -0.25% | 93% | 100% |  |
| Class C | -0.02% |  | -0.04% |  | -0.05% | 95% | 100% |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.03% |  | -0.17% |  | -0.28% | 93% | 100% |  |
| Class D | -0.02% |  | -0.09% |  | -0.06% | 97% | 100% |  |
| Class F | -0.02% |  | 0.03% |  | -0.18% | 95% | 100% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | Over VTM-6.0+MaxTbSizeY32+MTS3 |  |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | #VALUE! |  | #VALUE! |  | #VALUE! | #DIV/0! | #DIV/0! |  |
| Class C | -0.11% |  | -0.13% |  | -0.04% | 95% | #DIV/0! |  |
| Class E | -0.38% |  | -1.25% |  | -1.40% | 88% | #DIV/0! |  |
| Overall | #VALUE! |  | #VALUE! |  | #VALUE! | #DIV/0! | #DIV/0! |  |
| Class D | -0.06% |  | 0.04% |  | 0.24% | 97% | #DIV/0! |  |
| Class F | #VALUE! |  | #VALUE! |  | #VALUE! | #NUM! | #DIV/0! |  |

Conclusion

In this contribution, it is proposed to disallow MTS for 64xN and Nx64 (N>=4) CUs when the maximum transform size is equal to 32. This would harmonize the maximum CU block size that can utilize MTS as 32x32 irrespective of the signaled maximum transform size. It is shown that the proposed method achieves coding efficiency improvements when the maximum transform size is configured to 32 in VTM-6.0. Additionally, when inter MTS is enabled, the proposed method can significantly reduce encoding time with negligible loss. It is recommended to adopt the proposed method into the next version of draft text and reference software.

References

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, 15th Meeting, Gothenburg, SE, 3–12 July 2019

- VTM-6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-6.0

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th Meeting, Geneva, CH, 19–27 Mar. 2019

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).