| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0542 |
| --- | --- |

| Title: | Non-CE4: Cleanup on alternative half-pel interpolation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Hyeongmun Jang Naeri Park Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {junghak.nam, hm.jang, naeri.park, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the previous meeting, half-pel AMVR was adopted with alternative half-pel interpolation filter. This contribution proposes two cleanups for an index of half-pel interpolation filter. The first aspect is that pair-wise candidate in merge mode do not use index of half-pel sample interpolation filter. The second aspect is that motion compensation for TPM mode uses an index of half-pel sample interpolation filter.

Introduction

In the previous meeting, half-pel adaptive motion vector resolution (AMVR) for normal inter block was extended. In addition, when half-pel AMVR is selected, alternative interpolation filter for half-pel sample was introduced and propagated neighboring blocks coded by normal merge and MMVD blocks except TPM block. In this contribution, clean-up of interpolation filter index for half-pel sample proposes.

Proposed method

Aspect 1: No inherit an index of half sample interpolation filter for pair-wise candidate

In VVC, the half sample interpolation filter index for pair-wise candidate is conditionally derived. The proposed method is to remove the derivation process. The hpelIfIdxavgCand is always set equal to 0.

8.5.2.4 Derivation process for pairwise average merging candidate Inputs to this process are: - a merging candidate list mergeCandList, - the reference indices refIdxL0N and refIdxL1N of every candidate N in mergeCandList, - the prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N in mergeCandList, - the motion vectors in 1/16 fractional-sample accuracy mvL0N and mvL1N of every candidate N in mergeCandList, - the half sample interpolation filter index hpelIfIdxN of every candidate N in mergeCandList, - the number of elements numCurrMergeCand within mergeCandList. Outputs of this process are: - the merging candidate list mergeCandList, - the number of elements numCurrMergeCand within mergeCandList, - the reference indices refIdxL0avgCand and refIdxL1avgCand of candidate avgCand added into mergeCandList during the invocation of this process, - the prediction list utilization flags predFlagL0avgCand and predFlagL1avgCand of candidate avgCand added into mergeCandList during the invocation of this process, - the motion vectors in 1/16 fractional-sample accuracy mvL0avgCand and mvL1avgCand of candidate avgCand added into mergeCandList during the invocation of this process, - the half sample interpolation filter index hpelIfIdxavgCand of every candidate avgCand added into mergeCandList during the invocation of this process. The variable numRefLists is derived as follows: numRefLists = ( slice_type = = B ) ? 2 : 1 (8-344) The following assignments are made, with p0Cand being the candidate at position 0 and p1Cand being the candidate at position 1 in the merging candidate list mergeCandList: p0Cand = mergeCandList[ 0 ] (8-345) p1Cand = mergeCandList[ 1 ] (8-346) The candidate avgCand is added at the end of mergeCandList, i.e., mergeCandList[ numCurrMergeCand ] is set equal to avgCand, and the reference indices, the prediction list utilization flags and the motion vectors of avgCand are derived as follows and numCurrMergeCand is incremented by 1: - For each reference picture list LX with X ranging from 0 to ( numRefLists − 1 ), the following applies: - If predFlagLXp0Cand is equal to 1 and predFlagLXp1Cand is equal to 1, the variables refIdxLXavgCand, predFlagLXavgCand, mvLXavgCand[ 0 ], and mvLXavgCand[ 1 ] are derived as follows: refIdxLXavgCand = refIdxLXp0Cand (8-347) predFlagLXavgCand = 1 (8-348) - The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to mvLXp0Cand[ 0 ] + mvLXp1Cand[ 0 ], rightShift set equal to1, and leftShift set equal to 0 as inputs and the rounded mvLXavgCand[ 0 ] as output. - The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to mvLXp0Cand[ 1 ] + mvLXp1Cand[ 1 ], rightShift set equal to1, and leftShift set equal to 0 as inputs and the rounded mvLXavgCand[ 1 ] as output. - Otherwise, if predFlagLXp0Cand is equal to 1 and predFlagLXp1Cand is equal to 0, the variables refIdxLXavgCand, predFlagLXavgCand, mvLXavgCand[ 0 ], mvLXavgCand[ 1 ] are derived as follows: refIdxLXavgCand = refIdxLXp0Cand (8-349) predFlagLXavgCand = 1 (8-350) mvLXavgCand[ 0 ] = mvLXp0Cand[ 0 ] (8-351) mvLXavgCand[ 1 ] = mvLXp0Cand[ 1 ] (8-352) - Otherwise, if predFlagLXp0Cand is equal to 0 and predFlagLXp1Cand is equal to 1, the variables refIdxLXavgCand, predFlagLXavgCand, mvLXavgCand[ 0 ], mvLXavgCand[ 1 ] are derived as follows: refIdxLXavgCand = refIdxLXp1Cand (8-353) predFlagLXavgCand = 1 (8-354) mvLXavgCand[ 0 ] = mvLXp1Cand[ 0 ] (8-355) mvLXavgCand[ 1 ] = mvLXp1Cand[ 1 ] (8-356) - Otherwise, if predFlagLXp0Cand is equal to 0 and predFlagLXp1Cand is equal to 0, the variables refIdxLXavgCand, predFlagLXavgCand, mvLXavgCand[ 0 ], mvLXavgCand[ 1 ] are derived as follows: refIdxLXavgCand = −1 (8-357) predFlagLXavgCand = 0 (8-358) mvLXavgCand[ 0 ] = 0 (8-359) mvLXavgCand[ 1 ] = 0 (8-360) - When numRefLists is equal to 1, the following applies: refIdxL1avgCand = −1 (8-361) predFlagL1avgCand = 0 (8-362) - The half sample interpolation filter index hpelIfIdxavgCand is derived as follows: - If hpelIfIdxp0Cand is equal to hpelIfIdxp1Cand, hpelIfIdxavgCand is set equal to hpelIfIdxp0Cand. - Otherwise, hpelIfIdxavgCand is set equal to 0.

Aspect 2: Inherit an index of half sample interpolation filter for TPM

In VVC, the half sample interpolation filter index for TPM is always set equal to 0. The proposed method uses the half sample interpolation filter index when motion compensation is conducted for each partitioning. However, the half sample interpolation filter index is not propagated to next blocks.

8.5.4 Derivation process for triangle motion vector components and reference indices 8.5.4.1 General Inputs to this process are: - a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture, - a variable cbWidth specifying the width of the current coding block in luma samples, - a variable cbHeight specifying the height of the current coding block in luma samples. Outputs of this process are: - the luma motion vectors in 1/16 fractional-sample accuracy mvA and mvB, - the chroma motion vectors in 1/32 fractional-sample accuracy mvCA and mvCB, - the reference indices refIdxA and refIdxB, - the half sample interpolation filter index hpelIfIdxA and hpelIfIdxB, - the prediction list flags predListFlagA and predListFlagB. The derivation process for luma motion vectors for triangle merge mode as specified in clause 8.5.4.2 is invoked with the luma location ( xCb, yCb ), the variables cbWidth and cbHeight as inputs, and the output being the luma motion vectors mvA, mvB, the reference indices refIdxA, refIdxB, the half sample interpolation filter index hpelIfIdxA, hpelIfIdxB and the prediction list flags predListFlagA and predListFlagB. The derivation process for chroma motion vectors in clause 8.5.2.13 is invoked with mvA and refIdxA as input, and the output being mvCA. The derivation process for chroma motion vectors in clause 8.5.2.13 is invoked with mvB and refIdxB as input, and the output being mvCB. 8.5.4.2 Derivation process for luma motion vectors for merge triangle mode This process is only invoked when MergeTriangleFlag[ xCb ][ yCb ] is equal to 1, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture. Inputs to this process are: - a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture, - a variable cbWidth specifying the width of the current coding block in luma samples, - a variable cbHeight specifying the height of the current coding block in luma samples. Outputs of this process are: - the luma motion vectors in 1/16 fractional-sample accuracy mvA and mvB, - the reference indices refIdxA and refIdxB, - the half sample interpolation filter index hpelIfIdxA and hpelIfIdxB, - the prediction list flags predListFlagA and predListFlagB. The motion vectors mvA and mvB, the reference indices refIdxA and refIdxB and the prediction list flags predListFlagA and predListFlagB are derived by the following ordered steps: - The derivation process for luma motion vectors for merge mode as specified in clause 8.5.2.2 is invoked with the luma location ( xCb, yCb ), the variables cbWidth and cbHeight inputs, and the output being the luma motion vectors mvL0[ 0 ][ 0 ], mvL1[ 0 ][ 0 ], the reference indices refIdxL0, refIdxL1, the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], the bi-prediction weight index bcwIdx and the merging candidate list mergeCandList. - The variables m and n, being the merge index for triangle partition 0 and 1 resepctively, are derived using merge_triangle_idx0[ xCb ][ yCb ] and merge_triangle_idx1[ xCb ][ yCb ] as follows: m = merge_triangle_idx0[ xCb ][ yCb ] (8-475) n = merge_triangle_idx1[ xCb ][ yCb ] + ( merge_triangle_idx1[ xCb ][ yCb ] >= m ) ? 1 : 0 (8-476) - Let refIdxL0M and refIdxL1M, predFlagL0M and predFlagL1M, and mvL0M and mvL1M be the reference indices, the prediction list utilization flags and the motion vectors of the merging candidate M at position m in the merging candidate list mergeCandList ( M = mergeCandList[ m ] ). - The variable X is set equal to ( m & 0x01 ). - When predFlagLXM is equal to 0, X is set equal to ( 1 − X ). - The following applies: mvA[ 0 ] = mvLXM[ 0 ] (8-477) mvA[ 1 ] = mvLXM[ 1 ] (8-478) refIdxA = refIdxLXM (8-479) hpelIfIdxA = hpelIfIdxM (8-xxx) predListFlagA = X (8-480) - Let refIdxL0N and refIdxL1N, predFlagL0N and predFlagL1N, and mvL0N and mvL1N be the reference indices, the prediction list utilization flags and the motion vectors of the merging candidate N at position m in the merging candidate list mergeCandList ( N = mergeCandList[ n ] ). - The variable X is set equal to ( n & 0x01 ). - When predFlagLXN is equal to 0, X is set equal to ( 1 − X ). - The following applies: mvB[ 0 ] = mvLXN[ 0 ] (8-481) mvB[ 1 ] = mvLXN[ 1 ] (8-482) refIdxB = refIdxLXN (8-483) hpelIfIdxB = hpelIfIdxN (8-xxx) predListFlagB = X (8-484)

Experimental results

The proposed methods have been implemented on VTM-6.0 and tested under common test conditions.

Table 1. Performance of aspect 1 in RA

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.10% | 0.00% | 102% | 102% |
| Class A2 | 0.02% | -0.01% | 0.00% | 103% | 102% |
| Class B | -0.01% | 0.07% | -0.07% | 100% | 101% |
| Class C | -0.04% | 0.06% | -0.03% | 97% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.02% | -0.03% | 100% | 100% |
| Class D | -0.07% | -0.07% | -0.10% | 96% | 93% |
| Class F | -0.02% | -0.04% | -0.01% | 98% | 99% |

Table 2. Performance of aspect 2 in RA

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.14% | -0.04% | 100% | 99% |
| Class A2 | 0.03% | 0.08% | 0.06% | 99% | 99% |
| Class B | -0.01% | 0.07% | -0.01% | 100% | 100% |
| Class C | -0.01% | 0.00% | -0.01% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.07% | 0.00% | 99% | 99% |
| Class D | 0.00% | 0.10% | 0.10% | 96% | 95% |
| Class F | 0.00% | -0.01% | 0.00% | 97% | 98% |

Conclusion

In this contribution, two modifications for half-pel sample interpolation filter are proposed. One is to disable half-pel interpolation for pair-wise candidate and the other is to allow for TPM mode. It is recommended to adopt the proposed method in the next specification.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).