| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0547-v2 |
| --- | --- |

| Title: | Non-CE5: Deblocking filter at PROF sub-block boundary |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyohei Unno Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-70-4560-2752 ky-unno@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

In this contribution, it is proposed that boundary strength calculation for deblocking filtering at PROF (Prediction Refinement with Optical-Flow) sub-block boundary. In the proposed method, differences between P side and Q side are treated as zero in the boundary strength calculation at PROF sub-block boundary. Experimental results show that BD-rates for luma are -0.02%/0.00%/0.04% (RA/LDB/LDP) in comparison with VTM-6.0. And BD-rates for luma are -0.01%/-0.02%/0.00% (RA/LDB/LDP) in comparison with VTM-6.0 without ALF.

Introduction

Prediction Refinement with Optical-Flow (PROF) was adopted to VVC D6 in the 15th meeting at Gothenburg on July 2019 [1]. PROF is an enhancement tool of prediction accuracy considering rotation for affine prediction blocks. On the other hand, motion vector differences are considered in the boundary strength calculation for deblocking filter. At PROF sub-block boundary, motion vectors both P side and Q side do not indicate actual motion for the boundary strength calculation since PROF considers rotation as mentioned above. Additionally, it is clamed that PROF improves both objective and subjective quality.

Proposed Method

To prevent over smoothing at PROF sub-block boundary, it is proposed that motion vector differences between P side and Q side are treated as zero. More precisely, if PROF is applied for list X and X being 0 or 1, a difference between list X motion vector for P side and list X motion vector for Q side is set equal to 0. On the other hand, a difference between list X motion vector for P side and list Y (not equal to X) motion vector for Q side is calculated as same as VVC D6. The proposed text changes in the boundary strength calculation are as follows.

    - Otherwise, if cIdx is equal to 0 and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

      - The coding subblock containing the sample p0 and the coding subblock containing the sample q0 are both coded in IBC prediction mode, and the absolute difference between the horizontal or vertical component of the block vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

      - For the prediction of the coding subblock containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the coding subblock containing the sample q0.

NOTE 1 – The determination of whether the reference pictures used for the two coding sublocks are the same or different is based only on which pictures are referenced, without regard to whether a prediction is formed using an index into reference picture list 0 or an index into reference picture list 1, and also without regard to whether the index position within a reference picture list is different.

NOTE 2 – The number of motion vectors that are used for the prediction of a coding subblock with top-left sample covering ( xSb, ySb ), is equal to PredFlagL0[ xSb ][ ySb ] + PredFlagL1[ xSb ][ ySb ].

      - One motion vector is used to predict the coding subblock containing the sample p0 and one motion vector is used to predict the coding subblock containing the sample q0, and the absolute difference between the horizontal or vertical component of the motion vectors used is greater than or equal to 8 in units of 1/16 luma samples. If the current boundary is not a coding block boundary (edgeType is equal to EDGE_VER and xDi is not equal to 0 or nCbW, or edgeType is equal to EDGE_ HOR and yDi is not equal to 0 or nCbH), and cbProfFlagL0 is equal to 1, the absolute difference between the horizontal or vertical component of the motion vectors used is set equal to 0.

      - Two motion vectors and two different reference pictures are used to predict the coding subblock containing the sample p0, two motion vectors for the same two reference pictures are used to predict the coding subblock containing the sample q0 and the absolute difference between the horizontal or vertical component of the two motion vectors used in the prediction of the two coding subblocks for the same reference picture is greater than or equal to 8 in units of 1/16 luma samples. If the current boundary is not a coding block boundary (edgeType is equal to EDGE_VER and xDi is not equal to 0 or nCbW, or edgeType is equal to EDGE_ HOR and yDi is not equal to 0 or nCbH), and cbProfFlagLX is equal to 1, the absolute difference between the horizontal or vertical component of the list X motion vectors used is set equal to 0.

      - Two motion vectors for the same reference picture are used to predict the coding subblock containing the sample p0, two motion vectors for the same reference picture are used to predict the coding subblock containing the sample q0 and both of the following conditions are true:

        - The absolute difference between the horizontal or vertical component of list 0 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples. If the current boundary is not a coding block boundary (edgeType is equal to EDGE_VER and xDi is not equal to 0 or nCbW, or edgeType is equal to EDGE_ HOR and yDi is not equal to 0 or nCbH), and cbProfFlagLX is equal to 1, the absolute difference between the horizontal or vertical component of the list X motion vectors used is set equal to 0.

        - The absolute difference between the horizontal or vertical component of list 0 motion vector used in the prediction of the coding subblock containing the sample p0 and the list 1 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vector used in the prediction of the coding subblock containing the sample p0 and list 0 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples.

    - Otherwise, the variable bS[ xDi ][ yDj ] is set equal to 0.

Experimental Results

The proposed method is implemented on top of VTM-6.0. In order to verify the performance of the proposed method, we have run simulations according to the common test conditions of JVET-N1010 [2].

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Test results

The proposed method doesn’t have any effect to the results for the AI condition. Therefore, tests are conducted under the RA/LDB/LDP condition with ALF ON/OFF respectively.

From Table 1 to Table 3 show the experimental results of ALF ON case. BD-rate for luma is -0.02%/0.00%/0.04% (RA/LDB/LDP) in comparison with VTM-6.0.

Table 1 Experimental results for RA condition with ALF ON.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.01% | -0.01% | 100% | 100% |
| Class A2 | -0.04% | 0.08% | 0.02% | 100% | 101% |
| Class B | -0.02% | 0.06% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.04% | -0.02% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.05% | 0.00% | 100% | 100% |
| Class D | -0.03% | -0.08% | 0.00% | 100% | 99% |
| Class F | 0.01% | 0.03% | 0.00% | 100% | 100% |

Table 2 Experimental results for LDB condition with ALF ON.

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.02% | -0.13% | 100% | 101% |
| Class C | -0.03% | -0.13% | 0.03% | 100% | 101% |
| Class E | 0.01% | -0.08% | 0.69% | 100% | 100% |
| Overall | 0.00% | -0.05% | 0.13% | 100% | 100% |
| Class D | 0.05% | 0.60% | 0.24% | 100% | 102% |
| Class F | 0.01% | -0.07% | 0.95% | 100% | 100% |

Table 3 Experimental results for LDP condition with ALF ON.

|  |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.13% | 0.37% | 100% | 100% |
| Class C | 0.03% | -0.05% | 0.13% | 100% | 99% |
| Class E | 0.12% | 0.02% | 0.29% | 100% | 99% |
| Overall | 0.04% | 0.04% | 0.27% | 100% | 100% |
| Class D | 0.03% | -0.24% | -0.30% | 100% | 100% |
| Class F | -0.05% | 0.12% | 0.04% | 100% | 99% |

From Table 4 to Table 6 show the experimental results of ALF OFF case. BD-rate for luma is -0.01%/-0.02%/0.00% (RA/LDB/LDP) in comparison with VTM-6.0.

Table 4 Experimental results for RA condition with ALF OFF.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.15% | 0.03% | 100% | 100% |
| Class A2 | -0.07% | -0.07% | 0.01% | 100% | 100% |
| Class B | 0.00% | -0.06% | -0.05% | 100% | 100% |
| Class C | 0.02% | 0.01% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.00% | -0.01% | 100% | 100% |
| Class D | -0.02% | 0.04% | 0.00% | 100% | 98% |
| Class F | 0.01% | 0.01% | 0.04% | 100% | 100% |

Table 5 Experimental results for LDB condition with ALF OFF.

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.07% | -0.11% | 100% | 100% |
| Class C | 0.02% | 0.16% | -0.13% | 100% | 100% |
| Class E | -0.12% | -0.23% | -0.17% | 100% | 99% |
| Overall | -0.02% | -0.04% | -0.13% | 100% | 100% |
| Class D | 0.01% | -0.11% | -0.22% | 100% | 100% |
| Class F | 0.02% | 0.02% | 0.03% | 100% | 98% |

Table 6 Experimental results for LDP condition with ALF OFF.

|  |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.01% | 0.12% | 100% | 100% |
| Class C | -0.02% | -0.07% | -0.17% | 100% | 101% |
| Class E | 0.01% | 0.03% | -0.18% | 100% | 99% |
| Overall | 0.00% | -0.02% | -0.05% | 100% | 100% |
| Class D | -0.02% | 0.18% | -0.02% | 100% | 100% |
| Class F | 0.08% | 0.11% | 0.06% | 100% | 100% |

Subjective evaluations

Figure 1 and Figure 2 show examples of subjective quality differences. In Figure 1, the decoded image of the proposed method is sharper than that of VTM-6.0. In Figure 2, there are blur around texts “effects to an” in the decoded image of VTM-6.0.

Cropped area

VTM-6.0                              Proposed method

Figure 1 Examples of decoded images (Cactus, QP37, RA, ALF ON, POC80).

Cropped area

VTM-6.0                              Proposed method

Figure 2 Examples of decoded images (SlideShow, QP37, RA, ALF ON, POC171).

Conclusion

In this contribution, it was proposed that boundary strength calculation for deblocking filtering at PROF sub-block boundary. In the proposed method, differences between P side and Q side were treated as zero in the boundary strength calculation at PROF sub-block boundary. Experimental results showed that BD-rates for luma were -0.02%/0.00%/0.04% (RA/LDB/LDP) in comparison with VTM-6.0. And BD-rates for luma were -0.01%/-0.02%/0.00% (RA/LDB/LDP) in comparison with VTM-6.0 without ALF.

References

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, Gothenburg, SE, July 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, Mar. 2019.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).