| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0552-v2 |
| --- | --- |

| Title: | AHG12/Non-CE5: Extending slice boundary processing for adaptive loop filter for raster scanned slices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu Vadim Seregin Muhammed Coban Adarsh Krishnan Ramasubramonian Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In VVC draft 6, mirrored padding is applied to adaptive loop filter for slice boundaries when loop_filter_across_slices_enabled_flag is equal to 0. However, the definition of clipping boundaries is missing in section 8.8.5.3 when calculating ac and sumH/V/D0/HV. In addition, if raster-scan slice is used there can be some cases that are not supported depending on the slice configuration. In this document, it is proposed to extend slice boundary processing when loop_filter_across_slices_enabled_flag is equal to 0 to all slice configurations by converting non-supported cases to the existed method of the slice boundaries handling.

# Introduction

In VVC draft 6, when ALF can be constrained by only using samples of the same slice and if current to-be-filtered sample needs a sample in the top and/or left coding tree unit (CTU) belong to a different slice, the mirrored padding method is applied. However, in section 8.8.5.3, when calculating ac and sumH/V/D0/HV for a 4x4 sub-block, the definition of variables clipTopPos, clipBottomPos, clipLeftPos and clipRightPos are missed. The fix is provided in the second reversion of this document.

In addition, if the current to-be-filtered sample needs a sample in a CTU in diagonal direction, current draft cannot handle this case. For example, in Figure 1, 2 and 3, when current to-be-filtered sample (in CTUd) needs a sample in its top-left CTUa, current draft will just use this outside sample. Same happened to the example in Figure 4, when current to-be-filtered sample (in CTUa) needs a sample in its bottom-left CTUa.

Figure 1. Diagonal sample required for ALF filtering.

Figure 2. Diagonal samples required for Laplacian calculations

Figure 3. Diagonal samples/Laplacians required for 4x4 block classification

# Proposed method

To handle all unsupported cases in VVC draft 6, it is proposed to convert such cases to a case, which is already supported, such as to treat the above CTU boundary or the left CTU boundary as a slice boundary even some samples belong to the same slice.

In Figure 4, the problem is converted into the case when the top CTU is in a different slice such that mirrored padding is applied along the top CTU boundary.

In Figure 5, the problem is converted into the case when the left CTU is in a different slice such that mirrored padding is applied along left CTU boundary.

Figure 4. Mirrored padding applied along the top CTU boundary

Figure 5. Mirrored padding applied along the left CTU boundary

In the working draft, the clipping boundaries are calculated for each sample when calculating Laplacians and filtering. However, those values are calculated for each 4x4 sub-block when doing the classification based on the top-left position of each 4x4 sub-block. To check if a sample/sub-block is under a special case, two thresholds are introduced for section 8.8.5.5, one is for top-left threshold and the other is for bottom-right threshold. If the distance between a sample/sub-block and top-left boundary is less than the top-left threshold or the distance between a sample/sub-block and bottom-right boundary is less than the bottom-right threshold, a special case is identified. The thresholds are defined in the table below.

|  | Threshold for top-left | Threshold for bottom-right |
| --- | --- | --- |
| Laplacian calculation | 1 | 1 |
| 4x4 classification | 4 | 8 |
| luma filtering | 2 | 2 |
| chroma filtering | 1 | (SubWidthC >> 1) + (SubHeightC >> 1) |

# Working draft change

On top of a clean version of JVET-O2001-vE, the fixes to the range and clipped positions are heighted in green and the proposed method is highlighted in yellow.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).