| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0555-v2 |
| --- | --- |

| Title: | CE5-related: Dynamic range reduction for coefficients of cross component adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Jie Dong, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In JVET-P0080, a cross component adaptive loop filter (CC-ALF) is tested to refine chroma components by using luma samples. Each filter has 14 filter coefficients and 18 taps. To reduce the complexity of multiplication, in this contribution, the dynamic range of a CC-ALF coefficient is further reduced. Under common test conditions, compared to CE5-2.1, similar overall coding efficiency and running time are achieved.

# Introduction

In JVET-P0080, a cross component adaptive loop filter (CC-ALF) is tested to refine chroma components by using luma sample values. Each filter has 14 filter coefficients and 18 taps. Every coefficient has an 8-bit dynamic range and is signalled with a third order exponential-Golomb code. However, this may increase the complexity, especially in decoder, by introducing additional 12 8-bit multipliers, 1 9-bit multiplier and 1 10-bit multiplier per chroma pixel. In version 2, results are updated.

# Proposed method

Two methods are proposed in this proposal:

- Dynamic range of a CC-ALF coefficient is further reduced (from 3 to 6 bits ranges are tested)

- Fixed length codes (FLC) instead of third order exponential-Golomb codes is used to signal CC-ALF coefficients.

# Experimental results

The proposed methods are implemented on top of the in-loop version of CC-ALF in CE5-2.1. The simulations were conducted under JVET common test conditions [1].

## Fixed length coding

In this test EG3 is replaced with FLC8.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | 0.03% | 100% | 101% |
| Class A2 | 0.01% | 0.01% | 0.00% | 101% | 100% |
| Class B | 0.02% | 0.02% | 0.07% | 100% | 98% |
| Class C | 0.03% | 0.13% | 0.18% | 99% | 101% |
| Class E | 0.04% | 0.04% | 0.44% | 100% | 98% |
| Overall | 0.02% | 0.05% | 0.14% | 100% | 100% |
| Class D | -0.02% | 0.62% | 0.57% | 100% | 100% |
| Class F | 0.03% | 0.07% | 0.06% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.04% | 0.06% | 101% | 101% |
| Class A2 | 0.00% | 0.18% | 0.11% | 101% | 101% |
| Class B | -0.02% | 0.50% | 0.52% | 99% | 100% |
| Class C | 0.00% | 0.47% | 0.40% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.34% | 0.32% | 100% | 100% |
| Class D | -0.02% | 1.23% | 0.49% | 98% | 101% |
| Class F | 0.01% | 0.21% | 0.18% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.39% | 0.69% | 100% | 101% |
| Class C | -0.02% | 0.42% | 0.12% | 100% | 99% |
| Class E | -0.06% | 1.13% | 0.70% | 99% | 99% |
| Overall | -0.03% | 0.58% | 0.50% | 100% | 100% |
| Class D | 0.01% | 1.00% | 0.72% | 100% | 98% |
| Class F | 0.09% | 0.45% | 0.17% | 101% | 101% |

## 6-bit dynamic range

In this test dynamic range of the coefficients is reduced to 6 bits.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class A2 | -0.01% | 0.28% | 0.01% | 101% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.05% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class A2 | 0.00% | 0.11% | 0.01% | 99% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.00% | 99% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.02% | 0.21% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.01% | 0.09% | 99% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class F | -0.02% | 0.15% | 0.00% | 99% | 100% |

## 6-bit dynamic range + fixed length coding

In this test dynamic range of the coefficients is reduced to 6 bits and EG3 is replaced with FLC6.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.00% | 100% | 99% |
| Class A2 | -0.01% | 0.28% | 0.01% | 99% | 101% |
| Class B | 0.01% | 0.01% | 0.02% | 102% | 100% |
| Class C | 0.01% | 0.04% | 0.06% | 99% | 101% |
| Class E | 0.02% | 0.01% | 0.12% | 99% | 100% |
| Overall | 0.01% | 0.06% | 0.04% | 100% | 100% |
| Class D | -0.01% | 0.32% | 0.26% | 100% | 100% |
| Class F | 0.01% | 0.02% | 0.02% | 100% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.06% | 100% | 100% |
| Class A2 | -0.01% | 0.13% | 0.10% | 100% | 99% |
| Class B | -0.01% | 0.26% | 0.21% | 100% | 102% |
| Class C | -0.01% | 0.17% | 0.11% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.16% | 0.13% | 101% | 100% |
| Class D | -0.02% | 0.59% | 0.17% | 100% | 100% |
| Class F | 0.00% | 0.10% | 0.07% | 101% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.09% | 0.29% | 101% | 101% |
| Class C | -0.01% | -0.22% | -0.15% | 100% | 99% |
| Class E | -0.05% | 0.55% | 0.36% | 99% | 100% |
| Overall | -0.02% | 0.10% | 0.16% | 100% | 100% |
| Class D | -0.01% | 0.60% | 0.08% | 99% | 99% |
| Class F | 0.12% | 0.20% | -0.27% | 101% | 101% |

## 5-bit dynamic range

In this test dynamic range of the coefficients is reduced to 5 bits.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.02% | 99% | 101% |
| Class A2 | -0.03% | 1.02% | 0.25% | 99% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | -0.01% | 0.17% | 0.05% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.03% | 100% | 100% |
| Class A2 | -0.02% | 0.67% | 0.32% | 99% | 101% |
| Class B | 0.00% | -0.01% | 0.00% | 99% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.12% | 0.07% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class F | 0.00% | 0.02% | 0.01% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.14% | 0.26% | 100% | 101% |
| Class C | -0.01% | -0.08% | -0.07% | 101% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.03% | 0.09% | 100% | 100% |
| Class D | 0.00% | 0.03% | -0.01% | 100% | 100% |
| Class F | 0.12% | -0.03% | 0.72% | 99% | 100% |

## 5-bit dynamic range + fixed length coding

In this test dynamic range of the coefficients is reduced to 5 bits and EG3 is replaced with FLC5.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.01% | 101% | 100% |
| Class A2 | -0.03% | 1.02% | 0.25% | 99% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class C | 0.00% | 0.00% | 0.01% | 99% | 99% |
| Class E | 0.00% | 0.00% | 0.01% | 99% | 99% |
| Overall | 0.00% | 0.17% | 0.05% | 100% | 99% |
| Class D | 0.00% | 0.07% | 0.09% | 101% | 100% |
| Class F | 0.00% | 0.01% | 0.00% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.07% | 0.06% | 100% | 101% |
| Class A2 | -0.02% | 0.68% | 0.34% | 100% | 101% |
| Class B | -0.01% | 0.04% | 0.05% | 100% | 99% |
| Class C | -0.01% | 0.04% | 0.03% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.15% | 0.10% | 100% | 100% |
| Class D | 0.00% | 0.14% | 0.00% | 99% | 100% |
| Class F | 0.00% | 0.07% | 0.03% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.08% | 0.23% | 99% | 101% |
| Class C | 0.00% | -0.03% | -0.41% | 99% | 101% |
| Class E | -0.04% | 0.48% | -0.13% | 98% | 100% |
| Overall | -0.01% | 0.14% | -0.08% | 99% | 100% |
| Class D | 0.04% | 0.21% | 0.66% | 100% | 100% |
| Class F | 0.07% | -0.16% | 0.51% | 99% | 100% |

## 4-bit dynamic range + fixed length coding

In this test dynamic range of the coefficients is reduced to 4 bits and EG3 is replaced with FLC4.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.29% | 0.16% | 101% | 98% |
| Class A2 | -0.05% | 1.40% | 1.06% | 99% | 102% |
| Class B | 0.00% | 0.07% | 0.02% | 99% | 100% |
| Class C | -0.01% | -0.03% | -0.03% | 99% | 99% |
| Class E | -0.01% | -0.01% | -0.10% | 99% | 99% |
| Overall | -0.01% | 0.29% | 0.19% | 99% | 100% |
| Class D | 0.01% | -0.19% | -0.16% | 102% | 98% |
| Class F | -0.01% | 0.03% | 0.01% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.62% | 0.32% | 101% | 100% |
| Class A2 | 0.00% | 1.41% | 1.37% | 100% | 100% |
| Class B | 0.00% | -0.03% | -0.07% | 99% | 101% |
| Class C | 0.01% | -0.07% | -0.06% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.38% | 0.30% | 100% | 100% |
| Class D | 0.00% | -0.42% | -0.10% | 102% | 100% |
| Class F | 0.00% | 0.14% | 0.08% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.33% | 0.03% | 101% | 100% |
| Class C | 0.03% | -0.05% | -0.38% | 100% | 100% |
| Class E | -0.07% | -0.83% | 0.00% | 101% | 100% |
| Overall | 0.01% | -0.08% | -0.11% | 101% | 100% |
| Class D | -0.01% | -0.16% | -0.10% | 100% | 101% |
| Class F | 0.00% | 0.63% | -0.01% | 100% | 100% |

## 3-bit dynamic range + fixed length coding

In this test dynamic range of the coefficients is reduced to 3 bits and EG3 is replaced with FLC3.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.88% | 0.89% | 101% | 99% |
| Class A2 | -0.06% | 2.18% | 3.77% | 100% | 99% |
| Class B | -0.01% | 0.73% | 0.51% | 101% | 99% |
| Class C | -0.02% | -0.03% | -0.07% | 98% | 100% |
| Class E | -0.03% | 0.29% | 0.09% | 100% | 99% |
| Overall | -0.03% | 0.76% | 0.92% | 100% | 99% |
| Class D | 0.01% | -0.45% | -0.34% | 101% | 100% |
| Class F | -0.01% | 0.13% | -0.03% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 1.53% | 1.39% | 101% | 101% |
| Class A2 | 0.02% | 2.63% | 3.60% | 102% | 100% |
| Class B | 0.01% | 0.87% | 0.18% | 100% | 101% |
| Class C | 0.02% | -0.14% | -0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 1.08% | 1.02% | 101% | 100% |
| Class D | 0.00% | -0.78% | -0.33% | 101% | 100% |
| Class F | -0.01% | 0.36% | 0.20% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 1.49% | 0.89% | 100% | 100% |
| Class C | 0.07% | -0.26% | -0.35% | 99% | 100% |
| Class E | -0.14% | -0.23% | 0.37% | 98% | 99% |
| Overall | -0.02% | 0.48% | 0.34% | 99% | 100% |
| Class D | 0.05% | -0.46% | -0.31% | 101% | 101% |
| Class F | 0.15% | 0.60% | 0.26% | 100% | 99% |

# Reference

[1] JVET-N1010, “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).