| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0559 |
| --- | --- |

| Title: | AHG18: Rice parameter derivation for coefficient level coding in lossless |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Hongtao Wang, Yung-Hsuan Chao, Muhammed Coban, | Tel: Email: | martak@qti.qualcomm.com hongtaow@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

The document marks the spec changes on top of CE7-1.2d-alternative

9.3.3.2	Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ]

Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the Rice parameter cRiceParam.

Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo code:

- If transform_skip_flag[ x0 ][ y0 ] is equal to 1, trafoSkip is set equal to 1 and the following applies:

offSet = (baseLevel = = 0) ? 0: ( (baseLevel = = 1)? 4: -8 )

noPos = 0

locSumAbs = 0
if( xC > 0 ) {

noPos += 1
	locSumAbs += AbsLevel[ xC − 1 ][ yC ]

}
if( yC > 0 ) {	(9-9)
	noPos += 1

locSumAbs += AbsLevel[ xC ][ yC − 1 ]

}

locSumAbs = (noPos = = 1 )? 2*locSumAbs : locSumAbs
locSumAbs = Clip3( 0, 31, locSumAbs + offSet ) + (locSumAbs > 128) + ( (offSet = = 0)? 0: (locSumAbs > 64))

- Otherwise (transform_skip_flag[ x0 ][ y0 ] is equal to 0), trafoSkip is set equal to 0 and the following applies:

noPos = 0

locSumAbs = 0
if( xC < (1 << log2TbWidth) − 1 ) {

noPos += 1
	locSumAbs += AbsLevel[ xC + 1 ][ yC ]
	if( xC < (1 << log2TbWidth) − 2 ) {

noPos += 1
		locSumAbs += AbsLevel[ xC + 2 ][ yC ]

}
	if( yC < (1 << log2TbHeight) − 1 ) {

noPos += 1
		locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ]	(9-10)
   }

}
if( yC < (1 << log2TbHeight) − 1 ) {

noPos += 1
	locSumAbs += AbsLevel[ xC ][ yC + 1 ]
	if( yC < (1 << log2TbHeight) − 2 ) {

noPos += 1
		locSumAbs += AbsLevel [ xC ][ yC + 2 ]

}
}

locSumAbs = (noPos = = 0 | | noPos = = 5)? locSumAbs : (locSumAbs*5/noPosNorm[noPos-1])

The list noPosNorm[] is specified as noPosNorm[j] = {1, 2, 2, 4} with j = 0, …,3.
locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 ) + (locSumAbs > 256) + (baseLevel? 0: (locSumAbs > 128))

The following applies:

- If baseLevel is equal to 0, the variable s is set equal to Max( 0, QState − 1 ) and given the variables locSumAbs, trafoSkip and s, the Rice parameter cRiceParam and the variable ZeroPos[ n ] are derived as specified in Table 9-78.

- Otherwise (baseLevel is greater than 0), given the variables locSumAbs and trafoSkip, the Rice parameter cRiceParam is derived as specified in Table 9-78.

Table 9-78 – Specification of cRiceParam and ZeroPos[ n ] based on locSumAbs, trafoSkip and s

| trafoSkip | s | locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  | cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| 1 |  | cRiceParam | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 |
|  | 0 | ZeroPos[ n ] | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 4 | 4 | 4 | 4 |
|  | 1 | ZeroPos[ n ] | 1 | 1 | 1 | 1 | 2 | 3 | 4 | 4 | 4 | 6 | 6 | 6 | 8 | 8 | 8 | 8 |
|  | 2 | ZeroPos[ n ] | 1 | 1 | 2 | 2 | 2 | 3 | 4 | 4 | 4 | 6 | 6 | 6 | 8 | 8 | 8 | 8 |
|  |  | locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 0 |  | cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |
| 1 |  | cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
|  | 0 | ZeroPos[ n ] | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 8 | 8 | 8 | 8 | 8 | 16 | 16 | 16 | 16 |
|  | 1 | ZeroPos[ n ] | 4 | 4 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 16 | 16 | 16 | 16 | 16 | 16 |
|  | 2 | ZeroPos[ n ] | 8 | 8 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
|  |  | locSumAbs | 32 | 33 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  | cRiceParam | 4 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | cRiceParam | 4 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | ZeroPos[ n ] | 16 | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | ZeroPos[ n ] | 16 | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | ZeroPos[ n ] | 16 | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Reference

- A. Nalci, H. Wang, H. E. Egilmez, M. Coban, M. Karczewicz, “AHG18: BDPCM for Lossless”, JVET-P0082, 15th meeting, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)