| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0562 |
| --- | --- |

| Title: | Non-CE8: Transform skip residual coding simplification |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | +1-858-7052600 yiwenchen@kwai.com |  |
| Source: | Kwai Inc. |  |  |  |

_____________________________

# Abstract

In residual coding of a transform skip block, for each sample, the remaining absolute levels are adaptively binarized using the rice parameters derived depending on the levels of the left and above residual samples. In this contribution, a fix codeword (rice parameter =K) is used to replace this per-sample adaptive codeword derivation.

The results of using fixed rice parameter=1 on TGM sequences compared to VTM6.0 are shown below:

Standard QP (22, 27, 32, 37): 0.02% AI, -0.02% RA, -0.06% LB

Low QP (2, 7, 12, 17): -0.37% AI, 0.08% RA, -0.19% LB

The results of using fixed rice parameter=2 on TGM sequences compared to VTM6.0 are shown below:

Standard QP (22, 27, 32, 37): 0.25% AI, 0.11% RA, 0.03% LB

Low QP (2, 7, 12, 17): -2.15% AI, -1.81% RA, -1.35% LB

Moreover, it is observed that larger rice parameters are preferred under lower QPs because the codewords of larger rice parameters better fit the distributions of the residual value under lower QPs. It is proposed to use adaptive binary codewords for the syntax element abs_remainder according to the CU quantization parameter (QP).

The results of QP-dependent rice parameter determination on TGM sequences compared to VTM6.0:

Standard QP (22, 27, 32, 37): 0.25% AI, 0.11% RA, 0.03% LB

Low QP (2, 7, 12, 17): -3.55% AI, -2.97% RA, -2.08% LB

# Introduction

In residual coding of a transform skip block, the residuals of a TU are also coded in units of non-overlapped subblocks of size 4x4. For each subblock, if the coded_subblock_flag is equal to 1 (i.e., there is at least one non-zero quantized residual in the subblock), coding of the quantized residual levels is performed in three scan passes:

- First scan pass: significance flag (sig_coeff_flag), sign flag (coeff_sign_flag), absolute level greater than 1 flag (abs_level_gtx_flag[0]), and parity (par_level_flag) are coded. For a given scan position, if sig_coeff_flag is equal to 1, then coeff_sign_flag is coded, followed by the abs_level_gtx_flag[0] (which specifies whether the absolute level is greater than 1). If abs_level_gtx_flag[0] is equal to 1, then the par_level_flag is additionally coded to specify the parity of the absolute level.

- Greater than x scan pass: for each scan position whose absolute level is greater than 1, up to four abs_level_gtx_flag[i] for i = 1...4 are coded to indicate if the absolute level at the given position is greater than 3, 5, 7, or 9, respectively.

- Remainder scan pass: The remainder of the absolute level (abs_remainder ) are coded for all scan positions with abs_level_gtx_flag[4] equal to 1 (that is, the absolute level is greater than 9). The remainder of the absolute levels are binarized using reduced rice parameter derivation template.

In the remainder scan pass, the remainder of the absolute levels are adaptively binarized using the rice parameters derived depending on the levels of the left and above residual samples.

# Proposal

In this proposal, it is proposed to use a fix binary codeword (e.g. cRiceParam equal to K) for the syntax element abs_remainder. The codeword binarization process is illustrated in the below Table. With the proposed simplification, the per-sample codeword determination can be removed.

Table 1. Proposed modification based on VVC draft specification

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ] Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the Rice parameter cRiceParam. Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo code: - If transform_skip_flag[ x0 ][ y0 ] is equal to 1, trafoSkip is set equal to 1 and the following applies: locSumAbs = 0 if( xC > 0 ) locSumAbs += AbsLevel[ xC − 1 ][ yC ] if( yC > 0 ) (9-9) locSumAbs += AbsLevel[ xC ][ yC − 1 ] locSumAbs = Clip3( 0, 31, locSumAbs ) - Otherwise (transform_skip_flag[ x0 ][ y0 ] is equal to 0), trafoSkip is set equal to 0 and the following applies: locSumAbs = 0 if( xC < (1 << log2TbWidth) − 1 ) { locSumAbs += AbsLevel[ xC + 1 ][ yC ] if( xC < (1 << log2TbWidth) − 2 ) locSumAbs += AbsLevel[ xC + 2 ][ yC ] if( yC < (1 << log2TbHeight) − 1 ) locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] (9-10) } if( yC < (1 << log2TbHeight) − 1 ) { locSumAbs += AbsLevel[ xC ][ yC + 1 ] if( yC < (1 << log2TbHeight) − 2 ) locSumAbs += AbsLevel [ xC ][ yC + 2 ] } locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 ) The following applies: - If baseLevel is equal to 0, the variable s is set equal to Max( 0, QState − 1 ) and given the variables locSumAbs, trafoSkip and s, the Rice parameter cRiceParam and the variable ZeroPos[ n ] are derived as specified in Table 9-78. - Otherwise (baseLevel is greater than 0), given the variables locSumAbs and trafoSkip, the Rice parameter cRiceParam is derived as specified in Table 9-78. Table 9-78 – Specification of cRiceParam and ZeroPos[ n ] based on locSumAbs, trafoSkip and s trafoSkip; s; locSumAbs; 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12; 13; 14; 15. 0; cRiceParam; 0; 0; 0; 0; 0; 0; 0; 1; 1; 1; 1; 1; 1; 1; 2; 2. 1; cRiceParam; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 1; 1; 1; 1. 0; ZeroPos[ n ]; 0; 0; 0; 0; 0; 1; 2; 2; 2; 2; 2; 2; 4; 4; 4; 4. 1; ZeroPos[ n ]; 1; 1; 1; 1; 2; 3; 4; 4; 4; 6; 6; 6; 8; 8; 8; 8. 2; ZeroPos[ n ]; 1; 1; 2; 2; 2; 3; 4; 4; 4; 6; 6; 6; 8; 8; 8; 8. locSumAbs; 16; 17; 18; 19; 20; 21; 22; 23; 24; 25; 26; 27; 28; 29; 30; 31. 0; cRiceParam; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 3; 3; 3; 3. 1; cRiceParam; 1; 1; 1; 1; 1; 1; 1; 1; 1; 2; 2; 2; 2; 2; 2; 2. 0; ZeroPos[ n ]; 4; 4; 4; 4; 4; 4; 4; 8; 8; 8; 8; 8; 16; 16; 16; 16. 1; ZeroPos[ n ]; 4; 4; 12; 12; 12; 12; 12; 12; 12; 12; 16; 16; 16; 16; 16; 16. 2; ZeroPos[ n ]; 8; 8; 12; 12; 12; 12; 12; 12; 12; 16; 16; 16; 16; 16; 16; 16. #### 9.3.3.11 Binarization process for abs_remainder[ ] Input to this process is a request for a binarization for the syntax element abs_remainder[ n ], the colour component cIdx, the current sub-block index i, and the luma location ( x0, y0 ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the binarization of the syntax element. The variables lastAbsRemainder and lastRiceParam are derived as follows: - If this process is invoked for the first time for the current sub-block index i, lastAbsRemainder and lastRiceParam are both set equal to 0. - Otherwise (this process is not invoked for the first time for the current sub-block index i), lastAbsRemainder and lastRiceParam are set equal to the values of abs_remainder[ n ] and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element abs_remainder[ n ] as specified in this clause. The rice parameter cRiceParam is derived by invoking the rice parameter derivation process for abs_remainder[] as specified in clause 9.3.3.2 with the variable baseLevel set equal to 4, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs. The rice parameter cRiceParam is set to K. The variable cMax is derived from cRiceParam as: cMax = 6 << cRiceParam (9-20) The binarization of the syntax element abs_remainder[ n ] is a concatenation of a prefix bin string and (when present) a suffix bin string. For the derivation of the prefix bin string, the following applies: - The prefix value of abs_remainder[ n ], prefixVal, is derived as follows: prefixVal = Min( cMax, abs_remainder[ n ] ) (9-21) - The prefix bin string is specified by invoking the TR binarization process as specified in clause 9.3.3.3 for prefixVal with the variables cMax and cRiceParam as inputs. When the prefix bin string is equal to the bit string of length 6 with all bits equal to 1, the suffix bin string is present and it is derived as follows: - The suffix value of abs_remainder[ n ], suffixVal, is derived as follows: suffixVal = abs_remainder[ n ] − cMax (9-22) - The suffix bin string is specified by invoking the limited k-th order EGk binarization process as specified in clause 9.3.3.6 for the binarization of suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1 and cRiceParam as input.

Moreover, it is observed that larger rice parameters are preferred under lower QPs because the codewords of larger rice parameters better fit the distributions of the residual value under lower QPs. It is proposed to use adaptive binary codewords for the syntax element abs_remainder according to the CU quantization parameter (QP). For a given block of residual samples, the rice parameters used are determined according to the CU QP, denoted as QPCU, instead of the neighboring level information. One specific example is illustrated as shown in Table 1, where TH1 to TH4 are predefined thresholds satisfying (TH1 < TH2 < TH3 < Th4),  and K0 to K4 are predefined rice parameters. It is worth noting that the same logics can be implemented differently in practice. For example, certain equations, or a look-up table, may also be used to derive the same rice parameters, as shown in Table 1, from a QP value of a current CU.

Table 1. Proposed modification based on VVC draft specification

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ] Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the Rice parameter cRiceParam. Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo code: - If transform_skip_flag[ x0 ][ y0 ] is equal to 1, trafoSkip is set equal to 1 and the following applies: locSumAbs = 0 if( xC > 0 ) locSumAbs += AbsLevel[ xC − 1 ][ yC ] if( yC > 0 ) (9-9) locSumAbs += AbsLevel[ xC ][ yC − 1 ] locSumAbs = Clip3( 0, 31, locSumAbs ) - Otherwise (transform_skip_flag[ x0 ][ y0 ] is equal to 0), trafoSkip is set equal to 0 and the following applies: locSumAbs = 0 if( xC < (1 << log2TbWidth) − 1 ) { locSumAbs += AbsLevel[ xC + 1 ][ yC ] if( xC < (1 << log2TbWidth) − 2 ) locSumAbs += AbsLevel[ xC + 2 ][ yC ] if( yC < (1 << log2TbHeight) − 1 ) locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] (9-10) } if( yC < (1 << log2TbHeight) − 1 ) { locSumAbs += AbsLevel[ xC ][ yC + 1 ] if( yC < (1 << log2TbHeight) − 2 ) locSumAbs += AbsLevel [ xC ][ yC + 2 ] } locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 ) The following applies: - If baseLevel is equal to 0, the variable s is set equal to Max( 0, QState − 1 ) and given the variables locSumAbs, trafoSkip and s, the Rice parameter cRiceParam and the variable ZeroPos[ n ] are derived as specified in Table 9-78. - Otherwise (baseLevel is greater than 0), given the variables locSumAbs and trafoSkip, the Rice parameter cRiceParam is derived as specified in Table 9-78. Table 9-78 – Specification of cRiceParam and ZeroPos[ n ] based on locSumAbs, trafoSkip and s trafoSkip; s; locSumAbs; 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12; 13; 14; 15. 0; cRiceParam; 0; 0; 0; 0; 0; 0; 0; 1; 1; 1; 1; 1; 1; 1; 2; 2. 1; cRiceParam; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 1; 1; 1; 1. 0; ZeroPos[ n ]; 0; 0; 0; 0; 0; 1; 2; 2; 2; 2; 2; 2; 4; 4; 4; 4. 1; ZeroPos[ n ]; 1; 1; 1; 1; 2; 3; 4; 4; 4; 6; 6; 6; 8; 8; 8; 8. 2; ZeroPos[ n ]; 1; 1; 2; 2; 2; 3; 4; 4; 4; 6; 6; 6; 8; 8; 8; 8. locSumAbs; 16; 17; 18; 19; 20; 21; 22; 23; 24; 25; 26; 27; 28; 29; 30; 31. 0; cRiceParam; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 3; 3; 3; 3. 1; cRiceParam; 1; 1; 1; 1; 1; 1; 1; 1; 1; 2; 2; 2; 2; 2; 2; 2. 0; ZeroPos[ n ]; 4; 4; 4; 4; 4; 4; 4; 8; 8; 8; 8; 8; 16; 16; 16; 16. 1; ZeroPos[ n ]; 4; 4; 12; 12; 12; 12; 12; 12; 12; 12; 16; 16; 16; 16; 16; 16. 2; ZeroPos[ n ]; 8; 8; 12; 12; 12; 12; 12; 12; 12; 16; 16; 16; 16; 16; 16; 16. #### 9.3.3.11 Binarization process for abs_remainder[ ] Input to this process is a request for a binarization for the syntax element abs_remainder[ n ], the colour component cIdx, the current sub-block index i, and the luma location ( x0, y0 ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight, and the quantization parameter Qp′Y. Output of this process is the binarization of the syntax element. The variables lastAbsRemainder and lastRiceParam are derived as follows: - If this process is invoked for the first time for the current sub-block index i, lastAbsRemainder and lastRiceParam are both set equal to 0. - Otherwise (this process is not invoked for the first time for the current sub-block index i), lastAbsRemainder and lastRiceParam are set equal to the values of abs_remainder[ n ] and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element abs_remainder[ n ] as specified in this clause. The rice parameter cRiceParam is derived by invoking the rice parameter derivation process for abs_remainder[] as specified in clause 9.3.3.2 with the variable baseLevel set equal to 4, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs. The rice parameter cRiceParam is derived as: if(Qp′Y <4) { cRiceParam = 1 } else if(Qp′Y <8) { cRiceParam = 2 } else if(Qp′Y <16) { cRiceParam = 3 } else if(Qp′Y <32) { cRiceParam = 4 } else { cRiceParam = 5 } The variable cMax is derived from cRiceParam as: cMax = 6 << cRiceParam (9-20) The binarization of the syntax element abs_remainder[ n ] is a concatenation of a prefix bin string and (when present) a suffix bin string. For the derivation of the prefix bin string, the following applies: - The prefix value of abs_remainder[ n ], prefixVal, is derived as follows: prefixVal = Min( cMax, abs_remainder[ n ] ) (9-21) - The prefix bin string is specified by invoking the TR binarization process as specified in clause 9.3.3.3 for prefixVal with the variables cMax and cRiceParam as inputs. When the prefix bin string is equal to the bit string of length 6 with all bits equal to 1, the suffix bin string is present and it is derived as follows: - The suffix value of abs_remainder[ n ], suffixVal, is derived as follows: suffixVal = abs_remainder[ n ] − cMax (9-22) - The suffix bin string is specified by invoking the limited k-th order EGk binarization process as specified in clause 9.3.3.6 for the binarization of suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1 and cRiceParam as input.

# Simulation Results

In the first version of proposal, all the simulations are run under CE8 CTC. In the updated proposal, the proposed methods are evaluated according to the CTC [1]. The HashME, BDPCM, IBC are turned on for Classes F and TGM. Standard QPs (22, 27, 32, 37) and low QPs (2, 7, 12, 17) are both tested. The detailed results can be found in the attached excels.

Table 3 Results of rice parameter always fixed to 1 (CTC sequences, standard QPs)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class B | 0.00% | -0.01% | -0.03% | 100% | 100% |
| Class C | 0.01% | -0.02% | -0.01% | 101% | 99% |
| Class E | 0.00% | 0.02% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | -0.01% | 100% | 99% |
| Class D | 0.00% | -0.04% | 0.01% | 100% | 103% |
| Class F | 0.00% | -0.11% | -0.03% | 100% | 99% |
| TGM | 0.02% | 0.00% | 0.00% | 100% | 101% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.02% | 100% | 99% |
| Class A2 | 0.00% | 0.04% | 0.01% | 100% | 101% |
| Class B | 0.00% | 0.05% | -0.04% | 100% | 100% |
| Class C | 0.00% | 0.03% | 0.00% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | -0.01% | 100% | 100% |
| Class D | -0.04% | -0.08% | -0.11% | 101% | 100% |
| Class F | -0.07% | -0.02% | -0.08% | 100% | 101% |
| TGM | -0.02% | -0.10% | -0.07% | 101% | 99% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.02% | -0.12% | -0.11% | 99% | 99% |
| Class E | 0.03% | -0.02% | 0.12% | 101% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.02% | 0.07% | 0.32% | 100% | 98% |
| Class F | 0.09% | -0.11% | 0.21% | 100% | 99% |
| TGM | -0.06% | -0.02% | -0.09% | 99% | 100% |

Table 4 Results of rice parameter always fixed to 1 (CTC sequences, low QPs)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Class C | -0.01% | 0.01% | 0.03% | 100% | 100% |
| Class E | 0.01% | -0.01% | 0.01% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class D | -0.02% | 0.01% | 0.05% | 100% | 100% |
| Class F | 0.02% | -0.09% | -0.08% | 100% | 100% |
| TGM | -0.37% | -0.41% | -0.41% | 101% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.01% | 0.01% | -0.01% | 100% | 99% |
| Class C | 0.00% | 0.00% | -0.02% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | -0.01% | 0.03% | 102% | 100% |
| Class F | 0.01% | -0.16% | -0.04% | 99% | 100% |
| TGM | 0.08% | -0.34% | -0.34% | 100% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.01% | -0.01% | 100% | 100% |
| Class C | -0.02% | 0.03% | 0.00% | 101% | 99% |
| Class E | 0.01% | -0.07% | 0.05% | 100% | 101% |
| Overall | 0.00% | -0.01% | 0.01% | 100% | 100% |
| Class D | 0.00% | -0.03% | -0.05% | 99% | 100% |
| Class F | 0.04% | 0.03% | 0.11% | 101% | 100% |
| TGM | -0.19% | -0.33% | -0.31% | 100% | 99% |

Table 5 Results of rice parameter always fixed to 2 (CTC sequences, standard QPs)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.01% | 99% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | -0.01% | -0.01% | 99% | 99% |
| Class E | 0.00% | 0.02% | -0.01% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.01% | 0.03% | 99% | 101% |
| Class F | 0.09% | 0.10% | 0.12% | 101% | 100% |
| TGM | 0.25% | 0.29% | 0.28% | 101% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.02% | 101% | 100% |
| Class A2 | 0.00% | 0.06% | 0.06% | 100% | 99% |
| Class B | 0.00% | 0.06% | -0.07% | 100% | 100% |
| Class C | -0.01% | 0.01% | 0.03% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | 0.00% | 100% | 100% |
| Class D | 0.01% | -0.06% | 0.03% | 100% | 100% |
| Class F | 0.07% | 0.00% | 0.03% | 100% | 99% |
| TGM | 0.11% | 0.05% | 0.07% | 101% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.01% | -0.08% | -0.05% | 100% | 100% |
| Class E | 0.02% | -0.24% | 0.11% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.02% | 0.04% | 100% | 101% |
| Class F | 0.13% | -0.04% | 0.50% | 100% | 101% |
| TGM | 0.03% | 0.02% | -0.03% | 101% | 100% |

Table 6 Results of rice parameter always fixed to 2 (CTC sequences, low QPs)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | -0.01% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.01% | 101% | 99% |
| Class E | 0.01% | -0.04% | -0.03% | 100% | 99% |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Class D | 0.00% | -0.03% | 0.00% | 100% | 99% |
| Class F | -0.65% | -0.63% | -0.67% | 99% | 100% |
| TGM | -2.15% | -1.97% | -1.97% | 101% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | -0.03% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | -0.05% | -0.04% | 100% | 100% |
| Class F | -0.64% | -0.74% | -0.76% | 101% | 101% |
| TGM | -1.81% | -1.55% | -1.54% | 100% | 97% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | 0.01% | -0.03% | 100% | 99% |
| Class E | 0.01% | -0.03% | 0.00% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | 0.02% | 0.02% | 100% | 99% |
| Class F | -0.47% | -0.50% | -0.51% | 100% | 100% |
| TGM | -1.35% | -1.18% | -1.13% | 99% | 97% |

Table 7 Results of QP dependent rice parameter derivation (CTC sequences, standard QPs)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class A2 | 0.00% | 0.00% | 0.01% | 99% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | -0.01% | -0.01% | 99% | 99% |
| Class E | 0.00% | 0.02% | -0.01% | 101% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.01% | 0.03% | 99% | 103% |
| Class F | 0.09% | 0.10% | 0.12% | 100% | 99% |
| TGM | 0.25% | 0.29% | 0.28% | 101% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.02% | 101% | 100% |
| Class A2 | 0.00% | 0.06% | 0.06% | 100% | 99% |
| Class B | 0.00% | 0.06% | -0.07% | 101% | 100% |
| Class C | -0.01% | 0.01% | 0.03% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | 0.00% | 100% | 99% |
| Class D | 0.01% | -0.06% | 0.03% | 100% | 101% |
| Class F | 0.07% | 0.00% | 0.03% | 98% | 101% |
| TGM | 0.11% | 0.05% | 0.07% | 99% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.01% | -0.08% | -0.05% | 100% | 98% |
| Class E | 0.02% | -0.24% | 0.11% | 101% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.02% | 0.04% | 100% | 98% |
| Class F | 0.13% | -0.04% | 0.50% | 100% | 99% |
| TGM | 0.03% | 0.02% | -0.03% | 100% | 101% |

Table 8 Results of QP dependent rice parameter derivation (CTC sequences, low QPs)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.01% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.01% | 0.01% | 99% | 99% |
| Class C | 0.00% | -0.01% | 0.04% | 99% | 100% |
| Class E | 0.01% | -0.02% | 0.00% | 99% | 101% |
| Overall | 0.00% | 0.00% | 0.01% | 99% | 100% |
| Class D | -0.01% | -0.04% | 0.04% | 101% | 99% |
| Class F | -0.90% | -1.04% | -1.02% | 100% | 99% |
| TGM | -3.55% | -3.18% | -3.16% | 100% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.01% | 0.01% | 0.01% | 100% | 100% |
| Class C | 0.00% | -0.01% | 0.00% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | -0.08% | -0.02% | 100% | 100% |
| Class F | -0.63% | -1.06% | -1.13% | 100% | 99% |
| TGM | -2.97% | -2.39% | -2.39% | 101% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | -0.01% | 100% | 101% |
| Class C | 0.00% | 0.01% | 0.00% | 100% | 100% |
| Class E | -0.01% | -0.02% | 0.02% | 101% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.01% | 0.01% | -0.02% | 101% | 100% |
| Class F | -0.79% | -0.68% | -0.63% | 99% | 101% |
| TGM | -2.08% | -1.74% | -1.69% | 100% | 99% |

# Conclusions

In this proposal, it is proposed to use a fix binary codeword (e.g. cRiceParam equal to 1, or 2) for the syntax element abs_remainder. The results show that the proposed simplification to use fixed rice parameter equal to 2 introduces minor coding performance change under standard QPs and introduce up to 2.15% AI gain under low QPs. Moreover, a QP-dependent rice parameter determination is proposed introducing up to 3.55% AI gain under low QPs. It is suggested to adopt the proposed simplification into VVC.

# Reference

- A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-O2011, 15th meeting, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).