| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0577 |
| --- | --- |

| Title: | AHG17/Non-CE8: APS support for Initial Palette Predictor Entry |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Seethal Paluri Sunmi Yoo Junghak Nam Seunghwan Kim Jaehyun Lim | Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to add support for signaling palette predictor entries in Adaptation Parameter Set (APS). Currently, the palette predictor table is initialized to zero at the beginning of each. Consequently, the palette predictor list has to be constructed from an empty list. It is asserted that this is not a desirable attribute as it could it could take time to construct the predictor list. Therefore, this contribution introduces support to signal the predictor palette entries in the APS to mitigate the effects of re-initializing the predictor palette at the start of each slice.

Introduction

Palette coding is a useful technique for representing blocks containing a small number of distinct color values. Instead of applying a prediction and transform to the block, a palette mode signals indices to indicate the color values of each sample. The palette mode coding in the current Working Draft (WD 6) of the Versatile Video Coding (VVC) specification text is based on the High Efficiency Video Coding (HEVC) Screen Content Coding (SCC) tool [1-2]. In order to decode a coding unit (CU) in the palette mode, a palette predictor is first initialized at the beginning of each slice, where the predictor is set to 0. The initialization process is used to compute the palette predictor size. The predictor palette used for the current CU is formed from a combination of the predictors that are reused from an earlier palette and also from a new set of predictors that are signalled.

In HEVC SCC, it was observed that in the presence of tiles and wavefront parallel processing palette coding resulted in substantial coding efficiency loss as the palette predictor would be reset at the beginning of each slice. It was asserted that after the reset, it took time to rebuild the palette predictor list. For SCC, the content provider or “smart” encoder could generate palette entry and signal entry information to help compression performance by initializing the base palette on the transmitted palette entry.

Proposal

This contribution proposes to signal the initial palette predictor data in the Adaptation Parameter Set (APS). It is asserted that signalling this data in the APS would alleviate or even mitigate the effects of the loss of coding efficiency that result from the re-initialization of the palette predictor at the beginning of each slice. Furthermore, it is also asserted that the APS would be a suitable position as opposed to the slice header as the multiple slices can share the data present in the APS. This contribution imposes a restriction that all the slices in one picture share the same PLT_APS content. Additionally, the number of bits used to signal the palette mode APS id in the slice header is limited to 2. Tables 1 and 2 below shows the syntax associated with the palette mode in the APS, i.e., a new data type PLT_APS is introduced. An APS containing palette_entry_data() would signal, the palette entry bit depths for luma and chroma as well as signal the number of palette entries. Then for each color component and each of the entries, a palette entry is parsed.

Table 1 shows the proposed APS syntax to support Palette entry data

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| else if( aps_params_type = = PLT_APS ) |  |
| palette_entry_data( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 2 shows the proposed data type introduced to support the palette entry

| aps_params_type | Name of aps_params_type | Type of APS parameters |
| --- | --- | --- |
| 0 | ALF_APS | ALF parameters |
| 1 | LMCS_APS | LMCS parameters |
| 2 | SCALING_APS | Scaling list parameters |
| 3 | PLT_APS | Palette entry patameters |
| 4..7 | Reserved | Reserved |

Table 3 shows the proposed palette entry data signaling

| palette_entry_data( ) { | Descriptor |
| --- | --- |
| palette_entry_bit_depth_luma_minus8 | ue(v) |
| palette_entry_bit_depth_chroma_minus8 | ue(v) |
| num_signalled_palette_entry_minus1 | ue(v) |
| for( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| for( i = 0; i < = num_signalled_palette_entries_minus1; i++ ) |  |
| initial_predictor_palette_entries[ cIdx ][ i ] | ue(v) |
| } |  |

palette_entry_bit_depth_luma_minus8 specifies the bit depth of the samples of the palette entry PltBitDepth as follows:

PltBitDepthY      = 8 + palette_entry_bit_depth_luma_minus8

palette_entry_bit_depth_luma_minus8 shall be in the range of 0 to 8, inclusive.

palette_entry_bit_depth_chroma_minus8 specifies the bit depth of the samples of the palette entry PltBitDepth as follows:

PltBitDepthC      = 8 + palette_entry_bit_depth_chroma_minus8

palette_entry_bit_depth_chroma_minus8 shall be in the range of 0 to 8, inclusive.

num_signaled_palette_entry_minus1 specifies the number of entries in the APS palette predictor entry.

initial_predictor_palette_entries[ cIdx ][ i ] specifies the value of the cIdx-th component of the i-th palette entry in the APS that is used to initialize the array PredictorPaletteEntries. For values of i in the range of 0 to num_signalled_palette_entry_minus1, inclusive, the value of initial_predictor_palette_entries[ 0 ][ i ] shall be in the range of 0 to ( 1 << PltBitDepthY) – 1, inclusive, and the values of initial_predictor_palette_entries[ 1 ][ i ] and initial_predictor_palette_entries[ 2 ][ i ] shall be in the range of 0 to ( 1 << PltBitDepthC) – 1, inclusive.

In the slice header, a flag is signaled to indicate the palette predictor entry is used or not. If the flag is parsed as 1, the APS id is parsed and the predictor palette entry is initialized by the value obtained from APS which is slice refers to. Otherwise the predictor palette entry is reset to empty.

Table 4 shows the proposed slice header syntax to support palette initialization from the palette entry of APS

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| … |  |
| if( sps_palette_enabled_flag ) { |  |
| slice_palette_enabled_flag | u(1) |
| slice_palette_entries_present_flag | u(1) |
| if( slice_palette_entries_present_flag ) |  |
| slice_palette_entries_aps_id | u(2) |
| } |  |
| … |  |
| } |  |

slice_palette_enabled_flag equal to 1 specifies that palette prediction mode is enabled for the current slice. slice_palette_enabled_flag equal to 0 specifies that palette prediction mode is not enabled for the current slice. When slice_p slice_palette_enabled_flag equal to 1 specifies that palette prediction mode is enabled for the current slice.

slice_palette_entries_present_flag equal to 1 specifies that the palette predictor used for the current slice is derived based on the palette entries data contained in the referenced palette entries APS and PredictorPaletteSize[cIdx] is set equal to num_signaled_palette_entry_minus1 + 1, cIdx=0..2. The slice_palette_entries_present_flag equal to 0 specifies that the palette predictor used for the current picture is reset to 0 and PredictorPaletteSize[cIdx] is set equal to 0, cIdx=0..2.. When not present, the value of slice_palette_entries_present_flag is inferred to be equal to 0.

slice_palette_aps_id specifies the adaptation_parameter_set_id of the PLT_APS that the slice refers to. The TemporalId of the APS NAL unit having aps_params_type equal to PLT_APS and adaptation_parameter_set_id equal to slice_palette_aps_id shall be less than or equal to the TemporalId of the coded slice NAL unit.

When present, the value of slice_palette_aps_id shall be the same for all slices of a picture.

In the decoding process, the proposed palette entry initialization is applied as following:

### Initialization process

Outputs of this process are initialized CABAC internal variables.

Figure 9-2 – Spatial neighbour T that is used to invoke the CTB availability derivation process relative to the current CTB (informative)

The context variables of the arithmetic decoding engine are initialized as follows:

–	If the CTU is the first CTU in a brick, the initialization process for context variables is invoked as specified in clause 9.3.2.2 and the initialization process for palette is invoked as specified in clause 9.3.2.X.

–	Otherwise, if entropy_coding_sync_enabled_flag is equal to 1 and either CtbAddrInRs % PicWidthInCtbsY is equal to 0 or BrickId[ CtbAddrInBs ] is not equal to BrickId[ CtbAddrRsToBs[ CtbAddrInRs − 1 ] ], the following applies:

–	The location ( xNbT, yNbT ) of the top-left luma sample of the spatial neighbouring block T (Figure 9-2) is derived using the location ( x0, y0 ) of the top-left luma sample of the current CTB as follows:

( xNbT, yNbT ) = ( x0, y0 − CtbSizeY )	(9-3)

–	The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ), the neighbouring location ( xNbY, yNbY ) set equal to ( xNbT, yNbT ), checkPredModeY set equal to FALSE, and cIdx set equal to 0 as inputs, and the output is assigned to availableFlagT.

–	The synchronization process for context variables is invoked as follows:

–	If availableFlagT is equal to 1, the synchronization process for context variables as specified in clause 9.3.2.4 is invoked with TableStateIdx0Wpp, TableStateIdx1Wpp, TableMpsValWpp as inputs.

–	Otherwise, the initialization process for context variables is invoked as specified in clause 9.3.2.2

The initialization process for palette is invoked as specified in clause 9.3.2.X.

–	Otherwise, the initialization process for context variables is invoked as specified in clause 9.3.2.2 and the variable PredictorPaletteSize is initialized to 0. [Ed. (YK): Check whether this can be merged with the condition "If the CTU is the first CTU in a brick", as the operation is the same. This is also the case in HEVC.]

#### Initialization process for palette

Outputs of this process are the initialized palette predictor variables PredictorPaletteSize and PredictorPaletteEntries.

The variable plt_predictor_plt_entry[cIdx][i] is derived from APS with slice_palette_aps_id inclusive, and the variable PredictorPaletteSize[cIdx] is set equal num_predictor_plt_entries_minus1 plus 1 that is signaled at APS with slice_palette_aps_id inclusive

The array PredictorPaletteEntries and the PredictorPaletteSize is derived or modified as follows:

–	If the slice_palette_entries_present_flag is equal to 1, the following applies:

For the luma component palette predictor, The variable shiftBitDepthy is set equal to (BitDepthY – pltBitDepthY )

for( i = 0; i < PredictorPaletteSize[0]; i++ ) {
	PredictorPaletteEntries[ 0 ][ i ] = initial_predictor_palette_entries[ 0 ][ i ] << shiftBitDepthy 
  PredictorPaletteSize[0] = num_signalled_palette_entry_minus1 + 1

}

For the chroma component palette predictor, The variable shiftBitDepthc is set equal to (BitDepthc – PltBitDepthc )

for( cIdx = 1; cIdx < 3; cIdx++ )
	for( i = 0; i < PredictorPaletteSize[cIdx]; i++ ) {
		PredictorPaletteEntries[ cIdx ][ i ] = initial_predictor_palette_entries[ cIdx ][ i ] << shiftBitDepthc 
      PredictorPaletteSize[cIdx] = num_signalled_palette_entry_minus1 + 1

}

–	Otherwise,

for( cIdx = 0; cIdx < 3; cIdx++ )
	for( i = 0; i < PredictorPaletteSize[cIdx]; i++ ) {
		PredictorPaletteEntries[ cIdx ][ i ] = 0
      PredictorPaletteSize[cIdx] = 0

}

It is a requirement of bitstream conformance that the value of PredictorPaletteSize[cIdx] shall be in the range of 0 to PaletteMaxPredictorSize, inclusive.

- 3. Reference

- C. Gisquet, T. Poirier, G. Laroche, P. Onno (Canon), “Non-CE1: On palette prediction for slices”, JCTVC-T0048, 20th Meeting: Geneva, Feb 2015.

- J. Ye, S. Liu, S. Lei (Mediatek), “On palette predictor initialization for Screen Content Coding”, JCTVC-U0084, 21st Meeting: Warsaw, June 2015.

- Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).