| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0579-v1 |
| --- | --- |

| Title: | On Subpicture Information Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Jonatan Samuelsson | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Following is proposed for subpicture information signaling:

- Proposal 1: It is proposed to signal a flag which indicates that each slice contains a sub-picture.

- Proposal 2: Syntax is proposed for conditional signalling of explicit subpicture IDs. When not signalled the subpicture IDs for subpictures are derived.

- Proposal 3: A bug-fix is proposed for the for loop which signals subpic_treated_as_pic_flag[ i ], loop_filter_across_subpic_enabled_flag[ i ] syntax elements.

# Introduction

VVC WD [1] includes signalling of subpictures.

Following is proposed for subpicture information signaling:

- Proposal 1: It is proposed to signal a flag which indicates that each slice contains a sub-picture.

- Proposal 2: Syntax is proposed for conditional signalling of explicit subpicture IDs. When not signalled the subpicture IDs for subpictures are derived.

- Proposal 3: A bug-fix is proposed for the for loop which signals subpic_treated_as_pic_flag[ i ], loop_filter_across_subpic_enabled_flag[ i ] syntax elements.

# Proposals

## Proposal 1

It is asserted that in a typical mode of using subpictures, each subpicture may be signalled in one slice. It is asserted that a signalling of this mode allows decoder to handle and decode each subpicture as soon as each slice is received. Thus the decoder does not need to wait for next slices to know if those include information for the same subpicture.

It is proposed to signal a flag which indicates that each slice contains a complete sub-picture.

The proposed changes are shown highlighted compared with VVC WD.

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_5bits | u(5) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| gdr_enabled_flag | u(1) |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| single_slice_per_subpic_flag | u(1) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |

single_slice_per_subpic_flag equal to 1 specifies that each slice in the CVS contains one complete subpicture. single_slice_per_subpic_flag equal to 0 specifies that in the CVS, a subpicture may be split in one or more slices.

Alternatively, this flag may be signalled as a constraint flag as follows:

#### General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| general_progressive_source_flag | u(1) |
| general_interlaced_source_flag | u(1) |
| general_non_packed_constraint_flag | u(1) |
| general_frame_only_constraint_flag | u(1) |
| intra_only_constraint_flag | u(1) |
| max_bitdepth_constraint_idc | u(4) |
| max_chroma_format_constraint_idc | u(2) |
| frame_only_constraint_flag | u(1) |
| single_slice_subpic_only_constraint_flag | u(1) |
| no_qtbtt_dual_tree_intra_constraint_flag | u(1) |
| no_partition_constraints_override_constraint_flag | u(1) |
| no_sao_constraint_flag | u(1) |
| no_alf_constraint_flag | u(1) |
| no_joint_cbcr_constraint_flag | u(1) |
| no_ref_wraparound_constraint_flag | u(1) |
| no_temporal_mvp_constraint_flag | u(1) |
| no_sbtmvp_constraint_flag | u(1) |
| no_amvr_constraint_flag | u(1) |
| no_bdof_constraint_flag | u(1) |
| no_dmvr_constraint_flag | u(1) |
| no_cclm_constraint_flag | u(1) |
| no_mts_constraint_flag | u(1) |
| no_sbt_constraint_flag | u(1) |
| no_affine_motion_constraint_flag | u(1) |
| no_bcw_constraint_flag | u(1) |
| no_ibc_constraint_flag | u(1) |
| no_ciip_constraint_flag | u(1) |
| no_fpel_mmvd_constraint_flag | u(1) |
| no_triangle_constraint_flag | u(1) |
| no_ladf_constraint_flag | u(1) |
| no_transform_skip_constraint_flag | u(1) |
| no_bdpcm_constraint_flag | u(1) |
| no_qp_delta_constraint_flag | u(1) |
| no_dep_quant_constraint_flag | u(1) |
| no_sign_data_hiding_constraint_flag | u(1) |
| // ADD reserved bits for future extensions |  |
| while( !byte_aligned( ) ) |  |
| gci_alignment_zero_bit | f(1) |
| } |  |

single_slice_subpic_only_constraint_flag equal to 1 specifies that CVS includes one complete subpicture per slice. single_slice_subpic_only_constraint_flag equal to 0 does not impose a constraint.

## Proposal 2

It is asserted that the specification should support either explicit signalling of subpicture IDs for each subpicture or if not signalled, the subpicture IDs for subpictures should be derived. Also it is proposed that the subpicture ID value should be signalled in slice, to specify which subpicture the slice belongs to.

The proposed changes are shown highlighted compared with VVC WD.

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_5bits | u(5) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| gdr_enabled_flag | u(1) |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| signalled_subpicture_id_flag | u(1) |
| if(signalled_subpicture_id_flag ) |  |
| signalled_subpicture_id_length_minus1 | ue(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| if(signalled_subpicture_id_flag ) |  |
| subpicture_id[ i ] | u(v) |
| } |  |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |

signalled_subpicture_id_flag equal to 1 specifies that the subpicture ID for each subpicture is signalled. signalled_subpicture_id_flag equal to 0 specifies that subpicture IDs are not signalled and are inferred.

signalled_subpicture_id_length_minus1 plus 1 specifies the number of bits used to represent the syntax element subpicture_id[ i ] when present, and the syntax element subpicture_id_val in slice headers. The value of signalled_subpicture_id_length_minus1 shall be in the range of 0 to 15, inclusive. When not present,  the value of signalled_subpicture_id_length_minus1 is inferred to be equal to Ceil( Log2( Max( 2, NumSubPics) ) ) − 1.

subpicture_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the subpicture_id[ i ] syntax element is signalled_subpicture_id_length_minus1 + 1 bits.  When not present, the value of subpicture_id[ i ] is inferred to be equal to i, for each i in the range of 0 to NumSubPics − 1, inclusive.

### Slice header syntax

#### General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( subpics_present_flag ) |  |
| subpic_id_val | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| non_reference_picture_flag | u(1) |
| slice_type | ue(v) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| slice_pic_order_cnt_lsb | u(v) |
| if( nal_unit_type = = GDR_NUT ) |  |
| recovery_poc_cnt | ue(v) |
| if( nal_unit_type = = IDR_W_RADL ¦ ¦ nal_unit_type = = IDR_N_LP ¦ ¦ nal_unit_type = = CRA_NUT ¦ ¦ NalUnitType = = GDR_NUT ) |  |
| no_output_of_prior_pics_flag | u(1) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| … |  |
| } |  |

subpic_id_val specifies the subpicture ID of the subpicture the slice belongs to. The length of the subpic_id_val is signalled_subpicture_id_length_minus1 + 1 bits. If signalled_subpicture_id_flag is equal to 0, the value of subpic_id_val shall be in the range of 0 to NumSubPics− 1, inclusive. Otherwise, the value of subpic_id_val shall be in the range of 0 to 2( signalled_subpicture_id_length_minus1 + 1 ) − 1, inclusive.

## Proposal 3

A bug-fix is proposed for the for loop which signals subpic_treated_as_pic_flag[ i ], loop_filter_across_subpic_enabled_flag[ i ] syntax elements.

It is asserted that the for loop should be changed from

for( i = 0; i  <= NumSubPics; i++ ) { subpic_treated_as_pic_flag[ i ], loop_filter_across_subpic_enabled_flag[ i ]}

to

for( i = 0; i  < NumSubPics; i++ ) { subpic_treated_as_pic_flag[ i ], loop_filter_across_subpic_enabled_flag[ i ]}

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_5bits | u(5) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| gdr_enabled_flag | u(1) |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |

# Conclusion

This document proposes syntax for subpicture information signalling. It is proposed to adopt the proposals in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-O2001-vE, Versatile Video Coding (Draft 6), Gothenburg, July 2019