| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0582-v2 |
| --- | --- |

| Title: | AhG16: On dynamic internal bit depth |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yixin Du, Xin Zhao, Xiang Li, Liang Zhao and Shan Liu | Email: | {yixindu, xinzzhao, xlxiangli, leolzhao, shanl}@tencent.com |  |
| Source: | Tencent |  |  |  |

_____________________________

# Abstract

In VTM-6.0, fixed internal bit depth is implemented throughout all modules. When InternalBitDepth is greater than 8, 8-bit SIMD arithmetic instructions cannot be used. Since 8-bit SIMD instructions have higher throughput than 16-bit ones, this contribution proposes to use dynamic InternalBitDepth on certain modules which are not sensitive to bit depth. It is proposed to set InternalBitDepth to 8 in deblocking, intra Planar mode, and intra angular mode, regardless of which InternalBitDepth is used globally.

# Introduction

SIMD implementation utilizes 128-bit or 256-bit registers available in general-purpose CPUs to accomplish multiples 8-bit/16-bit/32-bit/64-bit arithmetic with one single instruction so the calculation can be done in parallel. For example, a 128-bit register can perform sixteen 8-bit or eight 16-bit additions/multiplications within one SIMD instruction. However, for 10-bit internal data, a 128-bit SIMD has to treat it as 16-bit data and can only perform eight 10-bit additions/multiplications within one SIMD instruction. Therefore, given a huge amount of calculations, 10-bit arithmetic is roughly 2x slower than 8-bit arithmetic using SIMD instructions, which is not efficient.

# Proposed Solution

In order to use 8-bit SIMD instructions, it is proposed to shift the input data of certain modules to 8-bit before SIMD instructions are executed. As shifting from larger bit depth such as 10-bit to 8-bit introduces coding loss, only certain modules which are less sensitive to InternalBitDepth are proposed to use the dynamic internal bit depth method. The modules include deblocking, intra Planar mode, and intra angular mode (fractional-pel).

# Experimental Results

The proposed method regarding intra prediction and deblocking were implemented on VTM-6.0 reference software [1] and simulations are performed under CTC [2], namely Test #1 and Test #2.

Table 1: Results under CTC with intra prediction planar and angular InternalBitDepth equals to 8 on VTM-6.0 (Test #1).

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 | 0.10% |  | 0.06% |  | 0.03% |  | 101% |  | 99% |
| Class A2 | 0.14% |  | -0.07% |  | 0.06% |  | 100% |  | 98% |
| Class B | 0.08% |  | 0.04% |  | 0.08% |  | 101% |  | 98% |
| Class C | 0.02% |  | -0.12% |  | 0.03% |  | 101% |  | 104% |
| Class E | 0.16% |  | -0.02% |  | 0.29% |  | 101% |  | 102% |
| Overall | 0.09% |  | -0.02% |  | 0.09% |  | 101% |  | 100% |
| Class D | 0.02% |  | -0.01% |  | -0.19% |  | 101% |  | 103% |
| Class F | -0.01% |  | 0.09% |  | -0.13% |  | 100% |  | 100% |
| Class TGM | 0.01% |  | -0.01% |  | 0.00% |  | 100% |  | 97% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 | 0.11% |  | 0.42% |  | -0.06% |  | 100% |  | 100% |
| Class A2 | 0.11% |  | 0.31% |  | 0.24% |  | 100% |  | 100% |
| Class B | 0.05% |  | 0.23% |  | 0.21% |  | 100% |  | 99% |
| Class C | 0.01% |  | 0.13% |  | 0.04% |  | 101% |  | 103% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.06% |  | 0.26% |  | 0.12% |  | 100% |  | 100% |
| Class D | -0.04% |  | -0.04% |  | 0.09% |  | 100% |  | 101% |
| Class F | 0.03% |  | -0.02% |  | 0.14% |  | 100% |  | 100% |
| Class TGM | -0.01% |  | -0.03% |  | 0.01% |  | 101% |  | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.04% |  | 0.20% |  | -0.16% |  | 100% |  | 100% |
| Class C | -0.04% |  | -0.13% |  | -0.16% |  | 100% |  | 94% |
| Class E | -0.07% |  | 0.85% |  | 1.14% |  | 100% |  | 95% |
| Overall | -0.01% |  | 0.25% |  | 0.16% |  | 100% |  | 97% |
| Class D | 0.03% |  | 0.12% |  | 0.33% |  | 100% |  | 100% |
| Class F | 0.04% |  | 0.10% |  | 0.89% |  | 100% |  | 98% |
| Class TGM | -0.02% |  | -0.01% |  | 0.03% |  | 100% |  | 99% |

Table 2: Results under CTC with deblocking InternalBitDepth equals to 8 on VTM-6.0 (Test #2).

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.29% |  | 2.49% |  | 1.74% |  | 100% |  | 100% |  |
| Class A2 | 0.18% |  | 1.70% |  | 1.03% |  | 99% |  | 98% |  |
| Class B | 0.16% |  | 1.12% |  | 1.55% |  | 100% |  | 97% |  |
| Class C | 0.09% |  | 0.42% |  | 0.54% |  | 100% |  | 100% |  |
| Class E | 0.18% |  | 1.64% |  | 2.12% |  | 100% |  | 101% |  |
| Overall | 0.17% |  | 1.38% |  | 1.37% |  | 100% |  | 99% |  |
| Class D | 0.07% |  | 0.40% |  | 0.36% |  | 101% |  | 103% |  |
| Class F | 0.12% |  | 0.40% |  | 0.54% |  | 100% |  | 98% |  |
| Class TGM | 0.04% |  | 0.17% |  | 0.16% |  | 100% |  | 97% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.34% |  | 4.98% |  | 2.42% |  | 99% |  | 100% |  |
| Class A2 | 0.39% |  | 2.34% |  | 1.45% |  | 99% |  | 100% |  |
| Class B | 0.21% |  | 1.71% |  | 2.61% |  | 100% |  | 98% |  |
| Class C | 0.14% |  | 0.55% |  | 0.85% |  | 101% |  | 101% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.25% |  | 2.18% |  | 1.87% |  | 100% |  | 100% |  |
| Class D | 0.04% |  | 0.61% |  | 0.91% |  | 101% |  | 103% |  |
| Class F | 0.21% |  | 0.60% |  | 0.82% |  | 101% |  | 101% |  |
| Class TGM | 0.03% |  | 0.17% |  | 0.13% |  | 100% |  | 98% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.40% |  | 1.38% |  | 1.51% |  | 100% |  | 101% |  |
| Class C | 0.27% |  | 0.72% |  | 0.87% |  | 101% |  | 94% |  |
| Class E | 1.55% |  | 3.55% |  | 4.41% |  | 100% |  | 95% |  |
| Overall | 0.64% |  | 1.70% |  | 2.02% |  | 100% |  | 97% |  |
| Class D | 0.18% |  | 0.66% |  | 0.81% |  | 100% |  | 96% |  |
| Class F | 0.44% |  | 0.62% |  | 1.36% |  | 101% |  | 97% |  |
| Class TGM | 0.03% |  | 0.11% |  | 0.09% |  | 100% |  | 99% |  |

# Conclusion

This contribution proposed a dynamic internal bit depth method for deblocking, intra Planar mode and angular mode. Regardless of the global InternalBitDepth, it is proposed to use InternalBitDepth 8 in the aforementioned modules.

# References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-6.0.

- F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-5.0

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).