| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0584-v2 |
| --- | --- |

| Title: | AHG-18: BDPCM for Luma and Chroma in lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to enable BDPCM for Luma and Chroma in lossless coding. Proposed method has been integrated on top of CE8-4.1 source code. The coding performance comparing to AHG18 reference software is -0.79%, -0.17 % and z.zz% for AI, RA, and LB, respectively.

Note that simulations for classTGM for LB are not finished yet. It will be updated in the next revision.

Introduction

The current design of VVC supports BDPCM and Transform Skip, which are only applied to luma component. To make the design of VVC consistency for luma and chroma, CE8-4.1 proposed to allow BDCPM and Transform skip to be used on the chroma components, which is a combination of CE8-3.1 (JVET-O0166 [1]) and CE8-2.1 (JVET-O0081[2]).

Although, the AHG18 reference software [3] supports lossless coding with minimum changes, it doesn’t support BDPCM in lossless coding. Therefore, coding performance of BDPCM for luma and chroma is studied using CE8-4.1 source code for lossless coding.

Proposal

This contribution proposes to enable BDPCM for luma and chroma with TS residual coding in lossless coding. Changes relative to CE8-4.1 [4] are as below:

- TS residual coding is used for lossless coding when BDPCM is used.

- Non-TS residual coding is used for lossless coding when BDPCM is not used.

- Omit signaling transform_skip_flag when cu_transquant_bypass_flag is equal to 1.

Note that code changes for AHG18 reference software relative to VTM-6.0 are applied to CE8-4.1.

Simulation Results

The proposed method has been implemented on top of CE8-4.1 source code and following simulations were conducted under AHG18 test condition:

- Test1:	AHG18 anchor* vs CE8-4.1 with lossless coding support*

*: For lossless coding, following command options are used:

--TransquantBypassEnable=1 --CUTransquantBypassFlagForce=1 --CostMode=lossless --QP=0

Table 3-1: Coding performance relative to AHG18 anchor SW

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | AHG18 | Tested |  |
| Class A1 | 2.2 | 2.2 | -0.59% |
| Class A2 | 1.7 | 1.7 | -0.26% |
| Class B | 2.1 | 2.1 | -0.45% |
| Class C | 2.1 | 2.1 | -0.93% |
| Class D | 1.9 | 2.0 | -2.07% |
| Class E | 2.9 | 3.0 | -1.92% |
| Class F | 5.5 | 6.7 | -3.72% |
| TGM | 12.3 | 13.0 | -4.91% |
| Overall | 2.2 | 2.2 | -0.79% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 92% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | AHG18 | Tested |  |
| Class A1 | 2.3 | 2.3 | -0.23% |
| Class A2 | 1.9 | 1.9 | -0.10% |
| Class B | 2.3 | 2.3 | -0.11% |
| Class C | 2.6 | 2.6 | -0.27% |
| Class D | 2.8 | 2.8 | -0.44% |
| Class E |  |  |  |
| Class F | 39.4 | 40.7 | -1.89% |
| TGM | 112.6 | 114.8 | -1.83% |
| Overall | 2.3 | 2.3 | -0.17% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 97% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | AHG18 | Tested |  |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B | 2.3 | 2.3 | -0.12% |
| Class C | 2.6 | 2.6 | -0.28% |
| Class D | 2.7 | 2.8 | -0.38% |
| Class E | 3.3 | 3.3 | -0.44% |
| Class F | 60.7 | 61.8 | -1.39% |
| TGM | 142.4 |  |  |
| Overall | 2.6 |  |  |
| Enc Time[%] |  |  |  |
| Dec Time[%] |  |  |  |

Conclusion

For lossless coding, it is recommended to adopt BDPCM for luma and chroma into the next VTM/VVC WD or further study in CE.

Reference

[1] F. Henry, G. Clare, “CE8-related: BDPCM for chroma,” JVET-O0166, July 2019

[2] T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki, “CE8-3.2: Chroma Transform Skip,” JVET-O0081, July 2019

[3] https://vcgit.hhi.fraunhofer.de/jvet-o-ahg-18/VVCSoftware_VTM

[4] G. Clare, F. Henry, T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki, “CE8-4.1: BDPCM and Transform skip for Chroma,” JVET-P0052, October 2019

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Spec changes relative to JVET-P0059

Spec text is available in CE-8.4.1 in JVET-P0059. Some syntax is reproduced below considering the cu_transquant_bypass_flag.

7.3.2.3 Sequence parameter set RBSP syntax

| … | u(1) |
| --- | --- |
| if ( sps_bdpcm_enabled_flag && cu_transquant_bypass_flag ) |  |
| sps_bdpcm_chroma_enabled_flag | u(1) |
| … | u(1) |

- 7.3.8.11 Transform mode syntax

| transform_mode( x0, y0, tbWidth, tbHeight, cIdx ) { | Descriptor |
| --- | --- |
| if (sps_transform_skip_enabled_flag && (tbWidth <= MaxTsSize) && (tbHeight <= MaxTsSize) && !(BdpcmFlag[ x0 ][ y0 ] && cIdx==0) && !(BdpcmChromaFlag[ x0 ][ y0 ] && cIdx!=0) && (IntraSubPartitionsSplit[ x0 ][ y0 ] == ISP_NO_SPLIT ¦¦ cIdx!=0) && ( !cu_sbt_flag ) ) && !cu_transquant_bypass_flag |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if ( (cIdx==0) && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) && ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) && !trasnfrom_skip_flag[x0][y0][cIdx] ) |  |
| mts_idx [ x0 ][ y0 ][ cIdx ] | ae(v) |
| } |  |

- 7.4.9.11 Transform mode semantics

transform_skip_flag[ x0 ][ y0 ][ cIdx ] specifies whether a transform is applied to the associated transform block with cIdx or not. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. transform_skip_flag[ x0 ][ y0 ][ cIdx ] equal to 1 specifies that no transform is applied to the associated transform block with cIdx. transform_skip_flag[ x0 ][ y0 ] equal to 0 specifies that the decision whether transform is applied to the transform block or not depends on other syntax elements.

When transform_skip_flag[ x0 ][ y0 ][ cIdx ] is not present, it is inferred as follows:

- If BdcpmFlag[ x0 ][ x0 ] is equal to 1 and cIdx is equal to 0, transform_skip_flag[ x0 ][ y0 ] is inferred to be equal to 1.

- Otherwise if BdcpmFlag[ x0 ][ x0 ] is equal to 0 and cIdx is equal to 0), transform_skip_flag[ x0 ][ y0 ][ cIdx ] is inferred to be equal to 0.

- Othewise if BdpcmChromaFlag[ x0 ][ y0 ] is equal to 1 and cIdx is not equal to 0, transform_skip_flag[ x0 ][ y0 ][ cIdx ] is inferred to be equal to 1.

- Otherwise ( BdpcmChromaFlag[ x0 ][ y0 ] is equal to 0 and cIdx is not equal to 0, or cu_transquant_bypass_flag is equal_to 1), transform_skip_flag[ x0 ][ y0 ][ cIdx ] is inferred to be equal to 0.