| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0600 |
| --- | --- |

| Title: | CE4-related: Support large rotation and flipping in affine and PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alireza Aminlou Nokia Technologies, Tampere, Finland | Email: | alireza.aminlou@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Current syntax for affine motion vector supports signalling large rotation (e.g. close to 90/180/270 degree) and flipping, but engine of affine motion compensation and PROF are not working optimally for these cases. In these cases, there is a big discontinuity in borders of predicted subblocks, and PROF increases this discontinuity furthermore. This contribution proposes two solutions to fix these problems. At the first step, large rotation and flipping are detected based on affine motion vectors, and eight processing modes are defined for the current block. Then, based on the processing mode, 1) during PROF, motion vector refinements are modified, and/or 2) after performing subblock-based motion compensation, sampels of the motion compensated subblocks are rotated or flipped.

# Introduction/Problem statement

In the current working draft of VVC and in VTM6.0, there are 4-parameter and 6-parameter affine prediction modes. With 4- and 6-parameter affne, it is possible to singal a large rotation, for example around 90, 180, or 270 degree rotation. With 6-parameter affine, flipping (in four different direction) can also be indicated. This type large rotation may happen in some of video content. For example, in CatRobot test sequence, there is a rotationg object. As another example, in 3x2 layout of cubemap projection, when a content goes from one face to another face, it may get rotated by 90 degree. But current subblock-based affine motion compensation and PROF method fails when affine parameters indicate large rotation or flipping.

## Subblock-based affine

In subblock-based motion compensation, each subblock has only translational motion vector which are derived from affine motion vector of the current block. So, in the case of large rotation or flipping, there is a big discontinuity in borders of motion compensated subblocks. Figure 1 illustrates an example where the object in the current frame is rotated by 90 degree clockwise with respect to the reference frame. This 90 degree rotation can be signalled using 4-parameter affine as shows in the figure. But the generated prediction block using subblock-based affine motion compensation has big discontinty in subblock border, and is not a usefull prediction to predict the current block. Another example is illustrated in Figure 2 for the case of vertical flipping.

Figure 1. Subblock-based affine motion compensation in the case of 90-degree rotaion clockwise

Figure 2. Subblock-based affine motion compensation in the case of vertical flipping

## PROF

The assumption behind PROF method is that motion vector refinemets are small (see Figure 3-(a)), so samples of motion compensated subblock can be refined using motion vector refinements and gradient of the sampels of the subblocks. But in the case of large rotation and flipping, delta of affine motion vectors are large (see Figure 3-(b)), and motion vector refinements have large values. As a result, the abovementioed assumption does not hold, and applying PROF increase discontinuity in borders of subblocks.

| (a) | (b) |  |
| --- | --- | --- |

Figure 3. Motion vectore refinemts in PROF, (a) in the case of small rotation (e.g. 20 degree), (b) in the case of large rotation (e.g. 80 degree)

# Proposal

This contribution proposes two solutions to fix subblock-based motion compensation and PROF problems. In the following table, the proposed process for affine motion compensation is briefly compared to that of original ones in VTM6.0, and additional steps are highlighted. In the initializaiton step, first delt motion vectors (DMV) are calculated based on affine motion vectors (i.e. motion vectors of different corner). Then a processing mode is determined based on DMV, and then a secondary DMV (DMV2) is calculated. Then for each subblock, a MV is calculated, motion compensation is executed, PROF is applied, and then predicted/refined subblock is rotated/flipped based on the processin mode.

Table 1. Origianl affine motion compensation versus the proposed method

| Step | VTM6.0 | Proposed method |
| --- | --- | --- |
| Initializatoin | Calculate DMV | Calculate DMV |
|  |  | Determine a processing mode based on DMV |
|  |  | Calculate DMV2 based on processing mode |
|  | Calcualte MVR using DMV | Calcualte MVR using DMV2 |
| For each subblock | Calcuate subblock MV using DMV | Calcuate subblock MV using DMV |
|  | Do motion compensation | Do motion compensation |
|  | Apply PROF using MVR | Apply PROF using MVR |
|  |  | Rotate/Flip pedicted subblock-based on the processing mode |

## Flipping and rotation detection

At the first step, which is common for both solutions, large rotation and flipping are detected based on the affine motion vectors of the current block. For this purpose, as shown in Figure 4, based on affine motion vectors

- first, flipping is dected by examing the outer product of AB and AC vectors after affine transform, and

- second, rotation is detected by examing the location of (1,1) sample location with respect to the location of top-left corner after affine transformation.

| (a) | (b) | (c) |
| --- | --- | --- |

Figure 4. Detecting flipping and rotation, (a) current block, (b) prediction block after affine transform in the case of flipping (e.g. horizontal flipping), (c) prediction block after affine transform in the case of rotation (e.g. around 80 degree clockwise)

In this proposal, we have defined two modes of flipping (i.e. flipped or not flipped), and four modes of rotation (i.e. around 0/90/180/270 degree). Each mode of rotation coverl +-45 degree around the main angle. For example, rotation mode of 90 degree cover rotation angle in the range of {45, 135} degree.

Outer product of AB and AC curve is calculated using the following equation:

F=(iDMvHorX+(1<<(iBit+4+iScaleX)))*(iDMvVerY+(1<<(iBit+4+iScaleX)))-(iDMvVerX*iDMvHorY)

(1)

and the location of (Ex, Ey) is calculated as below:

Ex = (iDMvHorX + iDMvVerX) + (1 << (iBit+4+iScaleX))

Ey = (iDMvHorY + iDMvVerY) + (1 << (iBit+4+iScaleY))

So, eight different processing modes are defined based on the combination of flipping and rotation as presented in table below:

Table 2. Different combinations of flipping and rotation

| F | Ex | Ey | Processing mode | W' | H' |
| --- | --- | --- | --- | --- | --- |
| >0 | >0 | >0 | No extra operation | W | H |
| >0 | >0 | ≤0 | Rotate 90 degrees | H | W |
| >0 | ≤0 | ≤0 | Rotate 180 degrees | W | H |
| >0 | ≤0 | >0 | Rotate 270 degrees | H | W |
| ≤0 | >0 | >0 | Flip Diagonal1 | H | W |
| ≤0 | >0 | ≤0 | Flip vertically | W | H |
| ≤0 | ≤0 | ≤0 | Flip Diagonal2 | H | W |
| ≤0 | ≤0 | >0 | Flip Horizontally | W | H |

## Calculation of motion vectore refinement in PROF

In the next step, motion vector refinement for PROF is calculated according to the processing mode. The main idea here is to calculate the refined sample using the predicted sample which has the closest location. This will reduce the value of motion vectore refinements.

| (a) | (b) |  |
| --- | --- | --- |

In this proposal, based on the processing mdoe for the current block, a secondary delta motion vector is calculated using the orginal delta motion vector. Then motion vector refinements are derived based on the original PROF method using the secondary delta motion vector.

In current VVC draft (VTM6.0), delta motion vectors are calculated as below:

iDMvHorX = (mvRT - mvLT).getHor() << (iBit - g_aucLog2[cxWidth]);

iDMvHorY = (mvRT - mvLT).getVer() << (iBit - g_aucLog2[cxWidth]);

if ( pu.cu->affineType == AFFINEMODEL_6PARAM )

{

iDMvVerX = (mvLB - mvLT).getHor() << (iBit - g_aucLog2[cxHeight]);

iDMvVerY = (mvLB - mvLT).getVer() << (iBit - g_aucLog2[cxHeight]);

}

else

{

iDMvVerX = -iDMvHorY;

iDMvVerY = iDMvHorX;

}

According to this proposal, secondary delta motion vectors are calculated as below:

Table 3. Different combinations of flipping and rotation

| F | Ex | Ey | Processing mode | Secondary delta motion vector (DMV2) |
| --- | --- | --- | --- | --- |
| >0 | >0 | >0 | Normal case | DMvHorX2 = +DMvHorX DMvHorY2 = +DMvHorY DMvVerX2 = +DMvVerX DMvVerY2 = +DMvVerY |
| >0 | >0 | ≤0 | Rotate 90 | DMvHorX2 = +DMvVerX - (1<<(iBit+mvBits)) DMvHorY2 = +DMvVerY + (1<<(iBit+mvBits)) DMvVerX2 = -DMvHorX - (1<<(iBit+mvBits)) DMvVerY2 = -DMvHorY - (1<<(iBit+mvBits)) |
| >0 | ≤0 | ≤0 | Rotate 180 | DMvHorX2 = -DMvHorX - (2<<(iBit+mvBits)) DMvHorY2 = -DMvHorY + (0<<(iBit+mvBits)) DMvVerX2 = -DMvVerX + (0<<(iBit+mvBits)) DMvVerY2 = -DMvVerY - (2<<(iBit+mvBits)) |
| >0 | ≤0 | >0 | Rotate 270 | DMvHorX2 = -DMvVerX - (1<<(iBit+mvBits)) DMvHorY2 = -DMvVerY - (1<<(iBit+mvBits)) DMvVerX2 = +DMvHorX + (1<<(iBit+mvBits)) DMvVerY2 = +DMvHorY - (1<<(iBit+mvBits)) |
| ≤0 | >0 | >0 | Flip Diagonal1 | DMvHorX2 = +DMvVerX - (1<<(iBit+mvBits)) DMvHorY2 = +DMvVerY + (1<<(iBit+mvBits)) DMvVerX2 = +DMvHorX + (1<<(iBit+mvBits)) DMvVerY2 = +DMvHorY - (1<<(iBit+mvBits)) |
| ≤0 | >0 | ≤0 | Flip vertically | DMvHorX2 = +DMvHorX DMvHorY2 = +DMvHorY DMvVerX2 = -DMvVerX DMvVerY2 = -DMvVerY - (2<<(iBit+mvBits)) |
| ≤0 | ≤0 | ≤0 | Flip Diagonal2 | DMvHorX2 = -DMvHorX - (2<<(iBit+mvBits)) DMvHorY2 = -DMvHorY DMvVerX2 = +DMvVerX DMvVerY2 = +DMvVerY |
| ≤0 | ≤0 | >0 | Flip Horizontally | DMvHorX2 = -DMvVerX - (1<<(iBit+mvBits)) DMvHorY2 = -DMvVerY - (1<<(iBit+mvBits)) DMvVerX2 = -DMvHorX - (1<<(iBit+mvBits)) DMvVerY2 = -DMvHorY - (1<<(iBit+mvBits)) |

And then motion vector refinements are calculated using seconday delta motion vectors using the process defined in the original PROF. It should be noted that this secondary delta motion vectors are used only in PROF, not in motion vector derivation for subblocks.

## Rotation/Flipping of predicted subblock

Finally, sampels of subblock are rotated or flipped according to the processing mode.

# Simulation results

## Fix affine and fix PROF

Encoding was executed by “ENABLE_TRACING = 1”. Decoding was executed when the cluster was full and busy, so reported decoder time is a bit over estimated. New timing results will be provided later.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01 % | -0.01 % | 0.03 % | 112 % | 101 % |
| Class A2 | 0.02 % | 0.03 % | 0.03 % | 112 % | 102 % |
| Class B | -0.01 % | 0.10 % | 0.04 % | 112 % | 102 % |
| Class C | -0.02 % | 0.08 % | 0.05 % | 114 % | 102 % |
| Class E |  |  |  |  |  |
| Overall | -0.01 % | 0.06 % | 0.04 % | 113 % | 102 % |
| Class D | -0.01 % | 0.01 % | 0.09 % | 114 % | 102 % |
| Class F | 0.03 % | 0.05 % | 0.03 % | 113 % | 102 % |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.04 % | -0.06 % | 0.11 % | 112 % | #NUM! |
| Class E | -0.01 % | 0.51 % | 0.24 % | 108 % | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01 % | 0.33 % | 0.32 % | 112 % | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

The method in this proposal enhances the functionality of affine and fix PROF for the cases that there are large rotation or flipping in video contents. It almost does not change the RD performance of the codec under CTC, and it is expected to provide RD performance gain in special cases like coding 360-degree content in 3x2 layout of cubemap projection.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).