| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0601-v2 |
| --- | --- |

| Title: | Non-CE4: Support slice level disabling for PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo Masaru Ikeda 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@sony.com, Masaru.Ikeda@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to support slice level disabling for PROF. And, it’s reported that PROF brings visual artifacts in the specific frame. In the current VTM design, PROF doesn’t have a block level control flag, and it is used implicitly when Affine enable flag and PROF enable flag are enabled in SPS. It makes it difficult to avoid visual artifacts in slice level. In this contribution, slice level control flags are proposed to avoid the visual artifacts.

Problem statement

A technique, prediction refinement with optical flow (PROF) was adopted in last JVET meeting [1]. In the PROF process, a pixel level refinement is applied with optical flow after subblock level Affine motion compensation. Here, this refinement is not applied for chroma to reduce complexity. Therefore, a misalignment exists between luma and chroma.

Figure 1 is an example that is cropped from the test sequence “CatRobot”. The frame number is 16 that is encoded based on a CTC test condition, RA and QP 37.

Figure 1: An example of visual artifact (CatRobot, frame number 16)

The chroma components in Figure 1, Cb and Cr are shown in Figure 2 and Figure 3. As we can see, serrated visual artifacts are observed at the color boundary.

Figure 2: Cb component

Figure 3: Cr component

As well as in CatRobot, visual artifacts are observed in the other test sequence, SlideShow, frame number 72. It’s shown in Figure 4, and Figure 5 and Figure 6 show the chroma component.

Figure 4: An example for visual artifact (SlideShow, frame number 72)

Figure 5: Cb component

Figure 6: Cr component

If PROF is disabled using the SPS flag, it is expected that the above visual artifacts disappear. However, also it will bring some coding efficiency loss. Table 1 shows the BD-rate for RA case when PROF was disabled. For “CatRobot”, the BD-rates are 1.58%, -0.16% and -0.05% for Y, U and V. For “SlideShow”, the BD-rates are 3.06%, 1.85% and 1.74% for Y, U and V.

Table 1: BD-rate of PROF disabling

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | 0.04% | -0.05% |  | 98% |
| Class A2 | 0.88% | -0.03% | 0.06% |  | 98% |
| Class B | 0.59% | 0.38% | 0.16% |  | 99% |
| Class C | 0.30% | 0.12% | 0.12% |  | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.48% | 0.16% | 0.09% |  | 99% |
| Class D | 0.57% | 0.13% | 0.29% |  | 100% |
| Class F | 0.89% | 0.48% | 0.47% |  | 100% |

Figure 7 shows the changes of visual quality between PROF on/off. Cb and Cr components are shown at middle and bottom. As we can see, serrated artifacts disappeared when PROF-off (right).

Figure 7: PROF-ON (left) and PROF-OFF (right)

Proposed method

To also support PROF for chroma component is a straightforward way to solve above problem. In last JVET meeting, also PROF for chroma was proposed in a contribution JVET-O0583 [2]. However, it was not adopted due to increased complexity.

This visual artifact occurs in a specific frame that the motion of object has rotation and a long-time distance such as the frame number 16 in RA case. This contribution proposes the other way that an encoder controls PROF on/off in slice level, and proposes to add slice level flags for PROF. The syntax changes are described in below subclause. The detail draft text, the file is included in this contribution.

To control PROF on/off, this contribution proposes to add two flags that are sps_affine_prof_slice_present_flag and slice_disable_affine_prof_flag. Those semantics are described in below subclause. This approach is similar to sps_bdof_dmvr_slice_present_flag　and slice_disable_bdof_dmvr_flag that was proposed in [3].

Text changes

Sequence parameter set RBSP semantics

sps_affine_prof_slice_present_flag equal to 1 specifies that slice_disable_affine_prof_flag is present in slice headers referring to the SPS. sps_affine_prof_slice_present_flag equal to 0 specifies that slice_disable_affine_prof_flag is not present in slice headers referring to the SPS. When sps_affine_prof_slice_present_flag is not present, the value of sps_affine_prof_present_flag is inferred to be equal to 0.

Slice header semantics

slice_disable_affine_prof_flag equal to 1 specifies that the prediction refinement with optical flow for affine motion compensation is disabled in the current slice. slice_disable_affine_prof_flag equal to 0 specifies that the prediction refinement with optical flow for affine motion compensation may or may not be enabled in the current slice. When slice_disable_affine_prof_flag is not present, the value of slice_disable_affine_prof_flag is inferred to be 0.

Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_bdof_enabled_flag | u(1) |
| sps_smvd_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| if( sps_bdof_enabled_flag ¦ ¦ sps_dmvr_enabled_flag) |  |
| sps_bdof_dmvr_slice_present_flag | u(1) |
| … | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| sps_affine_type_flag | u(1) |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| if( sps_affine_prof_enabled_flag ) |  |
| sps_affine_prof_slice_present_flag | u(1) |
| } |  |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … | u(1) |
| if ( slice_type != I ) { |  |
| … |  |
| if( sps_bdof_dmvr_slice_present_flag ) |  |
| slice_disable_bdof_dmvr_flag | u(1) |
| if( sps_affine_prof_slice_present_flag ) |  |
| slice_disable_affine_prof _flag | u(1) |
| … |  |
| } |  |
| … |  |
| byte_alignment( ) |  |
| } |  |

Conclusion

This contribution shows that visual artifacts occur when PROF is applied. To avoid the visual artifacts, it’s proposed to support slice level disabling for PROF.

References

- Jiancong (Daniel) Luo, Yuwen He, “CE4: Prediction refinement with optical flow for affine mode (Test 2.1)”, JVET-O0070, Gothenburg, SE, July 2019.

- Yuwen He, Wei Chen, “CE4-related: Prediction refinement with optical flow for chroma components for affine mode”, JVET-O0583, Gothenburg, SE, July 2019.

- Yong He, Yuwen He, Ahmed Hamza, Han Huang, Wei-Jung Chien, Vadim Seregin, Muhammed Coban,Marta Karczewicz, Sriram Sethuraman , “Unified syntax for JVET-O0184/O0250/O0504 on DMVR and BDOF flag” , JVET-O1140, Gothenburg, SE, July 2019.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).