| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0619-v2 |
| --- | --- |

| Title: | Non-CE7: State-dependent Binarization for Transform Coefficient Levels |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Heiko Schwarz, Anastasia Henkel, Detlev Marpe, Thomas Wiegand | Email: | heiko.schwarz@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes to select the binarization of transform coefficient levels depending on the quantization state.  It only affects transform coefficient coding for dependent quantization; it does not modify the transform coefficient coding for conventional quantization.  It is asserted that the proposed change reduces the bit-to-bit ratio by about 5%, which has the effect that the bin-to-bit ratio for dependent quantization becomes very similar to the bin-to-bit ratio for conventional quantization. In addition to the VTM-6 binarization, a second binarization is used, in which the first bin indicates whether the absolute value is equal to 1. Two variants are presented. In variant A, the binarization is only dependent on the quantization state. In variant B, it depends on the state and on whether the sum of absolute levels inside the template is equal or unequal to zero.  It is asserted that variant B achieves a slightly larger reduction of the bin-to-bit ratio. For the common test conditions, the following results are reported:

Variant A:
	AI:		-0.03%,  -0.07%,  -0.16% (Y,Cb,Cr);  max/avg. bin-to-bit ratio of -4.84% / -4.73%
	RA:	 0.05%,  -0.07%,  -0.13% (Y,Cb,Cr);  max/avg. bin-to-bit ratio of -5.40% / -4.78%
	LB:		 0.07%,    0.02%,   0.15% (Y,Cb,Cr);  max/avg. bin-to-bit ratio of -5.09% / -5.65%

Variant B:
	AI:		-0.04%,  -0.22%,  -0.27% (Y,Cb,Cr);  max/avg. bin-to-bit ratio of -5.28% / -5.12%
	RA:	 0.03%,  -0.01%,  -0.16% (Y,Cb,Cr);  max/avg. bin-to-bit ratio of -5.62% / -5.03%
	LB:		 0.04%,  -0.04%,  -0.16% (Y,Cb,Cr);  max/avg. bin-to-bit ratio of -5.84% / -6.32%

It is asserted that both proposed variants prevent the insertion of cabac_zero_word’s in the bitstreams generated according to the common test conditions and the low QP configuration.

# Introduction

It has been reported that the bin-to-bit ratio for the VTM is roughly 20% higher than that for HM [1][2], for the common test conditions.  One reason is the usage of dependent quantization. By enabling dependent quantization, the bin-to-bit ratio is increased by roughly 5%.  This is demonstrated by the simulation results in Table 1, which summarizes the coding efficiency impact as well as the impact on the weighted bin-to-bit ratio (see JVET-P0050 [2]) by enabling dependent quantization for the standard QP range (22, 27, 32, 37) and the low QP range (2, 7, 12, 17).  In these experiments, the reference is VTM-6 with dependent quantization disabled and sign data hiding enabled. In the weighted bin-to-bit ratio, context-coded bins are weighted by 1 and EP bins are weighted by 0.25 (same as in JVET-P0050 [2]).

Table 1:  BD rates and changes of the weighted bin-to-bit ratio for VTM-6 compared to VTM-6 with dependent quantization disabled and sign data hiding enabled, for common test conditions (CTC) and low QP values (2, 7, 12, 17).

| CTC | Over VTM-6, DQ=0, SDH=1 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| AI | -1.98% | 0.86% | 0.92% | 1.08 | 1.14 | 5.21% | 1.08 | 1.14 | 5.01% | 105% | 96% |
| RA | -1.73% | 0.27% | 0.63% | 1.07 | 1.13 | 5.37% | 1.14 | 1.19 | 4.23% | 101% | 99% |
| LB | -1.31% | -1.46% | -1.42% | 1.07 | 1.13 | 5.63% | 1.17 | 1.22 | 4.46% | 101% | 99% |

| lowQP | Over VTM-6, DQ=0, SDH=1 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| AI | -1.43% | -2.58% | -2.63% | 0.90 | 0.93 | 3.88% | 0.90 | 0.94 | 3.90% | 120% | 92% |
| RA | -2.45% | -3.63% | -2.98% | 0.83 | 0.86 | 3.07% | 1.04 | 1.10 | 5.48% | 118% | 94% |
| LB | -2.44% | -5.42% | -4.67% | 0.88 | 0.92 | 3.90% | 1.06 | 1.12 | 6.24% | 118% | 97% |

In this contribution, we propose a method for reducing the bin-to-bit ratio for dependent quantization by using a state dependent binarization of transform coefficient levels.  It only modifies the coding of transform coefficient levels for dependent quantization; the coding of transform coefficient levels for conventional quantization is not modified.  As will be shown below, this change reduced the weighted bin-to-bit ratio for dependent quantization by roughly 5%.  As a result of the proposed change, dependent quantization requires roughly the same bit-to-bit ratio as conventional quantization.

# Proposed Method

## State-dependent Binarization

We propose a transform coefficient coding method that uses the two binarizations shown in Table 2.

Table 2:  Proposed binarizations. Binarization A is the same as the one used in VTM-6 and binarization B represents an alternative binarization, which differs from binarization A in the first two bins.

|  | binarization A (same as in VTM-6) |  |  |  |  | binarization B (differs in first 2 bins) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ¦q¦ | sig | gt1 | par | gt3 | rem | eq1 | gt1b | par | gt3 | rem |
| 0 | 0 | - | - | - | - | 0 | 0 | - | - | - |
| 1 | 1 | 0 | - | - | - | 1 | - | - | - | - |
| 2 | 1 | 1 | 0 | 0 | - | 0 | 1 | 0 | 0 | - |
| 3 | 1 | 1 | 1 | 0 | - | 0 | 1 | 1 | 0 | - |
| 4 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |
| 5 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 |
| 6 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 1 |
| 7 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |
| 8 | 1 | 1 | 0 | 1 | 2 | 0 | 1 | 0 | 1 | 2 |
| 9 | 1 | 1 | 1 | 1 | 2 | 0 | 1 | 1 | 1 | 2 |
| 10 | 1 | 1 | 0 | 1 | 3 | 0 | 1 | 0 | 1 | 3 |
| 11 | 1 | 1 | 1 | 1 | 3 | 0 | 1 | 1 | 1 | 3 |
| 12 | 1 | 1 | 0 | 1 | 4 | 0 | 1 | 0 | 1 | 4 |
| 13 | 1 | 1 | 1 | 1 | 4 | 0 | 1 | 1 | 1 | 4 |
| 14 | 1 | 1 | 0 | 1 | 5 | 0 | 1 | 0 | 1 | 5 |
| 15 | 1 | 1 | 1 | 1 | 5 | 0 | 1 | 1 | 1 | 5 |
| … | … | … | … | … | … | … | … | … | … | … |

The first binarization A is the same as the binarization used in the VTM-6.  The second binarization B, differs from binarization A in the first two bins (marked by colors).  When this binarization is used, the first bin eq1_flag (equal-to-one flag) indicates whether the transform coefficient level has an absolute value of one.  If eq1_flag is equal to 0, the second bin gt1b_flag indicates whether the absolute value is greater than one. If gt1b_flag is equal to 1, the remaining bins are the same as in binarization A (for gt1_flag equal to 1).

The usage of these two binarizations is motivated by the observation that the distribution of transform coefficient levels highly depends on the quantization state.  For states 0 and 1 (where the first quantizer is used), the absolute value of zero has the largest probability.  However, for states 2 and 3 (where the second quantizer is used), the absolute value of one has the highest probability.  This fact can be used for reducing the number of coded bins and, hence, the bin-to-bit ratio.

For the selection of the binarization, we propose two variants:

- Variant A:
The binarization only depends on the quantization state. For states 0 and 1, the binarization A (i.e., the VTM-6 binarization) is used, and for states 2 and 3, the alternative binarization B is used

if( state > 1 )
	use binarization B
else
	use binarization A

- Variant B:
In the second variant, the selected binarization additionally depends on whether there are any non-zero levels in the local template. Note the sum of absolute values sumAbs (of partially reconstructed levels after the first pass) is already used in the context modelling for the sig_flag, so that no additional data need to be calculated:

if( state > 1 && sumAbs > 0 )
	use binarization B
else
	use binarization A

It should be noted that the adaptive binarization only effects the first two bins (sig_flag and gt1_flag for binarization A; eq1_flag and gt1b_flag for binarization B). The remaining syntax elements par_flag, gt3_flag, and the remainder are coded in the same way for both binarization. In addition, the coding in the bypass mode (after reaching the maximum number of context-coded bins) remains unchanged.

The following C-style pseudo-code further illustrates the concept for the coding of coefficient groups. The yellow-marked parts represent the changes relative to VTM-6.

startScanIdx    = ( firstSubblock ? firstSigScanIdx : minSubblockScanIdx )
endScanIdx      = maxSubblockScanIdx
startIdxBypass  = ( remRegBins >= 4 ? maxSubblockScanIdx + 1 : startScanIdx )

// first pass (regular coded bins)
for( k = startScanIdx; k <= endScanIdx && remRegBins >= 4; k++ )  {
  coeff[k] = 0
#if VARIANT_B
  derive variable sumAbs
  if( state > 1 && sumAbs > 0 )
#else // VARIANT A
  derive variable sumAbs
  if( state > 1 )
#endif
  {
    // binarization B
    decode eq1_flag[k]
    remRegBins = remRegBins - 1
    if( eq1_flag[k] == 1)
      coeff[k] = 1
    else {
      if( gt1_flag cannot be inferred to be equal to 1 )  {
        decode gt1b_flag[k]  // different contexts than for gt1_flag of binarization A
        remRegBins = remRegBins - 1
      }
      coeff[k] = 2 * gt1_flag[k]
    }
  } else {
    // binarization A (VTM-6 binarization)
    if( sig_flag cannot be inferred to be equal to 1 )  {
      decode sig_flag[k]
      remRegBins = remRegBins - 1
    }
    if( sig_flag[k] != 0 )  {
      decode gt1_flag[k]
      remRegBins = remRegBins – 1
      coeff[k] = 1 + gt1_flag[k]
    }
  }
  // common part for both binarizations
  if( gt1_flag[k] != 0 )  {
    decode par_flag[k]
    decode gt3_flag[k]
    remRegBins = remRegBins - 2
    coeff[k] = coeff[k] + par_flag[k] + 2 * gt3_flag[k]
  }
  if( remRegBins < 4 )  {
    startIdxBypass = k + 1
  }
  state = stateTransTable[ state ][ coeff[k] & 1 ]
}

// second pass (bypass coding of remainder)
for( k = startScanIdx; k < startIdxBypass; k++ )  {
  if( gt3_flag[k] != 0 )  {
    decode rem[k]
    coeff[k] = coeff[k] + 2 * rem[k]
  }
}

// third pass (bypass coding of absolute levels)
for( k = startIdxBypass; k <= endScanIdx; k++ )  {
  derive pos0
  decode abs_level[k]
  if( abs_level[k] == pos0 )
    coeff[k] = 0
  else if ( abs_level[k] < pos0)
    coeff[k] = abs_level[k] + 1
  else
    coeff[k] = abs_level[k] 
  state = stateTransTable[ state ][ coeff[k] & 1 ]
}

// fourth pass (bypass coding of signs)
for( k = startScanIdx; k <= endScanIdx; k++ )  {
  if( coeff[k] != 0 )  {
    decode sign_flag[k]
    if( sign_flag[k] == 1 )
       coeff[k] = -coeff[k]
  }
}

## Context Modelling

For the flags sig_flag, gt1_flag, par_flag, and gt3_flag, the same context modelling as in VTM-6 is used. Also, the remainder and the bypass-coded absolute levels are coded in the same way as in VTM-6.

For the new bins of the second binarization eq1_flag and gt1b_flag, the context modelling is similar to that of the sig_flag.

The context of the eq1_flag depends on

- whether we code a luma or chroma block (same as for sig_flag in VTM-6),

- the quantization state (same as for sig_flag in VTM-6);

- the diagonal class  (same as for sig_flag in VTM-6);

- the sum of partially reconstructed absolute values inside local template (same as for sig_flag in VTM-6).

The context for the gt1b_flag depends on

- whether we code a luma or chroma block (same as for sig_flag in VTM-6),

- the diagonal class  (same as for sig_flag in VTM-6);

- the sum of partially reconstructed absolute values inside local template (same as for sig_flag in VTM-6).

The context for the gt1b_flag does not depend on the quantization state.

In both variants, the number of additional contexts for the eq1_flag is identical to the number of no longer used contexts for the sig_flag. For the gt1b_flag, 20 additional context (12 for luma and 8 for chroma) are required for variant A, and 15 additional context (9 for luma and 6 for chroma) are required for variant B. Hence, in total 20 additional contexts are introduced for variant A and 15 additional context are introduced for variant B.

# Experimental Results

We compared the coding efficiency and the bin-to-bit ratios of the proposed two variants (A and B) with two references:

- VTM-6 in standard configuration (dependent quantization enabled);

- VTM-6 with dependent quantization disabled (and sign data hiding enabled).

In the tested variants, dependent quantization was always enabled (i.e., the VTM standard configuration was used). Simulations were run for the common test conditions and a low QP configuration (QP values 2, 7, 12, 17 and 100 frames as used in CE7).

The reported bin-to-bit ratios represent weighted bin-to-bit ratio as used in CE7.  That means, context-coded bins are weighted with a factor of 1 and bypass-coded bins are weighted with a factor of 0.25.

The average results for CTC and the low QP configuration are summarized in Table 3 and Table 4, respectively. Detailed results are provided in Table 4 to Table 11.

It should further be noted that the proposed state-dependent binarization (both variants) prevents the insertion of cabac_zero_word’s in the generated bitstreams for all tested QP values (2, 7, 12, 17, 22, 27, 32, 37).

Table 3:  Summary of average simulation results (BD rates and weighted bin-to-bit ratios) for CTC. The first table uses VTM-6 with dependent quantization enabled as reference; the second table uses VTM-6 with dependent quantization disabled (and sign data hiding enabled) as reference.

| CTC |  | Over VTM-6 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
|  | AI | -0.03% | -0.07% | -0.16% | 1.14 | 1.08 | -4.84% | 1.14 | 1.08 | -4.73% | 103% | 103% |
| A | RA | 0.05% | -0.07% | -0.13% | 1.13 | 1.07 | -5.40% | 1.19 | 1.13 | -4.78% | 101% | 102% |
|  | LB | 0.07% | 0.02% | 0.15% | 1.13 | 1.08 | -5.09% | 1.22 | 1.15 | -5.65% | 104% | 107% |
|  | AI | -0.04% | -0.22% | -0.27% | 1.14 | 1.08 | -5.28% | 1.14 | 1.08 | -5.12% | 105% | 102% |
| B | RA | 0.03% | -0.01% | -0.16% | 1.13 | 1.07 | -5.62% | 1.19 | 1.13 | -5.03% | 102% | 102% |
|  | LB | 0.04% | -0.04% | -0.16% | 1.13 | 1.07 | -5.84% | 1.22 | 1.15 | -6.32% | 102% | 102% |
| CTC |  | Over VTM-6, DQ=0, SDH=1 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  |  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
|  | AI | -2.01% | 0.78% | 0.75% | 1.08 | 1.08 | 0.12% | 1.08 | 1.08 | 0.04% | 108% | 99% |
| A | RA | -1.68% | 0.19% | 0.49% | 1.07 | 1.07 | -0.32% | 1.14 | 1.13 | -0.75% | 102% | 101% |
|  | LB | -1.24% | -1.44% | -1.27% | 1.07 | 1.08 | 0.25% | 1.17 | 1.15 | -1.44% | 105% | 105% |
|  | AI | -2.02% | 0.63% | 0.64% | 1.08 | 1.08 | -0.35% | 1.08 | 1.08 | -0.37% | 110% | 98% |
| B | RA | -1.70% | 0.25% | 0.47% | 1.07 | 1.07 | -0.55% | 1.14 | 1.13 | -1.02% | 103% | 101% |
|  | LB | -1.27% | -1.51% | -1.59% | 1.07 | 1.07 | -0.54% | 1.17 | 1.15 | -2.14% | 102% | 100% |

Table 4:  Summary of average simulation results (BD rates and weighted bin-to-bit ratios) for low QP (2, 7, 12, 17). The first table uses VTM-6 with dependent quantization enabled as reference; the second table uses VTM-6 with dependent quantization disabled (and sign data hiding enabled) as reference.

| lowQP |  | Over VTM-6 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
|  | AI | 0.05% | -0.09% | -0.10% | 0.93 | 0.89 | -4.43% | 0.94 | 0.90 | -4.52% | 106% | 107% |
| A | RA | -0.05% | -0.22% | -0.25% | 0.86 | 0.83 | -3.09% | 1.10 | 1.02 | -7.39% | 104% | 103% |
|  | LB | 0.09% | -0.29% | -0.32% | 0.92 | 0.88 | -4.09% | 1.12 | 1.03 | -8.14% | 104% | 102% |
|  | AI | 0.11% | -0.20% | -0.21% | 0.93 | 0.89 | -4.37% | 0.94 | 0.90 | -4.38% | 105% | 105% |
| B | RA | -0.02% | -0.29% | -0.32% | 0.86 | 0.83 | -3.06% | 1.10 | 1.02 | -6.99% | 105% | 103% |
|  | LB | 0.12% | -0.34% | -0.38% | 0.92 | 0.88 | -4.04% | 1.12 | 1.03 | -7.82% | 104% | 103% |
| lowQP |  | Over VTM-6, DQ=0, SDH=1 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  |  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
|  | AI | -1.37% | -2.66% | -2.73% | 0.90 | 0.89 | -0.72% | 0.90 | 0.90 | -0.80% | 128% | 99% |
| A | RA | -2.50% | -3.84% | -3.23% | 0.83 | 0.83 | -0.11% | 1.04 | 1.02 | -2.32% | 123% | 96% |
|  | LB | -2.35% | -5.69% | -4.97% | 0.88 | 0.88 | -0.35% | 1.06 | 1.03 | -2.42% | 123% | 99% |
|  | AI | -1.32% | -2.78% | -2.83% | 0.90 | 0.89 | -0.66% | 0.90 | 0.90 | -0.65% | 127% | 97% |
| B | RA | -2.47% | -3.90% | -3.29% | 0.83 | 0.83 | -0.08% | 1.04 | 1.02 | -1.90% | 123% | 96% |
|  | LB | -2.33% | -5.74% | -5.03% | 0.88 | 0.88 | -0.30% | 1.06 | 1.03 | -2.07% | 123% | 100% |

Table 4:  BD rates and weighted bin-to-bit ratio changes for variant A compared to VTM-6 for CTC.

|  | All Intra |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| A1 | -0.11% | 0.10% | -0.19% | 1.19 | 1.12 | -5.98% | 1.19 | 1.12 | -6.10% | 103% | 101% |
| A2 | -0.05% | 0.05% | -0.03% | 1.18 | 1.12 | -5.42% | 1.18 | 1.12 | -5.33% | 104% | 100% |
| B | -0.02% | -0.12% | -0.28% | 1.15 | 1.10 | -4.56% | 1.15 | 1.10 | -4.77% | 103% | 102% |
| C | 0.01% | -0.25% | -0.20% | 1.08 | 1.03 | -5.06% | 1.07 | 1.03 | -4.16% | 103% | 110% |
| E | 0.01% | -0.05% | -0.03% | 1.11 | 1.07 | -3.27% | 1.10 | 1.06 | -3.45% | 103% | 103% |
| Avg | -0.03% | -0.07% | -0.16% | 1.14 | 1.08 | -4.84% | 1.14 | 1.08 | -4.73% | 103% | 103% |
| D | 0.04% | 0.09% | -0.08% | 1.06 | 1.01 | -4.08% | 1.05 | 1.01 | -3.84% | 102% | 111% |
| F | 0.03% | 0.03% | -0.03% | 1.16 | 1.14 | -2.02% | 1.17 | 1.14 | -2.02% | 102% | 103% |
|  | Random Access |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| A1 | 0.01% | -0.16% | -0.31% | 1.18 | 1.11 | -6.39% | 1.23 | 1.16 | -5.53% | 100% | 102% |
| A2 | -0.01% | -0.12% | -0.03% | 1.17 | 1.09 | -6.56% | 1.22 | 1.16 | -5.05% | 99% | 102% |
| B | 0.09% | 0.03% | -0.14% | 1.14 | 1.09 | -4.56% | 1.20 | 1.14 | -4.82% | 103% | 99% |
| C | 0.08% | -0.10% | -0.08% | 1.05 | 1.00 | -4.73% | 1.11 | 1.07 | -3.87% | 101% | 107% |
| Avg | 0.05% | -0.07% | -0.13% | 1.13 | 1.07 | -5.40% | 1.19 | 1.13 | -4.78% | 101% | 102% |
| D | 0.08% | -0.25% | -0.27% | 1.02 | 0.98 | -4.29% | 1.08 | 1.04 | -3.70% | 101% | 116% |
| F | 0.08% | -0.03% | 0.18% | 1.11 | 1.09 | -2.44% | 1.17 | 1.15 | -1.86% | 102% | 105% |
|  | Low Delay B |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| B | 0.12% | 0.27% | 0.21% | 1.18 | 1.11 | -6.15% | 1.25 | 1.18 | -6.21% | 106% | 99% |
| C | 0.11% | -0.03% | -0.19% | 1.09 | 1.04 | -4.96% | 1.17 | 1.11 | -5.30% | 102% | 113% |
| E | -0.08% | -0.35% | 0.51% | 1.11 | 1.07 | -3.42% | 1.25 | 1.19 | -5.43% | 104% | 110% |
| Avg | 0.07% | 0.02% | 0.15% | 1.13 | 1.08 | -5.09% | 1.22 | 1.15 | -5.65% | 104% | 107% |
| D | 0.16% | -0.08% | 0.82% | 1.05 | 1.01 | -3.71% | 1.16 | 1.10 | -5.04% | 103% | 125% |
| F | 0.11% | -0.02% | 0.64% | 1.19 | 1.16 | -1.83% | 1.26 | 1.23 | -2.32% | 102% | 111% |

Table 5:  BD rates and weighted bin-to-bit ratio changes for variant A compared to VTM-6 with dependent quantization disabled and sign data hiding enabled disabled for CTC.

|  | All Intra |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6, DQ=0, SDH=1 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| A1 | -2.62% | 1.06% | 0.63% | 1.13 | 1.12 | -1.05% | 1.13 | 1.12 | -1.13% | 103% | 95% |
| A2 | -3.49% | 1.45% | 1.21% | 1.12 | 1.12 | -0.04% | 1.12 | 1.12 | -0.08% | 108% | 95% |
| B | -1.76% | 1.09% | 1.12% | 1.09 | 1.10 | 0.50% | 1.10 | 1.10 | 0.20% | 108% | 97% |
| C | -1.32% | -0.11% | 0.16% | 1.02 | 1.03 | 0.08% | 1.02 | 1.03 | 0.37% | 114% | 109% |
| E | -1.27% | 0.50% | 0.59% | 1.06 | 1.07 | 0.91% | 1.05 | 1.06 | 0.65% | 107% | 101% |
| Avg | -2.01% | 0.78% | 0.75% | 1.08 | 1.08 | 0.12% | 1.08 | 1.08 | 0.04% | 108% | 99% |
| D | -1.33% | 0.30% | 0.28% | 1.01 | 1.01 | 0.71% | 1.00 | 1.01 | 0.65% | 114% | 108% |
| F | -0.66% | -0.09% | -0.09% | 1.14 | 1.14 | -0.13% | 1.14 | 1.14 | 0.02% | 109% | 104% |
|  | Random Access |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6, DQ=0, SDH=1 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| A1 | -1.88% | 0.22% | 1.02% | 1.12 | 1.11 | -1.26% | 1.17 | 1.16 | -1.17% | 97% | 100% |
| A2 | -1.93% | 0.30% | 0.20% | 1.10 | 1.09 | -0.69% | 1.17 | 1.16 | -0.85% | 101% | 100% |
| B | -1.82% | 0.84% | 1.02% | 1.08 | 1.09 | 0.47% | 1.15 | 1.14 | -0.77% | 103% | 96% |
| C | -1.15% | -0.74% | -0.33% | 1.00 | 1.00 | -0.27% | 1.07 | 1.07 | -0.32% | 104% | 109% |
| Avg | -1.68% | 0.19% | 0.49% | 1.07 | 1.07 | -0.32% | 1.14 | 1.13 | -0.75% | 102% | 101% |
| D | -1.19% | -0.65% | -0.52% | 0.97 | 0.98 | 0.39% | 1.04 | 1.04 | -0.47% | 104% | 113% |
| F | -0.64% | -0.83% | -0.82% | 1.09 | 1.09 | 0.04% | 1.15 | 1.15 | -0.03% | 105% | 106% |
|  | Low Delay B |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6, DQ=0, SDH=1 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| B | -1.64% | -1.26% | -0.88% | 1.11 | 1.11 | -0.22% | 1.20 | 1.18 | -1.73% | 106% | 98% |
| C | -0.94% | -2.44% | -2.22% | 1.04 | 1.04 | 0.17% | 1.12 | 1.11 | -1.27% | 104% | 114% |
| E | -0.98% | -0.38% | -0.65% | 1.06 | 1.07 | 1.12% | 1.20 | 1.19 | -1.37% | 102% | 107% |
| Avg | -1.24% | -1.44% | -1.27% | 1.07 | 1.08 | 0.25% | 1.17 | 1.15 | -1.44% | 105% | 105% |
| D | -1.00% | -2.90% | -1.94% | 1.00 | 1.01 | 1.09% | 1.12 | 1.10 | -1.73% | 107% | 122% |
| F | -0.41% | -2.63% | -2.10% | 1.16 | 1.16 | 0.50% | 1.24 | 1.23 | -0.28% | 106% | 111% |

Table 6:  BD rates and weighted bin-to-bit ratio changes for variant B compared to VTM-6 for CTC.

|  | All Intra |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| A1 | -0.13% | -0.02% | -0.29% | 1.19 | 1.12 | -5.36% | 1.19 | 1.13 | -5.57% | 104% | 103% |
| A2 | -0.04% | -0.08% | -0.19% | 1.18 | 1.12 | -5.49% | 1.18 | 1.12 | -5.29% | 104% | 104% |
| B | -0.04% | -0.33% | -0.43% | 1.15 | 1.08 | -5.46% | 1.15 | 1.09 | -5.44% | 105% | 101% |
| C | -0.01% | -0.39% | -0.19% | 1.08 | 1.02 | -5.49% | 1.07 | 1.02 | -4.80% | 105% | 101% |
| E | 0.01% | -0.17% | -0.19% | 1.11 | 1.06 | -4.54% | 1.10 | 1.05 | -4.40% | 105% | 102% |
| Avg | -0.04% | -0.22% | -0.27% | 1.14 | 1.08 | -5.28% | 1.14 | 1.08 | -5.12% | 105% | 102% |
| D | 0.03% | -0.09% | -0.22% | 1.06 | 1.00 | -5.24% | 1.05 | 1.00 | -4.83% | 104% | 109% |
| F | 0.01% | -0.05% | -0.17% | 1.16 | 1.14 | -2.24% | 1.17 | 1.14 | -2.38% | 105% | 100% |
|  | Random Access |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| A1 | -0.01% | -0.19% | -0.21% | 1.18 | 1.11 | -5.76% | 1.23 | 1.16 | -5.27% | 102% | 105% |
| A2 | -0.02% | -0.18% | -0.11% | 1.17 | 1.10 | -6.28% | 1.22 | 1.16 | -5.03% | 102% | 105% |
| B | 0.04% | 0.09% | -0.21% | 1.14 | 1.08 | -5.44% | 1.20 | 1.13 | -5.31% | 102% | 103% |
| C | 0.07% | 0.11% | -0.08% | 1.05 | 1.00 | -5.21% | 1.11 | 1.06 | -4.46% | 103% | 97% |
| Avg | 0.03% | -0.01% | -0.16% | 1.13 | 1.07 | -5.62% | 1.19 | 1.13 | -5.03% | 102% | 102% |
| D | 0.02% | -0.10% | -0.10% | 1.02 | 0.97 | -5.05% | 1.08 | 1.03 | -4.49% | 102% | 106% |
| F | 0.04% | 0.00% | 0.13% | 1.11 | 1.08 | -2.68% | 1.17 | 1.14 | -2.36% | 103% | 102% |
|  | Low Delay B |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| B | 0.08% | 0.31% | -0.24% | 1.18 | 1.10 | -6.82% | 1.25 | 1.17 | -6.72% | 101% | 103% |
| C | 0.10% | 0.01% | -0.18% | 1.09 | 1.03 | -5.62% | 1.17 | 1.10 | -6.03% | 103% | 101% |
| E | -0.11% | -0.70% | 0.01% | 1.11 | 1.06 | -4.36% | 1.25 | 1.18 | -6.21% | 101% | 100% |
| Avg | 0.04% | -0.04% | -0.16% | 1.13 | 1.07 | -5.84% | 1.22 | 1.15 | -6.32% | 102% | 102% |
| D | 0.17% | -0.05% | 0.72% | 1.05 | 1.00 | -4.79% | 1.16 | 1.09 | -5.80% | 103% | 106% |
| F | 0.20% | 0.90% | 0.32% | 1.19 | 1.16 | -2.58% | 1.26 | 1.22 | -3.14% | 103% | 102% |

Table 7:  BD rates and weighted bin-to-bit ratio changes for variant B compared to VTM-6 with dependent quantization disabled and sign data hiding enabled disabled for CTC.

|  | All Intra |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6, DQ=0, SDH=1 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| A1 | -2.63% | 0.94% | 0.53% | 1.13 | 1.12 | -0.40% | 1.13 | 1.13 | -0.58% | 105% | 97% |
| A2 | -3.49% | 1.32% | 1.04% | 1.12 | 1.12 | -0.12% | 1.12 | 1.12 | -0.04% | 108% | 98% |
| B | -1.78% | 0.88% | 0.97% | 1.09 | 1.08 | -0.46% | 1.10 | 1.09 | -0.51% | 110% | 96% |
| C | -1.33% | -0.24% | 0.18% | 1.02 | 1.02 | -0.37% | 1.02 | 1.02 | -0.30% | 116% | 100% |
| E | -1.27% | 0.38% | 0.43% | 1.06 | 1.06 | -0.42% | 1.05 | 1.05 | -0.35% | 109% | 99% |
| Avg | -2.02% | 0.63% | 0.64% | 1.08 | 1.08 | -0.35% | 1.08 | 1.08 | -0.37% | 110% | 98% |
| D | -1.34% | 0.11% | 0.13% | 1.01 | 1.00 | -0.51% | 1.00 | 1.00 | -0.39% | 117% | 106% |
| F | -0.69% | -0.18% | -0.23% | 1.14 | 1.14 | -0.35% | 1.14 | 1.14 | -0.35% | 111% | 101% |
|  | Random Access |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6, DQ=0, SDH=1 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| A1 | -1.91% | 0.19% | 1.11% | 1.12 | 1.11 | -0.59% | 1.17 | 1.16 | -0.90% | 99% | 103% |
| A2 | -1.94% | 0.25% | 0.11% | 1.10 | 1.10 | -0.39% | 1.17 | 1.16 | -0.83% | 103% | 103% |
| B | -1.87% | 0.91% | 0.94% | 1.08 | 1.08 | -0.46% | 1.15 | 1.13 | -1.27% | 103% | 101% |
| C | -1.16% | -0.52% | -0.34% | 1.00 | 1.00 | -0.77% | 1.07 | 1.06 | -0.92% | 105% | 98% |
| Avg | -1.70% | 0.25% | 0.47% | 1.07 | 1.07 | -0.55% | 1.14 | 1.13 | -1.02% | 103% | 101% |
| D | -1.25% | -0.49% | -0.35% | 0.97 | 0.97 | -0.40% | 1.04 | 1.03 | -1.29% | 105% | 103% |
| F | -0.68% | -0.81% | -0.86% | 1.09 | 1.08 | -0.21% | 1.15 | 1.14 | -0.54% | 106% | 103% |
|  | Low Delay B |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6, DQ=0, SDH=1 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| B | -1.68% | -1.22% | -1.34% | 1.11 | 1.10 | -0.94% | 1.20 | 1.17 | -2.26% | 102% | 101% |
| C | -0.95% | -2.41% | -2.22% | 1.04 | 1.03 | -0.53% | 1.12 | 1.10 | -2.03% | 105% | 102% |
| E | -1.00% | -0.77% | -1.16% | 1.06 | 1.06 | 0.13% | 1.20 | 1.18 | -2.18% | 99% | 97% |
| Avg | -1.27% | -1.51% | -1.59% | 1.07 | 1.07 | -0.54% | 1.17 | 1.15 | -2.14% | 102% | 100% |
| D | -1.00% | -2.88% | -2.00% | 1.00 | 1.00 | -0.03% | 1.12 | 1.09 | -2.51% | 107% | 103% |
| F | -0.32% | -1.72% | -2.41% | 1.16 | 1.16 | -0.27% | 1.24 | 1.22 | -1.11% | 107% | 101% |

Table 8:  BD rates and weighted bin-to-bit ratio changes for variant A compared to VTM-6 for lowQP (2, 7, 12, 17).

|  | All Intra |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| A1 | -0.16% | -0.48% | -0.51% | 1.02 | 0.96 | -5.41% | 1.02 | 0.97 | -5.66% | 105% | 103% |
| A2 | -0.04% | -0.16% | -0.14% | 0.90 | 0.86 | -4.44% | 0.91 | 0.87 | -4.44% | 106% | 102% |
| B | -0.01% | 0.01% | -0.02% | 0.93 | 0.90 | -3.44% | 0.94 | 0.91 | -3.46% | 107% | 107% |
| C | 0.22% | 0.07% | 0.08% | 0.83 | 0.80 | -3.15% | 0.83 | 0.81 | -3.33% | 107% | 109% |
| E | 0.27% | 0.00% | -0.02% | 1.01 | 0.95 | -5.85% | 1.01 | 0.95 | -5.93% | 105% | 113% |
| Avg | 0.05% | -0.09% | -0.10% | 0.93 | 0.89 | -4.43% | 0.94 | 0.90 | -4.52% | 106% | 107% |
| D | 0.27% | 0.03% | 0.11% | 0.80 | 0.78 | -2.67% | 0.81 | 0.79 | -2.61% | 108% | 120% |
| F | 0.12% | 0.02% | 0.03% | 0.95 | 0.93 | -2.58% | 0.94 | 0.91 | -2.42% | 104% | 109% |
|  | Random Access |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| A1 | -0.14% | -0.22% | -0.30% | 0.96 | 0.92 | -4.46% | 1.15 | 1.06 | -7.93% | 104% | 102% |
| A2 | -0.33% | -0.50% | -0.52% | 0.84 | 0.81 | -2.67% | 1.10 | 1.01 | -7.86% | 103% | 102% |
| B | -0.04% | -0.18% | -0.24% | 0.87 | 0.85 | -2.95% | 1.12 | 1.04 | -6.65% | 105% | 102% |
| C | 0.21% | -0.07% | -0.04% | 0.77 | 0.75 | -2.33% | 1.04 | 0.96 | -7.55% | 105% | 105% |
| Avg | -0.05% | -0.22% | -0.25% | 0.86 | 0.83 | -3.09% | 1.10 | 1.02 | -7.39% | 104% | 103% |
| D | 0.35% | -0.08% | 0.07% | 0.75 | 0.73 | -1.95% | 1.02 | 0.95 | -6.76% | 103% | 118% |
| F | 0.16% | -0.08% | -0.05% | 0.90 | 0.89 | -1.93% | 1.02 | 0.98 | -3.88% | 104% | 106% |
|  | Low Delay B |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| B | -0.04% | -0.23% | -0.24% | 0.93 | 0.90 | -3.37% | 1.10 | 1.03 | -6.56% | 104% | 103% |
| C | 0.18% | -0.15% | -0.19% | 0.82 | 0.79 | -3.02% | 1.05 | 0.97 | -7.68% | 104% | 103% |
| E | 0.19% | -0.58% | -0.61% | 1.01 | 0.95 | -5.89% | 1.24 | 1.11 | -10.61% | 104% | 101% |
| Avg | 0.09% | -0.29% | -0.32% | 0.92 | 0.88 | -4.09% | 1.12 | 1.03 | -8.14% | 104% | 102% |
| D | 0.37% | -0.05% | -0.04% | 0.79 | 0.77 | -2.41% | 1.04 | 0.97 | -7.37% | 103% | 118% |
| F | 0.30% | 0.08% | 0.18% | 0.92 | 0.90 | -2.39% | 1.10 | 1.05 | -4.78% | 101% | 108% |

Table 9:  BD rates and weighted bin-to-bit ratio changes for variant A compared to VTM-6 with dependent quantization disabled and sign data hiding enabled disabled for lowQP (2, 7, 12, 17).

|  | All Intra |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6, DQ=0, SDH=1 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| A1 | -0.66% | -2.38% | -2.45% | 0.98 | 0.96 | -1.48% | 0.98 | 0.97 | -1.74% | 127% | 93% |
| A2 | -1.78% | -3.33% | -3.38% | 0.87 | 0.86 | -0.91% | 0.87 | 0.87 | -0.90% | 126% | 93% |
| B | -1.79% | -2.35% | -2.52% | 0.90 | 0.90 | 0.11% | 0.91 | 0.91 | -0.03% | 125% | 98% |
| C | -0.90% | -2.61% | -2.57% | 0.80 | 0.80 | -0.11% | 0.81 | 0.81 | -0.17% | 136% | 106% |
| E | -1.64% | -2.87% | -2.95% | 0.96 | 0.95 | -1.59% | 0.96 | 0.95 | -1.45% | 124% | 101% |
| Avg | -1.37% | -2.66% | -2.73% | 0.90 | 0.89 | -0.72% | 0.90 | 0.90 | -0.80% | 128% | 99% |
| D | -0.85% | -2.32% | -2.26% | 0.78 | 0.78 | 0.37% | 0.78 | 0.79 | 0.34% | 131% | 113% |
| F | -0.60% | -1.96% | -1.77% | 0.93 | 0.93 | -0.05% | 0.91 | 0.91 | -0.02% | 117% | 107% |
|  | Random Access |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6, DQ=0, SDH=1 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| A1 | -2.30% | -2.88% | -2.19% | 0.93 | 0.92 | -0.83% | 1.10 | 1.06 | -3.19% | 124% | 88% |
| A2 | -2.69% | -4.25% | -4.33% | 0.82 | 0.81 | -0.11% | 1.04 | 1.01 | -2.89% | 124% | 93% |
| B | -2.41% | -3.50% | -2.89% | 0.85 | 0.85 | 0.11% | 1.06 | 1.04 | -1.55% | 122% | 99% |
| C | -2.62% | -4.65% | -3.60% | 0.75 | 0.75 | 0.25% | 0.99 | 0.96 | -2.16% | 122% | 103% |
| Avg | -2.50% | -3.84% | -3.23% | 0.83 | 0.83 | -0.11% | 1.04 | 1.02 | -2.32% | 123% | 96% |
| D | -2.11% | -4.66% | -3.85% | 0.73 | 0.73 | 0.46% | 0.97 | 0.95 | -1.89% | 118% | 115% |
| F | -0.64% | -3.55% | -3.13% | 0.88 | 0.89 | 0.22% | 0.99 | 0.98 | -1.01% | 113% | 106% |
|  | Low Delay B |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6, DQ=0, SDH=1 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| B | -1.98% | -4.70% | -4.19% | 0.90 | 0.90 | 0.06% | 1.04 | 1.03 | -1.33% | 122% | 100% |
| C | -2.14% | -6.00% | -5.15% | 0.79 | 0.79 | 0.00% | 0.99 | 0.97 | -2.29% | 124% | 102% |
| E | -3.26% | -6.91% | -6.00% | 0.96 | 0.95 | -1.20% | 1.15 | 1.11 | -3.83% | 125% | 93% |
| Avg | -2.35% | -5.69% | -4.97% | 0.88 | 0.88 | -0.35% | 1.06 | 1.03 | -2.42% | 123% | 99% |
| D | -1.63% | -5.88% | -5.42% | 0.77 | 0.77 | 0.38% | 0.99 | 0.97 | -1.93% | 119% | 119% |
| F | -0.88% | -5.25% | -4.58% | 0.90 | 0.90 | -0.08% | 1.06 | 1.05 | -1.58% | 110% | 106% |

Table 10:  BD rates and weighted bin-to-bit ratio changes for variant B compared to VTM-6 for lowQP (2, 7, 12, 17).

|  | All Intra |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| A1 | -0.11% | -0.60% | -0.61% | 1.02 | 0.96 | -5.36% | 1.02 | 0.97 | -5.27% | 105% | 104% |
| A2 | 0.06% | -0.33% | -0.32% | 0.90 | 0.86 | -4.38% | 0.91 | 0.87 | -4.40% | 105% | 102% |
| B | 0.07% | -0.11% | -0.16% | 0.93 | 0.90 | -3.67% | 0.94 | 0.91 | -3.68% | 107% | 106% |
| C | 0.23% | 0.01% | 0.02% | 0.83 | 0.80 | -3.03% | 0.83 | 0.81 | -3.17% | 107% | 104% |
| E | 0.30% | -0.10% | -0.07% | 1.01 | 0.95 | -5.37% | 1.01 | 0.95 | -5.43% | 103% | 109% |
| Avg | 0.11% | -0.20% | -0.21% | 0.93 | 0.89 | -4.37% | 0.94 | 0.90 | -4.38% | 105% | 105% |
| D | 0.25% | 0.07% | 0.11% | 0.80 | 0.78 | -2.69% | 0.81 | 0.79 | -2.64% | 108% | 110% |
| F | 0.16% | -0.06% | 0.01% | 0.95 | 0.93 | -2.86% | 0.94 | 0.91 | -2.54% | 106% | 106% |
|  | Random Access |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| A1 | -0.13% | -0.28% | -0.36% | 0.96 | 0.92 | -4.37% | 1.15 | 1.07 | -7.19% | 103% | 101% |
| A2 | -0.27% | -0.62% | -0.67% | 0.84 | 0.81 | -2.78% | 1.10 | 1.02 | -7.24% | 104% | 102% |
| B | 0.01% | -0.23% | -0.28% | 0.87 | 0.85 | -2.92% | 1.12 | 1.04 | -6.63% | 105% | 102% |
| C | 0.21% | -0.14% | -0.09% | 0.77 | 0.75 | -2.23% | 1.04 | 0.97 | -7.12% | 106% | 105% |
| Avg | -0.02% | -0.29% | -0.32% | 0.86 | 0.83 | -3.06% | 1.10 | 1.02 | -6.99% | 105% | 103% |
| D | 0.32% | 0.02% | -0.01% | 0.75 | 0.73 | -1.90% | 1.02 | 0.95 | -6.63% | 102% | 104% |
| F | 0.16% | 0.01% | 0.10% | 0.90 | 0.88 | -2.10% | 1.02 | 0.98 | -3.91% | 104% | 105% |
|  | Low Delay B |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| B | 0.04% | -0.33% | -0.33% | 0.93 | 0.90 | -3.69% | 1.10 | 1.03 | -6.60% | 106% | 103% |
| C | 0.19% | -0.21% | -0.21% | 0.82 | 0.80 | -2.86% | 1.05 | 0.97 | -7.21% | 102% | 103% |
| E | 0.16% | -0.55% | -0.70% | 1.01 | 0.96 | -5.36% | 1.24 | 1.11 | -9.95% | 104% | 103% |
| Avg | 0.12% | -0.34% | -0.38% | 0.92 | 0.88 | -4.04% | 1.12 | 1.03 | -7.82% | 104% | 103% |
| D | 0.34% | 0.03% | -0.07% | 0.79 | 0.77 | -2.36% | 1.04 | 0.97 | -7.19% | 103% | 104% |
| F | 0.21% | 0.07% | 0.05% | 0.92 | 0.90 | -2.42% | 1.10 | 1.05 | -4.78% | 103% | 106% |

Table 11:  BD rates and weighted bin-to-bit ratio changes for variant B compared to VTM-6 with dependent quantization disabled and sign data hiding enabled disabled for lowQP (2, 7, 12, 17).

|  | All Intra |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6, DQ=0, SDH=1 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| A1 | -0.61% | -2.50% | -2.55% | 0.98 | 0.96 | -1.43% | 0.98 | 0.97 | -1.33% | 126% | 94% |
| A2 | -1.67% | -3.49% | -3.55% | 0.87 | 0.86 | -0.85% | 0.87 | 0.87 | -0.86% | 125% | 93% |
| B | -1.71% | -2.48% | -2.66% | 0.90 | 0.90 | -0.13% | 0.91 | 0.91 | -0.26% | 125% | 98% |
| C | -0.88% | -2.68% | -2.62% | 0.80 | 0.80 | 0.02% | 0.81 | 0.81 | 0.00% | 136% | 102% |
| E | -1.60% | -2.97% | -2.99% | 0.96 | 0.95 | -1.08% | 0.96 | 0.95 | -0.94% | 121% | 97% |
| Avg | -1.32% | -2.78% | -2.83% | 0.90 | 0.89 | -0.66% | 0.90 | 0.90 | -0.65% | 127% | 97% |
| D | -0.87% | -2.29% | -2.26% | 0.78 | 0.78 | 0.35% | 0.78 | 0.79 | 0.31% | 131% | 104% |
| F | -0.57% | -2.04% | -1.79% | 0.93 | 0.93 | -0.34% | 0.91 | 0.91 | -0.14% | 119% | 104% |
|  | Random Access |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6, DQ=0, SDH=1 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| A1 | -2.30% | -2.94% | -2.25% | 0.93 | 0.92 | -0.74% | 1.10 | 1.07 | -2.41% | 123% | 87% |
| A2 | -2.63% | -4.37% | -4.47% | 0.82 | 0.81 | -0.22% | 1.04 | 1.02 | -2.23% | 125% | 93% |
| B | -2.37% | -3.55% | -2.92% | 0.85 | 0.85 | 0.14% | 1.06 | 1.04 | -1.53% | 122% | 99% |
| C | -2.62% | -4.72% | -3.65% | 0.75 | 0.75 | 0.35% | 0.99 | 0.97 | -1.71% | 124% | 103% |
| Avg | -2.47% | -3.90% | -3.29% | 0.83 | 0.83 | -0.08% | 1.04 | 1.02 | -1.90% | 123% | 96% |
| D | -2.13% | -4.56% | -3.93% | 0.73 | 0.73 | 0.51% | 0.97 | 0.95 | -1.76% | 118% | 101% |
| F | -0.64% | -3.46% | -2.98% | 0.88 | 0.88 | 0.05% | 0.99 | 0.98 | -1.04% | 113% | 105% |
|  | Low Delay B |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6, DQ=0, SDH=1 |  |  | Max - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U | V | Ref | Test | Diff | Ref | Test | Diff | EncT | DecT |
| B | -1.91% | -4.79% | -4.28% | 0.90 | 0.90 | -0.27% | 1.04 | 1.03 | -1.37% | 124% | 100% |
| C | -2.13% | -6.06% | -5.17% | 0.79 | 0.80 | 0.17% | 0.99 | 0.97 | -1.80% | 121% | 102% |
| E | -3.29% | -6.88% | -6.09% | 0.96 | 0.96 | -0.63% | 1.15 | 1.11 | -3.12% | 126% | 96% |
| Avg | -2.33% | -5.74% | -5.03% | 0.88 | 0.88 | -0.30% | 1.06 | 1.03 | -2.07% | 123% | 100% |
| D | -1.66% | -5.79% | -5.45% | 0.77 | 0.77 | 0.43% | 0.99 | 0.97 | -1.73% | 118% | 105% |
| F | -0.96% | -5.27% | -4.71% | 0.90 | 0.90 | -0.10% | 1.06 | 1.05 | -1.59% | 112% | 104% |

# References

- M. Zhou, “AHG16/Non-CE7: A study of bin to bit ratio for VTM5.0”, Joint Video Experts Team, doc. JVET-O0068, July 2019.

- M. Zhou, “AHG16/Non-CE7: A study of bin to bit ratio for VTM6.0”, Joint Video Experts Team, doc. JVET-P0050, October 2019.

# Patent Rights Declaration

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).