| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0638 |
| --- | --- |

| Title: | AHG10/CE3-related: Adaptive coding subset for intra mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Bhat, David Gommelet, Jean-Marc Thiesse, Didier Nicholson VITEC 99 rue Pierre Semard 92320 Chatillon, France | Tel: Email: | +33 1 46 73 06 06 madhukar.bat@vitec.com david.gommelet@vitec.com jean-marc.thiesse@vitec.com didier.nicholson@vitec.com |
| Source: | VITEC |  |  |

_____________________________

# Abstract

This contribution introduces an additional flexibility capacity at the encoder level. This proposal tries to optimize the coding cost of specific parameters in case of underuse of the full standard capacity. Two scenarios are considered: underuse due to specific content at a given encoding point and desired underuse of modes due to a specific encoder implementation. While several mechanisms are already available to handle these underuses in previous standards and in current VVC draft, including high level flags and CABAC probability update, it is asserted that higher flexibility might bring a better trade-off for encoder design. The contribution proposes a subset signalization of intra coding modes at Slice level. The low level normative changes are limited to avoid parsing of 0 to 3 bypass bits for the syntax element intra_luma_mpm_remainder depending on the decimation factor signalized at high level.  Sub-set signaling by decimation of intra angular candidates depending on block size is evaluated in the contribution and compared to similar sub-set implemented with encoder-only modifications. Proposed normative ACS allows dividing the loss by two in RA, ended with a loss of XX%.

# Introduction

At Gothenburg meeting, the contribution JVET-O0888 [1] introduces the concept of adaptive coding subset (ACS). The proposed approach consists in signaling at high level sub-set of coding tools which are using several coding options in competition in order to find better complexity/gain trade-off in an adaptive way depending on content, encoding configuration and QP. The proposed concept was applied to SAO offsets and early results for intra mode were presented in [1]. It was commented that the application to SAO was not the more interesting and further study was recommended.

For this contribution, we focus on the application of ACS to intra mode coding. VVC significantly increases the number of angular modes with 65 angular modes, in addition to DC, Planar and MIP. This increase provides a substantial improvement for the CTC but it is asserted that the trade-off between the improvement coming from the candidate diversity and the coding cost is not systematically optimal.

# Proposed method

## Overview

In this contribution, a method called Adaptive Coding Sub-set (ACS) is proposed to provide the ability to the encoder to define a subset of coding options for specific tool. This is enabled at PPS level according to pps_adaptive_coding_subset_enabled_flag flag and then the ACS mode used for each coding element handled by the method is specified at slice level in a syntax function: adaptive_coding_subset_data(). The intend by having it at slice level is to potentially change the data depending on the content and picture type, so maybe it can be more suited for APS. It is also possible to move these data at PPS level to limit the overhead in case of subset definition fixed for a whole encoding.

Several subset definition modes PARAM_acs_mode have been defined inspired by classical fast encoder implementation in order to limit the signalization burden. PARAM stands for the coding parameter considered for a given subset, for example intra. Two modes are defined but only the first one is considered in this contribution:

- Decimation mode: in this mode, only one syntax element is transmitted, the decimation factor PARAM_acs_decimation_factor which is in the range of 0 to log2 of the number of elements in the considered PARAM reference list. Thanks to this factor, the subset is built from only one decimation factor element.

- Explicit mode: in this mode, two syntax elements are transmitted, the size of the subset PARAM_acs_explicit_num which provides the number of elements in the subset and which is in the range of 0 to the number of elements in the considered PARAM reference list; and the subset itself PARAM_acs_explicit_subset.

As described below for intra, the ACS definition can be introduced according to CU sizes or potentially the QP values.

The proposed approach had been applied to intra angular modes as detailed below. Other potential coding elements include Merge candidate list, SAO edge offset class and MMVD distance index for example.

## Application to intra mode candidates

There are 65 Intra angular modes in current version of VVC draft [2]. The proposed approach is applied to intra mode for reducing the number of angular modes as represented in figure 1 depending on CU size.

Figure 1. Decimation factor applied to intra angular modes (factors = 0, 1, 2, 3).

The method allows dividing by 2, 4 or 8 the number of angular modes, leading to respectively to 35, 19 and 11 intra modes in total. Each additional factor avoids transmitting one bit for the syntax element intra_luma_mpm_remainder, leading respectively to 5, 4, or 3 bits bypass coded instead of the current 6 bits. The modification is slightly intrusive since it only consists in checking the intra_luma_acs_decimation_factor to avoid parsing of 0 to 3 bits.

The proposed modification for intra are as follows:

- at Slice level, within adaptive_coding_subset_data():

  - intra_acs_mode  equals to 0 or 1 specifies the adaptive coding subset mode used for intra angular modes subset generation.  intra_acs_mode equals 1 specifies a fixed decimation of intra mode candidate by an integer factor. When intra_acs_mode is not present, it is inferred to be equal to 0.

  - intra_luma_acs_decimation_factor[areaId]  specifies the decimation factor for the subset generation of intra luma candidate when intra_acs_mode equals 1 and depending on the coding block size M x N. Four values of areaId are possible as specified in table 7-12, areaId equals 0 stands for a coding block with a number of pixels inferior to 64, areaId equals 1 stands for a coding block with a number of pixels inferior to 256 and higher or equal to 64. The value of intra_luma_acs_decimation_factor[areaId]  shall be in the range of 0 to 3, inclusive. When intra_luma_acs_decimation_factor[areaId]  is not present, it is inferred to be equal to 0.

Table 7-12 – Specification of areaId

| Number of pixels | areaId |
| --- | --- |
| 64 | 0 |
| 256 | 1 |
| 1024 | 2 |
| 4096 | 3 |

- Changes in CABAC:

| intra_luma_mpm_remainder[ ][ ] | Bypass | bypass | bypass | (intra_luma_acs_decimation_factor[areaId] > = 3 ) ? na : bypass | (intra_luma_acs_decimation_factor[areaId] > = 2 ) ? na : bypass | (intra_luma_acs_decimation_factor[areaId] > = 1 ) ? na : bypass |
| --- | --- | --- | --- | --- | --- | --- |

| intra_luma_mpm_remainder[ ][ ] | TB | cMax = ( intra_acs_mode = = 0 ) ? 60 : ( 60 – ( 64 – 2 ^( 6 – intra_luma_acs_decimation_factor[areaId] ) ) ) |
| --- | --- | --- |

# Results

The proposed approach has been implemented on top of VTM 6.0 and the same method with encoder-only changes has been evaluated. Common test conditions are used for evaluation.  A decimation mode with the settings specified in Table 1 has been tested. This configuration typically matches with a potential hardware implementation of VVC encoder with the following trade-off decision design:

- The number of intra modes is significantly reduced for larger CU sizes to limit the silicium area of corresponding modules.

- The number of intra mode is set down to the HEVC number for small CU sizes because those CU are highly challenging to process in term of pixels by cycle.

- In-between, the number of modes is set to the maximum in order to retain compression efficiency.

|  | CU area < 64 pixels | CU area >= 64 and < 1024 pixels | CU area >= 1024 pixels |
| --- | --- | --- | --- |
| Decimation factor | 1 | 0 | 2 |
| Number of intra modes | 35 | 67 | 19 |

Table 1: Decimation factor selection based on CU area

Regarding the implementation in VTM 6.0, within the intra decision, the number of RD checks is let as in the reference to remain fair. It is suggested that a test with full RD checks in intra could be more representative of the impact of the proposed method but this has not been tried.

## Results for encoder-only implementation

In Table 2, the encoder-only results are provided in Random Access with a non-negligible loss against VTM-6.0 anchor.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0,28% | 0,21% | 0,25% | 100% |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0,20% | -0,07% | -0,04% | 103% |  |
| Class F (optional) | 0,29% | 0,11% | 0,35% | 97% |  |

Table 2: Results of encoder-only with 19/67/35 modes in Random Access.

## Results for proposed implementation

In Table 3, the performances of the proposed ACS scheme are compared against the reference with 67 modes. The loss is reduced compared to the encoder-only version.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0,18% | 0,20% | 0,08% | 99% |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0,07% | -0,09% | 0,13% | 100% |  |
| Class F (optional) | 0,17% | -0,02% | 0,16% | 99% |  |

Table 3: Results of intra ACS of 19/67/35 modes in Random Access.

## Results for proposed implementation depending on QP

In order to provide an input regarding the use of the proposed ACS signaling for improving the coding efficiency by adapting the number of intra candidates depending on content and QP. The results of picking the best version between the VTM anchor and the proposed ACS configuration are reported in Table 4. Really small gain are brought by higher tested QPs for some content.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0,01% | 0,01% | 0,01% | 99% |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0,01% | -0,11% | -0,04% | 100% |  |
| Class F (optional) | 0,00% | 0,00% | -0,03% | 100% |  |

Table 4: Results of intra ACS of 19/67/35 modes depending on QP in Random Access.

# Conclusion

In this contribution, a signalization dedicated to coding subset definition has been proposed. The intend was to provide an additional flexibility for the encoder implementers to exploit the expected coding tools powerfulness of VVC. Thanks to the proposed ACS signalization it is expected to limit the signalization overhead in case of underuse of the standard both for complexity or content adaptation reason. In this contribution, the ACS concept has been leveraged for the complexity aspect applied to intra mode. A configuration with three different decimations factors has been used depending on CU size. It is reported that the proposed normative method allows recovering half of the loss resulting from an encoder-only implementation of the same decimation configuration. Further study of the approach is requested within a CE after definition of relevant scenarios.

# References

[1] M. Bhat, D Gommelet, J.-M. Thiesse, D. Nicholson, “AHG10: Adaptive Coding Sub-set for encoder optimization”, JVET-O0888, July 2019.

[2] B. Bross, J. Chen, S. Liu	, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019.

# Patent rights declaration(s)

VITEC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).