| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0640-r1 |
| --- | --- |

| Title: | Non-CE6: High Level Syntax Flag for SBT Transform Selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam NASER Fabrice LE LEANNEC Michel KERDRANVAT Tangi POIRIER | Tel: Email: | karam.naser@InterDigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In JVET-O0538, SPS control was provided to enable DCT2 transform implementation of ISP and SBT, where non-DCT2 transforms are enabled for ISP and SBT if they are also enabled for other blocks. Specifically, it is controlled by the SPS flag sps_mts_enabled_flag. The motivation was to decouple the two tools from the transfom design such that a low-memory encoder that is not equipped with DST7 and DCT8 can still use ISP and SBT. However, in the case of implicit MTS, the encoder requires DCT2 and DST7 for ISP and normal blocks and DCT8 for SBT. That is, unless DCT8 is supported, SBT need to be off. This contribution proposes removing this limitation and allow SBT to use DCT2 if DCT8 is not supported by the encoder.

# Introduction

In VVC draft 6 specification, SBT transform selection is described as:

The variable implicitMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitMtsEnabled is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- cu_sbt_flag is equal to 1 and Max( nTbW, nTbH ) is less than or equal to 32

- sps_explicit_mts_intra_enabled_flag is equal to 0 and CuPredMode[ 0 ][ xTbY ][ yTbY ] is equal to MODE_INTRA and lfnst_idx[ x0 ][ y0 ] is equal to 0 and intra_mip_flag[ x0 ][ y0 ] is equal to 0

- Otherwise, implicitMtsEnabled is set equal to 0.

The specification describes that if sps_mts_enabled_flag is equal to zero, SBT and ISP can still be used with DCT2 transform as the normal transform blocks. When sps_mts_enabled_flag equals to one, and both intra and inter MTS are not allowed. This corresponds to Implicit MTS design, which is a fast encoder mode that does not code MTS index and a combination between DST7 and DCT2 are used. Interestingly, ISP uses exactly the same transform design as Implicit MTS. However, SBT further needs DCT8 transform for its transform selection. This results in a restriction to the encoder design, where the fast encoder that uses implicit MTS must implement DCT8 in order to perform SBT. With the same motivation as in JVET-O0538, we propose to remove this restriction and allow the encoder to perform SBT with DCT2.

# Proposal

# Method 1

It is proposed to add an SPS flag, named sps_sbt_dct8_flag, that is coded if only the Implicit MTS is allowed. That is, when both inter and intra MTS are off, and sps_mts_enabled_flag is on. If this flag is off, DCT2 transform will be used for SBT. Its value is inferred to 1 when it is not available in the stream.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| .. |  |
| sps_sbt_enabled_flag | u(1) |
| if( sps_sbt_enabled_flag){ |  |
| sps_sbt_max_size_64_flag | u(1) |
| if(sps_mts_enabled_flag && ! (sps_explicit_mts_intra_enabled_flag ¦¦ sps_explicit_mts_inter_enabled_flag)) |  |
| sps_sbt_dct8_flag | u(1) |
| } |  |
| … |  |

sps_sbt_dct8_flag equal to 0 specifies that the SBT transform is fixed DCT2. sps_sbt_dct8_flag equal to 1 specifies that the SBT transform is selected as in Table 8-15. When not present, the value of sps_sbt_dct8_flag is inferred to be equal to 1.

The variable implicitMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitMtsEnabled is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- cu_sbt_flag is equal to 1, sps_sbt_dct8_flag equal to 1, and Max( nTbW, nTbH ) is less than or equal to 32

- sps_explicit_mts_intra_enabled_flag is equal to 0 and CuPredMode[ 0 ][ xTbY ][ yTbY ] is equal to MODE_INTRA and lfnst_idx[ x0 ][ y0 ] is equal to 0 and intra_mip_flag[ x0 ][ y0 ] is equal to 0

To understand the importance of this method, the following two tests were conducted:

Test 1:

- Anchor: VTM6.0 with implicit MTS and SBT off (--MTS=0 --MTSImplicit=1 --SBT=0)

- Test: VTM6.0 with implicit MTS and DCT2 for SBT  (--MTS=0 --MTSImplicit=1 --SBT=1)

Test 2:

- Anchor: VTM6.0 with implicit MTS and SBT off (--MTS=0 --MTSImplicit=1 --SBT=0)

- Test: VTM6.0 with implicit MTS and default transform for SBT  (--MTS=0 --MTSImplicit=1 --SBT=1)

The experimental results are given below:

Table 2 Experiment results of Test 1

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 + ImplicitMTS+ SBT-Off |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.00% | -0.19% | 105% | 101% |
| Class A2 | -0.24% | -0.19% | -0.20% | 97% | 92% |
| Class B | -0.25% | -0.14% | 0.03% | 108% | 104% |
| Class C | -0.22% | -0.07% | 0.00% | 103% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.21% | -0.10% | -0.07% | 104% | 100% |
| Class D | -0.25% | -0.12% | 0.01% | 108% | 106% |
| Class F | -0.09% | -0.11% | -0.08% | 104% | 104% |

Table 3 Experiment results of Test 2

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 + ImplicitMTS+ SBT-Off |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | 0.16% | 0.00% | 105% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.42% | -0.02% | 0.03% | 110% | 106% |
| Class C | -0.42% | 0.00% | -0.05% | 108% | 105% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.47% | 0.05% | 0.05% | 107% | 107% |
| Class F | -0.18% | 0.00% | -0.05% | 104% | 101% |

From the experimenal results, it can be seen that more than half of the gain is presevered when DCT2 transform is used instead of the implicit transform. That is, if the encoder is not equiped with DCT8, the method allows it to still employ SBT with 0.21% coding gain.

## Method 2

In another method, an SPS flag namedd sps_sbt_dct2_flag is coded to indicate if DCT2 transform is used instead of its implicit transform selection. This solves the problem of the fast encoder, as well as allows having additional control on SBT transform design. Namely, if an encoder is requires having all inter block to be trasformed by DCT2, this flag allowes it to do that.

The corresponding specification changes are:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| .. |  |
| if( sps_mts_enabled_flag ) { |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| sps_sbt_dct2_flag | u(1) |
| } |  |
| … |  |

sps_sbt_dct2_flag equal to 1 specifies that the SBT transform is fixed DCT2. sps_sbt_dct2_flag equal to 0 specifies that the SBT transform is selected as in Table 8-15. When not present, the value of sps_sbt_dct2_flag is inferred to be equal to 0.

The variable implicitMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitMtsEnabled is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- cu_sbt_flag is equal to 1, sps_sbt_dct2_flag equal to 0, and Max( nTbW, nTbH ) is less than or equal to 32

- sps_explicit_mts_intra_enabled_flag is equal to 0 and CuPredMode[ 0 ][ xTbY ][ yTbY ] is equal to MODE_INTRA and lfnst_idx[ x0 ][ y0 ] is equal to 0 and intra_mip_flag[ x0 ][ y0 ] is equal to 0

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).