| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0663-v2 |
| --- | --- |

| Title: | CE4-1.14 related: Block size limitation of enabling TPM and GEO |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng, Li Zhang, Hongbin Liu, Kai Zhang, Yang Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two tests to limit the block size for TPM and GEO. Test #1 is to disable TPM and GEO for blocks greater than 32×32, while Test #2 is to disable TPM and GEO for blocks greater than 64×64. The proposed modifications were tested on top of CE4-1.14 software under CTC. The overall BD-rate changes for {Y, Cb, Cr} are reported as follows.

| Test #1 | RA | 0.04% | -0.01% | 0.02% | 98% | 100% |
| --- | --- | --- | --- | --- | --- | --- |
|  | LDB | 0.21% | 0.36% | 0.14% | 97% | 100% |
| Test #2 | RA | 0.00% | -0.02% | -0.01% | 98% | 98% |
|  | LDB | 0.00% | 0.05% | -0.09% | 98% | 98% |

If the blending masks are pre-calculated, it is asserted that memory storage required by TPM/GEO can be saved by 3.1 MB in Test #1 and 2.3 MB in Test #2, respectively. If the blending masks and motion storage masks of TPM and GEO are derived on the fly, the proposed methods can reduce the averaged computational complexity.

# Introduction

In CE4-1.14 [1], GEO merge mode is being studied as an extension of the existing TPM in VVC [2]. GEO uses the same prediction blending concept as TPM but extends the blending masks up to 140 different modes with 32 angles and 5 distance offsets. The blending masks of GEO modes are derived from the distance of the sample position and the split boundary using three look-up tables. The sample weights calculated for the luma samples in GEO are subsampled for chroma samples. GEO and TPM can be applied to blocks up to 128×128 which requires a large memory to store blending masks and motion storage masks. And if the blending masks and motion storage masks of luma and chroma are computed on the fly, the derivation of blending masks and motion storage masks would require an extra computational complexity.

# Proposal

Two tests are proposed to save the memory for blending masks of TPM and GEO.

Test #1: Disable TPM and GEO for blocks greater than 32×32.

Test #2: Disable TPM and GEO for blocks greater than 64×64.

If the blending masks and motion storage masks are pre-calculated, the memory storage required by TPM/GEO can be saved by 3.1 MB in Test #1 and 2.3 MB in Test #2, respectively, as shown in Table 1. If the blending masks and motion storage masks of TPM and GEO are derived on the fly, the proposed methods can reduce the averaged computational complexity.

Table 1: Calculation of memory saving for Test #1 and Test #2

|  | Memory saving for GEO (MB) | Memory saving for TPM (MB) | Memory saving for TPM and GEO (MB) |
| --- | --- | --- | --- |
| Test #1 | Blending masks: (128*128+128*64+64*128+64*64 +64*32+64*16+64*8 +8*64+16*64+32*64) *140*4/8/1024/1024 = 2.939 Motion storage masks: 2.939/4/4/2 = 0.092 | Blending masks for luma: (128*128+128*64+64*128+64*64 +64*32+64*16+64*8+64*4+4*64 +8*64+16*64+32*64) *2*4/8/1024/1024 = 0.042 Blending masks for chroma: 0.042/2/2 = 0.011 Motion storage masks: 0.042/4/4/2 = 0.001 | 2.939 + 0.092 + 0.042 + 0.011 + 0.001 = 3.1 |
| Test #2 | Blending masks: (128*128+128*64+64*128) *140*4/8/1024/1024 = 2.188 Motion storage masks: 2.188/4/4/2 = 0.068 | Blending masks for luma: (128*128+128*64+64*128) *2*4 /8/1024/1024 = 0.031 Blending masks for chroma: 0.031/2/2 = 0.008 Motion storage masks: 0.031/4/4/2 = 0.001 | 2.188 + 0.068 + 0.031 + 0.008 + 0.001 = 2.3 |

# Simulation results

The proposed methods were implemented on top of CE4-1.14 software [1] under JVET CTC conditions [3]. Simulation results for Test #1 and Test #2 reportedly are provided in Table 2 and Table 3, respectively, for RA and LDB configurations.

Table 2: BD-Rate results for Test #1

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE4-1.14 of VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.03% | 0.11% | 98% | 101% |
| Class A2 | 0.04% | 0.08% | 0.06% | 98% | 101% |
| Class B | 0.03% | -0.07% | -0.06% | 98% | 100% |
| Class C | 0.04% | 0.00% | 0.00% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.01% | 0.02% | 98% | 100% |
| Class D | -0.02% | -0.10% | -0.01% | 98% | 97% |
| Class F | 0.03% | 0.04% | 0.04% | 98% | 99% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over CE4-1.14 of VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.13% | 0.10% | -0.10% | 97% | 100% |
| Class C | 0.09% | 0.17% | 0.06% | 98% | 98% |
| Class E | 0.49% | 1.06% | 0.64% | 96% | 101% |
| Overall | 0.21% | 0.36% | 0.14% | 97% | 100% |
| Class D | 0.11% | -0.24% | -0.74% | 98% | 98% |
| Class F | 0.01% | -0.32% | 0.29% | 96% | 97% |

Table 3: BD-Rate results for Test #2

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE4-1.14 of VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | 0.01% | 98% | 99% |
| Class A2 | 0.00% | 0.01% | -0.01% | 99% | 100% |
| Class B | -0.01% | -0.01% | -0.04% | 98% | 98% |
| Class C | 0.00% | -0.04% | 0.02% | 98% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.01% | 98% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 92% |
| Class F | 0.00% | -0.01% | 0.01% | 96% | 95% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over CE4-1.14 of VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.18% | -0.21% | 97% | 99% |
| Class C | 0.02% | 0.19% | -0.10% | 99% | 98% |
| Class E | -0.03% | 0.22% | 0.11% | 98% | 97% |
| Overall | 0.00% | 0.05% | -0.09% | 98% | 98% |
| Class D | 0.04% | -0.10% | -0.35% | 98% | 94% |
| Class F | 0.01% | -0.22% | -0.13% | 97% | 93% |

# Conclusions

Two tests are proposed to limit the block sizes for TPM and GEO. Test #1 is to disable TPM and GEO for blocks greater than 32×32, saving 3.1 MB memory; while Test #2 is to disable TPM and GEO for blocks greater than 64×64, saving 2.3 MB memory. It is recommended adopting one of the proposed methods.

# References

- H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, M. Bläser, J. Sauer, “CE4: CE4-1.1, CE4-1.2 and CE4-1.14: Geometric Merge Mode (GEO)”, JVET-P0068, 16th JVET Meeting, Geneva, Switzerland, October 2019.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019, Gothenburg, SE.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, CH.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).