| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0666_v3 |
| --- | --- |

| Title: | Non-CE5: Combination of JVET-P0470 and JVET-P0557 on simplification of CC-ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li, Xin Zhao, Liang Zhao, Yixin Du, Shan Liu Nan Hu, Jie Dong, Vadim Seregin, Marta Karczewicz | Tel: Email: | xlxiangli@tencent.com nanh@qti.qualcomm.com |
| Source: | Tencent, Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal provides a combination of JVET-P0470 and JVET-P0557 on simplification of cross-component ALF. With the proposed method, the dynamic range of filter coefficients are constrained within 4-bit including 1-bit sign and shift instead of multiplication is used in filtering process so that 16-bit operation is kept for up to 12-bit internal bit-depth. Coding performance is reported as follows

- Over VTM-6, BD rate changes are 0.11%, -7.27%, -5.83% for AI, 0.17%, -10.38%, -9.86% for RA, 0.14%，-15.63%，-11.80% for LDB.

- Over CE5-2.1, BD rate changes are -0.02%, 0.35%, 0.21% for AI, 0.01%, 0.36%, 0.23% for RA, -0.03%， 0.04%， -0.05% for LDB.

- When CC-ALF offsets are further clipped and block level on/off, temporal prediction are disabled, slightly better coding performance is observed: over VTM5-2.1, BD rate changes are -0.04%, 0.59%, 0.44% for AI.

# Introduction

In CE5-2.1 (earlier JVET-O0636), Cross Component Adaptive Loop Filter (CC-ALF) was proposed to enhance chroma components by adding offsets to chroma samples after ALF, where the offsets are derived from a high-pass filter with reconstructed luma samples as input. The filtering process is as follows

ΔIix,y=x0,y0∈SiI0xC+x0,yC+y0cix0,y0,
where ΔIix,y denotes chroma offset, I0xC+x0,yC+y0 indicates luma reconstruction sample, and cix0,y0 represents filter coefficient. For the filter, a diamond shape is used with eighteen filter coefficients, among which fourteen coefficients are different.

In CE5-2.1, the bit-depth of filter coefficients is 8-bit. When internal bit-depth is 10-bit, eighteen 8-bit by 10-bit multipliers are needed to filter each chroma sample. With the consideration that some filter coefficients are the same, the number of multipliers may be reduced to fourteen but the bit-depth of some multipliers is also increased.

In JVET-P0470, it is proposed to constrain the dynamic range of filter coefficients within 4-bit to reduce the complexity of filtering process.

In JVET-P0557, it is proposed to constrain the absolute value of filter coefficients to be power of 2 such that multiplication in filtering process can be replaced with shift.

# Proposed method

In this contribution, it is proposed to combine JVET-P0470 and JVET-P0557 to only have following CC-ALF coefficients 0, ±1, ±2, ±4 and ±8 such that 16-bit operation may be kept for up to 12-bit internal bit-depth.

# Experimental results

The simulation results are summarized as follows, where the anchor on the left side is VTM-6, the anchor on the right side is CE5-2.1.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6 |  |  |  |  | Over CE5-2.1-noshift |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | -3.31% | -3.94% | 104% | 103% | 0.00% | 0.19% | 0.23% | 100% | 100% |
| Class A2 | 0.09% | -7.44% | -8.19% | 102% | 103% | -0.04% | 1.35% | 0.87% | 100% | 100% |
| Class B | 0.09% | -10.36% | -8.46% | 102% | 105% | -0.01% | 0.24% | 0.13% | 101% | 101% |
| Class C | 0.10% | -4.17% | -3.94% | 102% | 103% | -0.02% | 0.01% | -0.03% | 100% | 105% |
| Class E | 0.16% | -10.04% | -3.48% | 102% | 107% | -0.02% | 0.15% | -0.02% | 101% | 105% |
| Overall | 0.11% | -7.27% | -5.83% | 102% | 104% | -0.02% | 0.35% | 0.21% | 100% | 102% |
| Class D | 0.11% | -3.08% | -3.59% | 102% | 104% | 0.00% | -0.35% | -0.31% | 101% | 102% |
| Class F | 0.17% | -6.37% | -6.53% | 101% | 106% | -0.01% | 0.40% | 0.17% | 100% | 104% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6 |  |  |  |  | Over CE5-2.1-noshift |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | -5.04% | -5.10% | 103% | 102% | 0.00% | 0.67% | 0.25% | 101% | 100% |
| Class A2 | 0.21% | -10.54% | -14.68% | 103% | 103% | 0.00% | 1.33% | 1.27% | 101% | 100% |
| Class B | 0.22% | -17.62% | -14.10% | 102% | 105% | 0.01% | 0.03% | -0.09% | 101% | 102% |
| Class C | 0.11% | -5.21% | -4.53% | 101% | 103% | 0.02% | -0.18% | -0.18% | 101% | 105% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.17% | -10.38% | -9.86% | 102% | 103% | 0.01% | 0.36% | 0.23% | 101% | 102% |
| Class D | 0.11% | -5.09% | -4.84% | 102% | 106% | 0.00% | -0.70% | -0.43% | 101% | 103% |
| Class F | 0.20% | -4.63% | -4.59% | 102% | 104% | 0.01% | 0.16% | 0.05% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6 |  |  |  |  | Over CE5-2.1-noshift |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.19% | -21.01% | -18.97% | 102% | 102% | 0.01% | 0.73% | 0.29% | 100% | 98% |
| Class C | 0.17% | -8.24% | -6.66% | 102% | 108% | 0.05% | -0.08% | -0.25% | 101% | 101% |
| Class E | 0.03% | -16.52% | -6.70% | 104% | 107% | -0.21% | -0.95% | -0.36% | 100% | 101% |
| Overall | 0.14% | -15.63% | -11.80% | 102% | 105% | -0.03% | 0.04% | -0.05% | 100% | 100% |
| Class D | 0.23% | -8.76% | -6.51% | 101% | 107% | 0.05% | -0.74% | -0.51% | 101% | 107% |
| Class F | 0.28% | -10.09% | -7.56% | 102% | 106% | 0.15% | 0.47% | -0.04% | 100% | 102% |

It can be observed from the results that the proposed simplification has minor efficiency impact when compared to CE5-2.1.

It is further reported that when disabling CC-ALF temporal prediction, block level on/off and adding offset clip, slightly better coding performance is achieved.

|  |  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-6 |  |  |  |  |  |  |  |  |  |  | Over CE5-2.1-noshift |  |  |  |  |  |  |  |  |
|  |  | Y |  |  | U |  | V |  | EncT |  | DecT |  | Y |  |  | U |  | V |  | EncT | DecT |
| Class A1 | 0.11% | -3.26% | -3.77% | 101% | 102% | -0.02% | 0.24% | 0.41% | 97% | 98% |
| Class A2 | 0.05% | -7.32% | -7.84% | 100% | 102% | -0.08% | 1.49% | 1.26% | 98% | 99% |
| Class B | 0.07% | -10.00% | -8.17% | 99% | 101% | -0.03% | 0.67% | 0.50% | 98% | 97% |
| Class C | 0.08% | -3.92% | -3.81% | 100% | 95% | -0.05% | 0.27% | 0.11% | 99% | 97% |
| Class E | 0.15% | -9.88% | -3.44% | 100% | 99% | -0.04% | 0.34% | 0.02% | 99% | 97% |
| Overall | 0.09% | -7.06% | -5.63% | 100% | 100% | -0.04% | 0.59% | 0.44% | 98% | 98% |
| Class D |  | 0.09% |  |  | -2.81% |  | -3.26% |  | 100% |  | 96% |  | -0.03% |  |  | -0.06% |  | 0.05% |  | 99% | 95% |
| Class F | 0.07% | -5.14% | -5.82% | 99% | 99% | -0.11% | 2.30% | 1.06% | 99% | 96% |
|  |  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Over VTM-6 |  |  |  |  |  |  |  |  |  |  | Over CE5-2.1-noshift |  |  |  |  |  |  |  |  |
|  |  | Y |  |  | U |  | V |  | EncT |  | DecT |  | Y |  |  | U |  | V |  | EncT | DecT |
| Class A1 | 0.14% | -4.92% | -4.96% | 100% | 101% | 0.01% | 0.80% | 0.40% | 97% | 99% |
| Class A2 |  | #VALUE! |  |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |  | #VALUE! |  |  | #VALUE! |  | #VALUE! |  | #NUM! | #NUM! |
| Class B | 0.22% | -17.23% | -13.84% | 99% | 100% | 0.01% | 0.54% | 0.34% | 98% | 97% |
| Class C | 0.11% | -4.86% | -4.21% | 99% | 95% | 0.02% | 0.19% | 0.15% | 98% | 97% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  | #VALUE! |  |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |  | #VALUE! |  |  | #VALUE! |  | #VALUE! |  | #NUM! | #NUM! |
| Class D | 0.07% | -4.85% | -4.55% | 100% | 100% | -0.04% | -0.45% | -0.09% | 99% | 97% |
| Class F | 0.14% | -4.25% | -4.33% | 100% | 98% | -0.05% | 0.92% | 0.46% | 98% | 97% |
|  |  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Over VTM-6 |  |  |  |  |  |  |  |  |  |  | Over CE5-2.1-noshift |  |  |  |  |  |  |  |  |
|  |  | Y |  |  | U |  | V |  | EncT |  | DecT |  | Y |  |  | U |  | V |  | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | #VALUE! |  |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |  | #VALUE! |  |  | #VALUE! |  | #VALUE! |  | #NUM! | #NUM! |
| Class C | 0.12% | -7.27% | -5.68% | 100% | 105% | 0.00% | 1.02% | 0.83% | 99% | 98% |
| Class E | 0.09% | -16.05% | -5.90% | 101% | 103% | -0.14% | -0.45% | 0.50% | 98% | 97% |
| Overall |  | #VALUE! |  |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |  | #VALUE! |  |  | #VALUE! |  | #VALUE! |  | #NUM! | #NUM! |
| Class D | 0.12% | -6.79% | -5.06% | 100% | 98% | -0.06% | 1.46% | 1.09% | 100% | 98% |
| Class F |  | #VALUE! |  |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |  | #VALUE! |  |  | #VALUE! |  | #VALUE! |  | #NUM! | #NUM! |

# Spec text changes

The proposed text changes are on top of CE5-2.1 are as follows. The changes are highlighted.

Changes to syntax table and semantics

| alf_data( ) { | Descriptor |
| --- | --- |
| … |  |
| if ( alf_cross_component_cb_filter_signal_flag ) |  |
| for ( j = 0; j < 14; j++ ) { |  |
| alf_cross_component_cb_coeff_nonzeroabs[ j ] | u(1) |
| if( alf_cross_component_cb_coeff_nonzeroabs[ j ] ){ |  |
| alf_cross_component_cb_coeff_shift[ j ] | u(2) |
| alf_cross_component_cb_coeff_sign[ j ] | u(1) |
| } |  |
| } |  |
| if ( alf_cross_component_cr_ filter_signal_flag ) |  |
| for ( j = 0; j < 14; j++ ) { |  |
| alf_cross_component_cr_coeff_nonzeroabs[ j ] | u(1) |
| if( alf_cross_component_cr_coeff_nonzeroabs[ j ] ) { |  |
| alf_cross_component_cr_coeff_shift[ j ] | u(2) |
| alf_cross_component_cr_coeff_sign[ j ] | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

alf_cross_component_cb_coeff_nonzero[ j ] equal to 1 specifies that the j-th cross component Cb filter coefficient is non-zero, alf_cross_component_cb_coeff_shift[ j ] and alf_cross_component_cb_coeff_sign[ j ] are further signaled. alf_cross_component_cb_coeff_nonzero[ j ] equal to 0 specifies that the j-th cross component Cb filter coefficient is zero.

alf_cross_component_cb_coeff_shift[ j ] specifies the shift value of the j-th cross component Cb filter coefficient. When alf_cross_component_cb_coeff_shift[ j ] is not present, it is inferred to be equal to 0.

The cross component Cb filter coefficients CcAlfApsCoeffCb[ adaptation_parameter_set_id ] with elements CcAlfApsCoeffCb[ adaptation_parameter_set_id ][ j ], with j = 0..13 are derived as follows:

- If alf_cross_component_cb_coeff_nonzero[ j ] is equal to 0, CcAlfApsCoeffCb[ adaptation_parameter_set_id ][ j ] = 0

- Othersize, CcAlfApsCoeffCb[ adaptation_parameter_set_id ][ j ] = (1 << alf_cross_component_cb_coeff_shift[ j ]) *	( 1 − 2 * alf_cross_component_cb_coeff_sign[ j ] )

alf_cross_component_cr_coeff_nonzero[ j ] equal to 1 specifies that the j-th cross component Cr filter coefficient is non-zero, alf_cross_component_cr_coeff_shift[ j ] and alf_cross_component_cr_coeff_sign[ j ] are further signaled. alf_cross_component_cr_coeff_nonzero[ j ] equal to 0 specifies that the j-th cross component Cr filter coefficient is zero.

alf_cross_component_cb_coeff_shift[ j ] specifies the shift value of the j-th cross component Cr filter coefficient. When alf_cross_component_cr_coeff_shift[ j ] is not present, it is inferred to be equal to 0.

The cross component Cr filter coefficients CcAlfApsCoeffCr[ adaptation_parameter_set_id ] with elements CcAlfApsCoeffCr[ adaptation_parameter_set_id ][ j ], with j = 0..13 are derived as follows:

- If alf_cross_component_cr_coeff_nonzero[ j ] is equal to 0, CcAlfApsCoeffCr[ adaptation_parameter_set_id ][ j ] = 0

- Othersize, CcAlfApsCoeffCr[ adaptation_parameter_set_id ][ j ] = (1 << alf_cross_component_cr_coeff_shift[ j ]) *	( 1 − 2 * alf_cross_component_cr_coeff_sign[ j ] )

# Conclusions

In this contribution, CC-ALF is simplified by constraining filter coefficients to be 0, ±1, ±2, ±4 and ±8 such that 16-bit operation may be kept for up to 12-bit internal bit-depth. Simulation results show that the proposed method provides almost the same coding efficiency to CE5-2.1 with less complexity in both hardware and software implemenation. It is proposed to adopt this simplification.

# References

- F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).