| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0754 |
| --- | --- |

| Title: | Crosscheck of JVET-P0110 (AHG15: Quantization matrices with single identifier and enhanced prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Julien Le Tanou 5 rue Louis Jacques Daguerre 35136 St Jacques de la Lande France | Tel: Email: | +33 2 56 85 56 77 julien.letanou@mediakind.com |
| Source: | MediaKind, R&D |  |  |

_____________________________

# Abstract

This input document reports crosscheck results of JVET-P0110 on Quantization matrices with single identifier and enhanced prediction from Interdigital. JVET-P0110 proposes to modify quantization matrix (QM) signaling and prediction mechanism, initially adopted in VVC as per JVET-N0847. JVET-P0110 is a continuation work based on JVET-O0223 with 3 new added features. Authors provide related draft text changes on top of current VVC draft 6, as well as text count statistics for highlighting text simplification. Draft text and statistics have been verified and are reported consistent with the proposed changes. In addition, a GNU Octave source code, a test set of QM samples and a test procedure for running code are provided by Authors to measure the bit cost reduction of the proposed QMs signaling and prediction processes in comparison with HEVC, VVC draft 6 and JVET-O0223 mechanisms. The current crosscheck reports that the GNU Octave source code is implementing both encoding and decoding processes for bit generations, bit cost measurements and compliance verification. Encoding process compliance with decoding process has been checked for each compared method. Based on JVET-P0110 version 1, decoding compliance issues have been reported to Authors for some of the proposed coding methods, JVET-P0110 #0, #1 and #2, in the lossy test-case. Based on current JVET-P0110 version 2, decoding conformance issues are reported to be solved. Source code seems consistent with the proposed modifications in JVET- P0110, as well with the related specification texts in HEVC and in VVC draft 6 being used for comparison. Following the provided test procedure, code has been successfully run. The bit costs and relative bit savings results are reported to match the ones in the updated JVET-P0110 version 2. No implementation or test results on top of VTM-6.0 were provided nor cross-checked for performance and compliance verification purposes.

# Introduction

JVET-P0110 proposes to modify quantization matrix (QM) signaling and prediction mechanism, currently adopted in VVC as per JVET-N0847. JVET-P0110 is a continuation work based on JVET-O0223 with  new added features:

- Allow prediction across all QMs by using a single QM identifier (as in JVET-O0223)

  - QM coding scheme in VVC draft 6 allows copy from QMs associated to the same transform size only; JVET-O0223 allowed prediction (i.e. copy, or down-sample) of a QM from another one associated to a different block size or type

  - To do so, JVET-O0223 defined a single QM identifier (matrixId; instead of separate sizeId/matrixId), and QMs are ordered by decreasing block size with prediction using either a copy or decimation.

- Allow a residual on top of prediction

  - In addition to the single QM identifier and prediction across all QMs taken from JVET-O0223, JVET-P0110 proposes to allow refining prediction with a residual. That residual reuses the syntax currently used to transmit the list of QM coefficients, but with variable length coding. Besides, number of predicted coefficients (i.e. residual values) has to be signalled.

- Allow prediction with a scale factor instead of just copy

  - In addition to prediction across all QMs and refinement with a residual, JVET-P0110 proposes to allow prediction based on a scale factor instead of just copy

  - Scale factor is expressed in 1/16ths, limited to 0 – 32 range, and coded as an offset from 16

  - One scale factor per matrixId is coded as se(v)

- Remove the prediction flag

  - JVET-P0110 removes the prediction flag (as per JVET-O2001-vE) since it is possible to emulate the non-prediction case by using zero-scale, which makes the prediction flat.

# Crosscheck results

GNU Octave source code, a test set of QM samples and a test procedure for running code were provided.

The current crosscheck reports that the GNU Octave source code is implementing both the encoding and decoding processes for bit generations, bit cost measurements and compliance verification of the different coding techniques compared: HEVC, VVC draft 6, JVET-O0223, and the 3 additional features in JVET-P0110.

Code source has been checked; the implemented encoding and decoding processes seem consistent with the proposed modifications in JVET- P0110, as well with the related specification texts in HEVC and in VVC draft 6 (as per JVET-O2001-vE), being used for comparison.

It is noted that no implementation or test results on top of VTM-6.0 were provided nor cross-checked for performance and compliance verification purposes.

## Text complexity

Using statistics from Microsoft Word and draft texts from JVET-O2001-vE, JVET-N0847 and JVET-P0110, the statistics reported in section 3 of JVET- P0110 have been verified.

Consistency of draft text changes with proposal has been verified.

## Coding efficiency

Following the provided test procedure, the code has been successfully run using GNU Octave v5.1.0. The bit costs and relative bit savings results for each method are reported below, and in the attached excel-sheet.

The numbers are matching the ones reported in the updated JVET- P0110 version 2; with average QM bit cost saving over VVC draft 6 ranging from:

- -48% to -54% for the lossless use-case.

- -54% to -67% for the lossy use-case with an MAD <= 1.5

Figure 1: QM bit counts with different coding methods

Table 1: Average QM bit count comparison with different coding methods (lossless)

| ref. | HEVC | VVC draft 6 | JVET-O0223 | JVET-P0110-1 | JVET-P0110-2 |
| --- | --- | --- | --- | --- | --- |
| VVC draft 6 | +27% |  |  |  |  |
| JVET-O0223 | -28% | -43% |  |  |  |
| JVET-P0110-1 | -34% | -48% | -8% |  |  |
| JVET-P0110-2 | -41% | -53% | -18% | -11% |  |
| JVET-P0110-3 | -41% | -54% | -19% | -11% | -1% |

Table 2: Average QM bit count comparison with different coding methods (lossy:  MAD <= 1.5)

| ref. | HEVC | VVC draft 6 | JVET-O0223 | JVET-P0110-1 | JVET-P0110-2 |
| --- | --- | --- | --- | --- | --- |
| VVC draft 6 | +27% |  |  |  |  |
| JVET-O0223 | -28% | -43% |  |  |  |
| JVET-P0110-1 | -42% | -54% | -19% |  |  |
| JVET-P0110-2 | -57% | -66% | -40% | -26% |  |
| JVET-P0110-3 | -58% | -67% | -41% | -27% | -1.1% |

## Conformance

Following the provided test procedure, the encoding process compliance with the decoding process has been checked for each compared method.

Based on JVET-P0110 version 1, decoding compliance issues were reported for some of the proposed coding methods, JVET-P0110 #0 (as JVET-O0223), JVET-P0110 #1 and #2, in the lossy test-case when varying the “tolerance error” parameter. Issues have been reported to Authors for correction and fixed in the latter JVET-P0110 version 2. Proposed encoding/decoding processes are now reported to be compliant. A conformance log file is attached to this document.

# References

[JVET-O2001]	B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, Gothenburg, SE, July 2019

[JVET-O0223]	P. de Lagrange, F. Le Léannec, E. François, K. Naser, “Non-CE7: Quantization matrices with single identifier and prediction from larger ones,” JVET-NO0223, Gothenburg, SE, July 2019

[JVET-N0847]	O.Chubach, T.Toma, S.C.Lim, et al., “CE7-related: Support of quantization matrices for VVC”, JVET-N0847, Geneva, CH, March 2019

[JVET-P0110]	P. de Lagrange, F. Le Léannec, E. François, K. Naser, “AHG15: Quantization matrices with single identifier and enhanced prediction”, JVET-P0110, Geneva, CH, October 2019