| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0767 |
| --- | --- |

| Title: | Crosscheck of JVET-P0192 (CE5-related: Reducing the number of luma filters in ALF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tsung-Chuan Ma 8910 University Center Dr., Suite 400, San Diego, CA 92122, USA | Email: | tsung-chuanma@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This document presents a cross-check report on “CE5-related: Reducing the number of luma filters in ALF” as proposed in JVET-P0192. It is confirmed that all the test results are matched with those provided by the proponent. Meanwhile, the proposed software is also confirmed that matches with proposal.

# Introduction

In JVET-P0192 [1] it is proposed to reduce the maximum number of luma filters signalled for ALF from 25 to 16.

# Experiment results

The cross-check tests were performed based on VTM-6.0 software [2] with VTM configuration files under JVET common test condition [3].

The cross-check results of JVET-P0192 are summarized in Table I. All the test results are matched with those provided by the proponent. And, the detailed results can be found in the accompanying spreadsheet.

Table I. Cross-check results of JVET-P0192

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 95% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 98% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 95% | 95% |
| Class C | 0.00% | 0.00% | 0.00% | 92% | 88% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 96% | 96% |
| Class D | 0.00% | 0.00% | 0.00% | 95% | 90% |
| Class F | 0.00% | 0.00% | 0.00% | 93% | 91% |

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.01% | 101% | 104% |
| Class A2 | 0.00% | 0.03% | -0.03% | 102% | 104% |
| Class B | 0.00% | 0.00% | 0.00% | 93% | 95% |
| Class C | 0.00% | 0.00% | 0.00% | 93% | 86% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.01% | 96% | 96% |
| Class D | 0.00% | 0.00% | 0.00% | 92% | 87% |
| Class F | 0.00% | 0.00% | 0.01% | 94% | 91% |

|  |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.07% | 0.09% | 106% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 106% |
| Class E | 0.00% | 0.00% | 0.00% | 98% | 104% |
| Overall | 0.00% | 0.03% | 0.04% | 102% | 104% |
| Class D | 0.00% | 0.00% | 0.00% | 95% | 94% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 97% |

# References

- O. Chubach, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei, “CE5-related: Reducing the number of luma filters in ALF,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P0192, Oct. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019