| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0827 |
| --- | --- |

| Title: | Crosscheck of JVET-P0258 (AHG18: Disabling Dependent Quantization for Lossless) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tangi Poirier 975 Avenue des champs blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33.2.56.85.62.76 tangi.poirier@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports crosscheck results of JVET-P0258 (AHG18: Disabling Dependent Quantization for Lossless). JVET-P0258 proposes to disable DependantQuantization in transquant bypass mode using regular residual coding. This proposal giva adiitional results in combination with JVET-P0082 where BDCM is enabled for lossless. Source code has been checked and simulations results match with those from JVE-P0258.

# Experimental results

The crosscheck has been performed for an intra-only (AI) and a random-access (RA) configuration with the VTM-6.0 software. Simulations results of the method of the proponents input document [1] is reported in Table 1. In this method, DependantQuantization is disable for transQuantBypass blocks.

Table 1: Results of proposed method over VTM 6.0

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal |  | AHG18 LL | Proposal |  |
| Class A1 | 2.2 | 2.2 | -0.13% | 2.3 | 2.3 | -0.15% |
| Class A2 | 1.7 | 1.7 | 0.00% | 1.9 | 1.9 | -0.07% |
| Class B | 2.1 | 2.1 | -0.17% | 2.3 | 2.3 | -0.28% |
| Class C | 2.1 | 2.1 | -0.24% | 2.6 | 2.6 | -0.44% |
| Class D | 1.9 | 2.0 | -0.26% | 2.8 | 2.8 | -0.50% |
| Class E | 2.9 | 2.9 | -0.19% |  |  |  |
| Class F | 5.5 | 5.5 | -0.42% | 39.4 | 39.7 | -0.60% |
| TGM | 12.3 | 12.3 | -0.54% | 112.6 | 113.4 | -0.71% |
| Class A1 | 2.2 | 2.2 | -0.15% | 2.3 | 2.3 | -0.25% |
| Overall | 4.0 | 4.0 | -0.25% | 25.1 | 25.2 | -0.41% |
| Enc Time[%] | 101% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 96% |  |  |

Variant 2 combined the proposed method with proposal JVET-P0082 where BDPCM is enabled for TS residual coding.

Table 2: Results of variant 2 over VTM6.0

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal |  | AHG18 LL | Proposal |  |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C | 2.1 | 2.1 | -0.81% | 2.6 | 2.6 | -0.62% |
| Class D | 1.9 | 2.0 | -1.73% | 2.8 | 2.8 | -0.81% |
| Class E |  |  |  |  |  |  |
| Class F | 5.5 | 5.7 | -3.42% | 39.4 | 41.3 | -2.94% |
| TGM | 12.3 | 12.7 | -3.58% | 112.6 | 116.2 | -3.09% |
| Overall (A1, A2, B, C, E) |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |

Variant 1 combined the proposed method with proposal JVET-P0082 where BDPCM is enabled for both TS residual coding and regular transform.

Table 3: Results of variant 1 over VTM6.0

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal |  |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C | 2.1 | 2.1 | -1.67% |
| Class D | 1.9 | 2.0 | -2.88% |
| Class E |  |  |  |
| Class F | 5.5 | 5.8 | -4.13% |
| TGM | 12.3 | 12.8 | -3.97% |
| Overall (A1, A2, B, C, E) |  |  |  |
| Overall |  |  |  |

Variant 3 combined the proposed method with proposal JVET-P0082 where BDPCM is enabled for both TS residual coding and regular transform and BDPCM is enabled for TR residual coding for chroma.

Table 4: Resuls of variant 3 over VTM6.0

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal |  |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C | 2.1 | 2.1 | -2.43% |
| Class D | 1.9 | 2.0 | -4.02% |
| Class E |  |  |  |
| Class F | 5.5 | 5.9 | -5.68% |
| TGM | 12.3 | 13.3 | -7.04% |
| Overall (A1, A2, B, C, E) |  |  |  |
| Overall |  |  |  |

Variant 3 combined the proposed method with proposal JVET-P0082 where BDPCM is enabled for both TS residual coding and regular transform and BDPCM is enabled for TR residual coding for chroma.

Table 5: Results of variant 4 over VTM6.0

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal |  |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C | 2.1 | 2.1 | -2.45% |
| Class D | 1.9 | 2.0 | -4.00% |
| Class E |  |  |  |
| Class F | 5.5 | 5.9 | -5.70% |
| TGM | 12.3 | 13.4 | -7.91% |
| Overall (A1, A2, B, C, E) |  |  |  |
| Overall |  |  |  |

# Conclusion

The contribution P0258 proposes to disable DependantQuantization in lossless mode for regular residual coding. It shows that it may be combined with JVET-P0082 in which BDPCM is enabled for lossless mode. Source code has been checked and simulations results matches results from JVET-P0258.

# References

[1] M. Karczewicz, A. Nalci, Y.-H. Chao, H. Wang, H. E. Egilmez, M. Coban, “AHG18: Disabling Dependent Quantization for Lossless”, JVET-P0258, Geneva, CH, 2019.

[2] A. Nalci, H. Wang, H. E. Egilmez, M. Coban, M. Karczewicz, “AHG18: BDPCM for Lossless”, JVET-P0082, Geneva, CH, 2019.