| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0842 |
| --- | --- |

| Title: | Crosscheck of JVET-P0193: Gradual Decoding Refresh (GDR) using encoder and normative restrictions (Test 2.1.b) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Limin Wang Seungwook Hong | Tel: Email: | +1-858-335-9196 limin.2.wang@nokia.com seungwook.hong@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-P0193 “Gradual Decoding Refresh (GDR) using encoder and normative restrictions, Test 2.1.b”. The crosscheck results match the ones presented in JVET-P0193.

# Introduction

JVET-P0193 presents the results of CE2 tests 2.1a, 2.1.b and 2.1.c on gradual decoding refresh (GDR). In test 2.1.a, the GDR method applies motion vector (MV) and intra mode restrictions at the encoder as well as virtual boundaries with PPS signalling. Test 2.1.b adds virtual boundary signalling at slice level on top of test 2.1.a. Test 2.1.c introduces new normative changes to virtual boundaries which allow the removal of intra mode restrictions.

# Experiment Results

Crosscheck tests were conducted for CE2.1.b over JVET test sequences. Each test is conducted with the following conditions:

- Use low-delay B configuration in current common test condition (CTC) (i.e., encoder_lowdelay_vtm.cfg) with the following changes:

  - The first picture is coded as an IDR picture,

  - The GOP size is 1. Each picture refers up to 4 immediately preceding pictures in decoding order,

  - For test sequences with 60 fps, a GDR picture is inserted every 60 pictures,

  - For test sequences with 50 fps, a GDR picture is inserted every 50 pictures,

  - For test sequences with 30 fps, a GDR picture is inserted every 30 pictures,

  - For test sequences with 20 fps, a GDR picture is inserted every 20 pictures,

  - IntraQPOffset and QPoffset are set equal to 0.

- Maximum GDR refresh period is 1 second.

- SEIDecodedPictureHash is set to 1.

- HashME is disabled for class F.

The test results are compared with GDR anchor. Anchor bitstreams are encoded with the following conditions

- Use low-delay B configuration in current common test condition (CTC) (i.e., encoder_lowdelay_vtm.cfg) with the following changes:

  - The first picture is coded as an IDR picture,

  - The GOP size is 1. Each picture refers up to 4 immediately preceding pictures in decoding order,

  - For test sequences with 60 fps, a CRA picture is inserted every 60 pictures,

  - For test sequences with 50 fps, a CRA picture is inserted every 50 pictures,

  - For test sequences with 30 fps, a CRA picture is inserted every 30 pictures,

  - For test sequences with 20 fps, a CRA picture is inserted every 20 pictures.

  - IntraQPOffset and Qpoffset are set equal to 0

- SEIDecodedPictureHash is set to 1.

The crosscheck test results are presented in table 1, which matches the results of JVET-P0193. In BD rate comparisons, the pictures of the first second of sequences are excluded.

Table 1 Test results of test 2.1.b.

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over O-CE2 Anchor (VTM6.0) on Linux |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 2.86% | 6.15% | 9.64% | 105% | 103% |
| Class C | 5.62% | 6.74% | 7.33% | 104% | 105% |
| Class E | 13.90% | 16.21% | 18.72% | 112% | 108% |
| Overall | 6.54% | 8.86% | 11.14% | 106% | 105% |
| Class D | 9.84% | 12.67% | 13.65% | 106% | 109% |
| Class F | 32.50% | 39.65% | 40.78% | 121% | 108% |

# References

David Gommelet, Jean-Marc Thiesse and Didier Nicholson, “Gradual Decoding Refresh (GDR) using encoder and normative restrictions”, JVET-P0193, October 2019.

# Patent rights declaration(s)

Nokia does not have any current or pending patent rights relating to the technology described in this contribution.