| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0854 |
| --- | --- |

| Title: | Cross-check of JVET-P0392 (Non-CE6: Combination of ISP and LFNST) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | F. Le Léannec | Tel: Email: | fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution is the cross-checking of contribution JVET-P0392 “Non-CE6: Combination of ISP and LFNST”. The delivered source code has been checked and conform to description of JVET-P0392. Moreover, the source code has been compiled and run, and results obtained confirm the results described in JVET-P0392.

# Introduction

JVET-P0392 proposes to combine LFNST and ISP.

To do so, LFNST is applied to all TU in a CU coded in ISP mode. When LFNST is on, then the primary transform is DCT2_DCT2.

The TU-level conditions for LFNST on the last non-zero coefficient position is kept as in VTM6. The condition on having at lest one non-zero coefficient other than DC in at least a TU is removed for ISP CUs. Finally, some rules on the CU size are used to allow LFNST with ISP: TU must have a size at least 4x4.

The source code delivers conforms to the description of the proposed LFNST/ISP combination.

# Results

The following table gives the result obtained with the source code of the test 1 proposed in JVET-P0392. Obtained BD-rate results are identical to the ones claimed in contribution JVET-P0392.

Table 1: results obtained by the cross-check of JVET-P0392 test 1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.04% | 0.25% | 117% | 103% |
| Class A2 | -0.09% | 0.01% | -0.03% | 116% | 105% |
| Class B | -0.15% | -0.12% | -0.16% | 117% | 104% |
| Class C | -0.38% | -0.18% | -0.02% | 120% | 108% |
| Class E | -0.24% | -0.23% | -0.21% | 119% | 104% |
| Overall | -0.19% | -0.10% | -0.05% | 118% | 105% |
| Class D | -0.25% | -0.07% | -0.28% | 118% | 111% |
| Class F | -0.31% | -0.14% | -0.15% | 109% | 103% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.22% | 0.00% | -0.11% | 104% | 95% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.16% | -0.35% | -0.19% | 104% | 88% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.