| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0856-v1 |
| --- | --- |

| Title: | Non-CE4: On switchable interpolation filter and bi-prediction weight indices cleanup |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Timofey Solovyev, Sergey Ikonin, Alexander Karabutov, Chernyak Roman | Tel: Email: | +79250089652 solovyev.timofey@huawei.com sergey.ikonin@huawei.com karabutov.alexander@huawei.com chernyak.roman@huawei.com |  |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution describes changes in pruning during merge list construction process and history-based motion vector prediction lookup table update. Two options are proposed. The first option is do not use switchable interpolation filter index and bi-prediction weight indices in candidate comparison. In this case only specification changes are need to make specification text aligned with the test model software. The second option is to use switchable interpolation filter and bi-prediction weight indices in motion information comparison. In this case both specification text and test model software have to be changed.

# Introduction

In VVC draft 6 [1] there are three processes where motion information comparison is needed:

- 8.5.2.3 Derivation process for spatial merging candidates
(comparison of new spatial candidate with previously added merge candidates during merge list construction).

- 8.5.2.6 Derivation process for history-based merging candidates
(comparison of HMVP candidate with previously added merge candidates).

- 8.5.2.16 Updating process for the history-based motion vector predictor candidate list
(comparison of current block motion information with HMVP table records).

In current test model software switchable interpolation filter and bi-prediction weight indices are not used in motion information comparison in any of all abovementioned processes. Hovewer in specification draft in processes 8.5.2.6 and 8.5.2.16 (items 2 and 3) there is candidate comparison with the following text:

- “The candidate HmvpCandList[ NumHmvpCand − hMvpIdx] is equal to the merging candidate N…”

- “When hMvpCand is equal to HmvpCandList[ hMvpIdx ]…”

and there is no clear indication which components (motion vectors, reference indices, SIF and BCW indices) are used for this comparison. By default it could be interpreted as all components of merge candidate or HMVP record are used. This cause mismatch between software and specification text.

In this contribution we propose two options to clean up this part of VVC:

- Do not use switchable interpolation filter index and bi-prediction weight indices in candidate comparison in any of abovementioned processes. In this case only specification changes are need to make specification text clear and aligned with the test model software.

- To use switchable interpolation filter and bi-prediction weight indices in candidate comparison in all abovementioned processes. In this case both specification text and test model software have to be changed.

# Proposed Solutions

## Option 1. Aligning specification text to test model software.

The proposed changes on top of VVC draft 6 [1] are described as below. Newly added changes are highlighted in yellow and deleted texts are marked with strikethrough.

8.5.2.6	Derivation process for history-based merging candidates

Inputs to this process are:

–	a merge candidate list mergeCandList,

–	the number of available merging candidates in the list numCurrMergeCand.

Outputs to this process are:

–	the modified merging candidate list mergeCandList,

–	the modified number of merging candidates in the list numCurrMergeCand.

The variables isPrunedA1 and isPrunedB1 are both set equal to FALSE.

For each candidate in HmvpCandList[ hMvpIdx ] with index hMvpIdx = 1..NumHmvpCand, the following ordered steps are repeated until numCurrMergeCand is equal to MaxNumMergeCand − 1:

1.	The variable sameMotion is derived as follows:

–	If all of the following conditions are true for any merging candidate N with N being A1 or B1, sameMotion and isPrunedN are both set equal to TRUE:

–	hMvpIdx is less than or equal to 2.

–	The candidate HmvpCandList[ NumHmvpCand − hMvpIdx] is equal to and the merging candidate N have the same motion vectors and the same reference indices.

–	isPrunedN is equal to FALSE.

–	Otherwise, sameMotion is set equal to FALSE.

[…]

8.5.2.16		Updating process for the history-based motion vector predictor candidate list

[…]

The candidate list HmvpCandList is modified using the candidate hMvpCand by the following ordered steps:

1.	The variable identicalCandExist is set equal to FALSE and the variable removeIdx is set equal to 0.

2.	When NumHmvpCand is greater than 0, for each index hMvpIdx with hMvpIdx = 0..NumHmvpCand − 1, the following steps apply until identicalCandExist is equal to TRUE:

–	When hMvpCand is equal to and HmvpCandList[ hMvpIdx ] have the same motion vectors and the same reference indices, identicalCandExist is set equal to TRUE and removeIdx is set equal to hMvpIdx.

## Option 2. Using switchable interpolation filter and bi-prediction weight indices in motion information comparison.

The proposed changes on top of VVC draft 6 [1] are described as below. Newly added changes are highlighted in yellow and deleted texts are marked with strikethrough.

8.5.2.3	Derivation process for spatial merging candidates

[...]

For the derivation of availableFlagB1, refIdxLXB1, predFlagLXB1 and mvLXB1 the following applies:

–	The luma location ( xNbB1, yNbB1 ) inside the neighbouring luma coding block is set equal to ( xCb + cbWidth − 1, yCb − 1 ).

–	The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ). the neighbouring luma location ( xNbB1, yNbB1 ), checkPredModeY set equal to TRUE, and cIdx set equal to 0 as inputs, and the output is assigned to the block availability flag availableB1.

–	The variables availableFlagB1, refIdxLXB1, predFlagLXB1 and mvLXB1 are derived as follows:

–	If one or more of the following conditions are true, availableFlagB1 is set equal to 0, both components of mvLXB1 are set equal to 0, refIdxLXB1 is set equal to −1 and predFlagLXB1 is set equal to 0, with X being 0 or 1, and bcwIdxB1 is set equal to 0:

–	availableB1 is equal to FALSE.

–	availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB1, yNbB1 ) have the same motion vectors, the same reference indices, the same bi-prediction weight indices and the same half sample interpolation filter indices.

…

–	If one or more of the following conditions are true, availableFlagB0 is set equal to 0, both components of mvLXB0 are set equal to 0, refIdxLXB0 is set equal to −1 and predFlagLXB0 is set equal to 0, with X being 0 or 1, and bcwIdxB0 is set equal to 0:

–	availableB0 is equal to FALSE.

–	availableB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB0, yNbB0 ) have the same motion vectors, the same reference indices, the same bi-prediction weight indices and the same half sample interpolation filter indices.

…

–	If one or more of the following conditions are true, availableFlagA0 is set equal to 0, both components of mvLXA0 are set equal to 0, refIdxLXA0 is set equal to −1 and predFlagLXA0 is set equal to 0, with X being 0 or 1, and bcwIdxA0 is set equal to 0:

–	availableA0 is equal to FALSE.

–	availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbA0, yNbA0 ) have the same motion vectors, the same reference indices, the same bi-prediction weight indices and the same half sample interpolation filter indices.

…

–	If one or more of the following conditions are true, availableFlagB2 is set equal to 0, both components of mvLXB2 are set equal to 0, refIdxLXB2 is set equal to −1 and predFlagLXB2 is set equal to 0, with X being 0 or 1, and bcwIdxB2 is set equal to 0:

–	availableB2 is equal to FALSE.

–	availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB2, yNbB2 ) have the same motion vectors, the same reference indices, the same bi-prediction weight indices and the same half sample interpolation filter indices.

–	availableB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB2, yNbB2 ) have the same motion vectors, the same reference indices, the same bi-prediction weight indices and the same half sample interpolation filter indices.

…

8.5.2.6	Derivation process for history-based merging candidates

…

1.	The variable sameMotion is derived as follows:

–	If all of the following conditions are true for any merging candidate N with N being A1 or B1, sameMotion and isPrunedN are both set equal to TRUE:

–	hMvpIdx is less than or equal to 2.

–	The candidate HmvpCandList[ NumHmvpCand − hMvpIdx] is equal to the merging candidate N, having the same motion vectors, the same reference indices, the same bi-prediction weight indices and the same half sample interpolation filter indices.

–	isPrunedN is equal to FALSE.

–	Otherwise, sameMotion is set equal to FALSE.

…

8.5.2.16		Updating process for the history-based motion vector predictor candidate list

…

The candidate list HmvpCandList is modified using the candidate hMvpCand by the following ordered steps:

1.	The variable identicalCandExist is set equal to FALSE and the variable removeIdx is set equal to 0.

2.	When NumHmvpCand is greater than 0, for each index hMvpIdx with hMvpIdx = 0..NumHmvpCand − 1, the following steps apply until identicalCandExist is equal to TRUE:

–	When hMvpCand is equal to HmvpCandList[ hMvpIdx ], having the same motion vectors, the same reference indices, the same bi-prediction weight indices and the same half sample interpolation filter indices, identicalCandExist is set equal to TRUE and removeIdx is set equal to hMvpIdx.

…

## Supplementary specification draft bug fixes

The specification text drafts that we propose also comprise specification text bug fixes that were discussed as tickets #557 “Mismatch between SW and Spec on setting of half sample interpolation filter index in HMVP table” and #344 “spatial merge candidate derivation process with wrong prunings” in VVC bug tracker. We consider these bug fixes as important and propose also include these bug fixes to the next version of VVC specification draft.

# Conclusion

Two options for usage of interpolation filter and bi-prediction weight indices in motion information comparison are described in the contribution. The first option is do not use these indices in motion information comparison. The second option is to use both of these indices in motion information comparison in the following three processes:

- 8.5.2.3 Derivation process for spatial merging candidates (comparison of new spatial candidate with previously added merge candidates during merge list construction).

- 8.5.2.6 Derivation process for history-based merging candidates (comparison of HMVP candidate with previously added merge candidates).

- 8.5.2.16 Updating process for the history-based motion vector predictor candidate list
(comparison of current block motion information with HMVP table records).

The proposed specification draft includes bug fixes #557 “Mismatch between SW and Spec on setting of half sample interpolation filter index in HMVP table” and #344 “spatial merge candidate derivation process with wrong prunings” from VVC bug tracker.

# References

[1] B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team, JVET-O2001.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).