| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0868 |
| --- | --- |

| Title: | Crosscheck of JVET-P0255: CE5-related: Bilateral filter with SAO band offset |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Jacob Ström Torshamnsgatan 23 164 83 Stockholm Sweden | Email: | jacob.strom@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

A crosscheck of JVET-P0255 has been carried out for the CTC test set. It was reported that BDR figures match for all three test configurations (AI, RA, LDB).

# Encoder and decoder run times

The decoder run times are created by measuring the decoder run times of the test and comparing against the decoder run times of a previously run anchor. Hence the decoder run times in the cross checks are a bit inaccurate, but they seem to correspond well enough with the run times reported in JVET-P0255 so no more exact measurement was deemed necessary.

# Subjective viewing

We have informally looked at the Cactus sequence, qp 32, RA for the proposed scheme. Some artifacts around the “JOLLY JOKER” were observed that are not present in CE5-3.1.1 where the bilateral filter is combined with the unaltered version of SAO. However, compared to the CTC anchor, the artifacts around the “JOLLY JOKER” are not worse.

# Results

All intra: (Decoder timings were not made on the same machine and are therefore not reliable.)

|  | Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | 0.20% | -0.10% | 107% | 105% |
| Class A2 | -0.36% | 0.14% | 0.11% | 106% | 105% |
| Class B | -0.34% | 0.26% | 0.26% | 108% | 101% |
| Class C | -0.52% | 0.06% | 0.23% | 106% | 97% |
| Class E | -0.43% | 0.24% | 0.17% | 102% | 100% |
| Overall | -0.40% | 0.18% | 0.15% | 106% | 102% |
| Class D | -0.48% | 0.11% | 0.12% | 104% | 94% |
| Class F | -0.55% | -0.03% | -0.12% | 107% | 103% |

Random access: (Decoder timings were not made on the same machine and are therefore not reliable.)

|  | Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | 0.17% | 0.09% | 102% | 102% |
| Class A2 | -0.33% | 0.21% | 0.21% | 102% | 102% |
| Class B | -0.37% | 0.41% | 0.30% | 104% | 98% |
| Class C | -0.52% | 0.40% | 0.34% | 103% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.39% | 0.32% | 0.25% | 103% | 100% |
| Class D | -0.68% | 0.06% | 0.37% | 104% | 97% |
| Class F | -0.48% | 0.04% | 0.03% | 112% | 98% |

Low-delay B: (Decoder timings were not made on the same machine and are therefore not reliable.)

|  | Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.17% | 0.82% | 0.71% | 103% | 101% |
| Class C | -0.47% | 0.49% | 0.37% | 103% | 100% |
| Class E | -0.21% | 0.63% | 1.10% | 101% | 101% |
| Overall | -0.28% | 0.66% | 0.70% | 102% | 100% |
| Class D | -0.82% | 0.85% | 0.96% | 101% | 96% |
| Class F | -0.25% | 0.11% | 0.30% | 103% | 103% |

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).