| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0881 |
| --- | --- |

| Title: | Crosscheck of JVET-P0508 (CE7-related: unification for joint chroma residue coding modes) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jani Lainema | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution provides a crosscheck for the solution proposed in JVET-P0508 (CE7-related: unification for joint chroma residue coding modes). Software provided by proponents of JVET-P0508 was studied and it was found to match the description given in the contribution document.

Simulations following JVET common test conditions were performed by the crosschecker and objective results matched perfectly with those reported in JVET-P0508. The runtime differences between the crosscheck and the original proposal are explained by different cluster loads during the time the crosscheck was performed and when the anchor was generated.

Full simulation results are provided in the attached spreadsheets.

# Additional simulation results

In addition to CTC configurations, VTM-6.0 and the proposal were both tested also using config file parameter CbCrQpOffset with a value of -2 in LD-B configuration (instead of default value of -1 used in CTC). Combined YUV BD-rate using 8-1-1 weighting between Y, U and V indicate improved performance for all cases, among which using VTM-6.0 with configuration change seems to give the biggest impact.

## VTM-6.0 with CbCrQpOffset = -2

|  |  |  | LD-B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | YUV |
| Class B | 0.02 % | -0.69 % | -1.12 % | 100 % | 101 % | -0.17 % |
| Class C | 0.02 % | -0.78 % | -0.96 % | 100 % | 102 % | -0.16 % |
| Class E | -0.17 % | -0.93 % | -0.76 % | 100 % | 101 % | -0.30 % |
| Overall | -0.03 % | -0.78 % | -0.98 % | 100 % | 101 % | -0.20 % |
| Class D | 0.01 % | -0.52 % | -0.53 % | 101 % | 103 % | -0.10 % |
| Class F | 0.13 % | -0.64 % | -0.18 % | 101 % | 102 % | 0.02 % |

## Proposal with CbCrQpOffset = -2

|  |  |  | LD-B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | YUV |
| Class B | 0.07 % | -0.67 % | -1.48 % | 100 % | 100 % | -0.16 % |
| Class C | 0.10 % | -1.28 % | -0.76 % | 100 % | 100 % | -0.12 % |
| Class E | 0.05 % | -0.25 % | -0.54 % | 100 % | 101 % | -0.04 % |
| Overall | 0.08 % | -0.77 % | -1.01 % | 100 % | 100 % | -0.12 % |
| Class D | 0.11 % | -1.76 % | 0.56 % | 100 % | 99 % | -0.03 % |
| Class F | 0.13 % | -1.10 % | -1.42 % | 100 % | 100 % | -0.15 % |

## Proposal with CTC configuration (CbCrQpOffset = -1)

|  |  |  | LD-B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | YUV |
| Class B | 0.04 % | -0.32 % | -0.88 % | 100 % | 101 % | -0.09 % |
| Class C | -0.02 % | -0.99 % | -0.68 % | 100 % | 101 % | -0.18 % |
| Class E | -0.17 % | -0.16 % | 0.13 % | 100 % | 101 % | -0.14 % |
| Overall | -0.03 % | -0.50 % | -0.56 % | 100 % | 101 % | -0.13 % |
| Class D | -0.03 % | -1.16 % | 0.43 % | 100 % | 98 % | -0.10 % |
| Class F | 0.05 % | -0.26 % | -0.39 % | 100 % | 101 % | -0.03 % |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).